3GPP TSG-RAN5 Meeting #107			R5-253627

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	5.3.21.9

Source: 	Qualcomm Inc

Title: 	Discussion paper on FR2 PRACH UE beam correspondence

Document for:	Discussion and Endorsement

## 1.	Introduction

In response to the RAN4 LS [1] to RAN5 for the FR2 UE beam correspondence for initial access and RRC_inactive requirements, there has been several discussions in previous RAN5 meetings.

In [4] an alternate method without requiring beam lock test function was proposed. Action point AP#104.24 was created to include evaluating the testability aspects for testing beam correspondence requirement without beamlock function in addition to with beamlock

| AP#104.24 | RF | Check the testability aspects [with beam lock / without beam lock] for testing beam correspondence requirement in IDLE and INACTIVE mode. If the Beam lock function is confirmed not feasible the alternative method is considered. | Qualcomm, Huawei, Apple, Nokia | R5-244538, R5-246054, R5-245019, R5-244945, R5-247075, R5-247145, R5-247422, R5-250548/9,R5-250673/4 | RAN5#106 | Open |
| --- | --- | --- | --- | --- | --- | --- |

This paper provides the test procedure for verifying UE beam correspondence during initial access requirement without beamlock function.

## 2.	Discussion

As highlighted in past meetings, below are the key parameters/configuration/test procedure to ensure a reliable way to verify the UE beam correspondence for initial access requirements.

### Enabling multiple PRACH transmissions

Modify the preambleTransMax IE value to a large value (n200) to allow multiple PRACH transmissions.

For each received PRACH transmission, TE shall send a RAR containing Random access preamble identifier that do not match the transmitted random access preamble. UE computed power reaches Pcmax before PRACH EIRP measurement

Modify the preambleReceivedTargetPower IE to a large value (-60) such that UE computed Tx power reaches Pcmax.

### Accumulated PRACH power measurement period is atleast 1ms

Use PRACH format C2 prach-ConfigurationIndex value 190. Each RACH is 6 symbol duration with a CP+Seq duration of 41.67 us. This means for 1ms accumulated PRACH measurement period, 24 PRACH is needed.

### # of SSB and SSB power level

Configuring a single SSB for this test will ensure UE has only one SSB to measure and send its PRACH towards the direction of the SSB.

### Test procedure

Below is the test procedure to verify the SA FR2 PRACH beam correspondence requirements without beamlock.

Without [UE_beamlock_idle]

1.	Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-7 [3] to mount the DUT inside the QZ

2.	Position the DUT in DUT Orientation 1 from Tables  N.2-1 through N.2-7 [3].

3.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [10] clause 4.5.

4.	SS switches to SSB transmitting from PolLink = theta.

5.	SS transmits a Paging message (including PagingRecord with ue-Identity) for the UE to trigger PRACH

6.	The UE starts sending PRACH to the SS. In response, SS shall transmit RAR containing Random access preamble identifier that do not match the transmitted random access preamble.

7.	SS shall measure simultaneously the PRACH transmission power of both measurement polarization.

8.	Among the collected PRACH transmission from the UE, SS picks the set of X (X = 24 for PRACH format C2) consecutive PRACH providing the highest average PRACH power.

9. SS determines the EIRP for PolLink = theta by adding up the EIRP of the 2 measurement polarization.

10.	Once SS has completed the PRACH transmission power measurement, SS shall send a valid RAR containing random access preamble identifier that matches the transmitted random access preamble to the UE to ensure it goes to RRC_CONN. If UE fails to go to RRC_CONN, power cycle the UE.

11.	The SS shall transmit RRC release to release the RRC connection to ensure UE is in state RRC_IDLE.

10.Repeat steps 5-11 with TE switching to SSB transmitting from PolLink = phi. This is the EIRP PRACH power for PolLink = phi .

11.SS calculates UE PRACH EIRP for that grid point as a result of  EIRP (PolLink = Phi) + EIRP (PolLink = Theta).

12.	Advance to the next grid point and repeat steps 4 through 11 until measurements within zenith range 0o≤≤90o  have been completed.

13.	After the measurements within zenith range 0o≤≤90o have been completed and

a) if the re-positioning concept is applied to the TX test cases, position the device in DUT Orientation 2 (either Options 1 or 2) from Tables N.2-1 through N.2-7 [3] for the Alignment Option selected in Step 1. For the TX beam peak search in the second hemisphere, perform steps 3 through 12 for the range of zenith angles 90o>q≥0o.

b) if the re-positioning concept is not applied to the TX test cases, continue steps 3 through 12 for the range of zenith angles 90o<≤180o

14.	Calculate the CDF for all grid points and pass the UE if the derived %-tile EIRP in measurement distribution exceeds the test requirement. Otherwise fail the UE.

The above approach is implemented in the RAN5 CR R5-252987 for TC 6.6.3.

## RAN4 requirements for beam correspondence in initial access.

The beam correspondence requirement for power class 3 UEs in initial access and in RRC_INACTIVE consists of UE spherical coverage only:

-              UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.3-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.4.3.4.

Table 6.2.1.3-3: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n259 | 5.8 |
| n260 | 8 |
| n261 | 11.5 |
| n262 | 2.9 |
| n263 | 2.3 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance in RRC_CONNECTED. NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 4: Minimum EIRP at 50%-tile CDF is defined as the lower limit minus 2 dB in initial access and RRC_INACTIVE |  |

Explanation of RAN4 requirement:

- As long as UE’s UL power is sufficiently high (meeting min EIRP at 50%ile) in the direction of the downlink signal, it meets the conformance requirement.

- Autonomously chosen UL beam means UE makes it’s own decision.

- Additionally RAN4 made below recommendation

  - Collect 1ms worth of accumulated PRACH (note: RAN4 did not specify how 1ms worth of PRACH is collected. It can be any set of PRACH sent by the UE among the allowed number of PRACH Or it can be consecutive PRACH worth 1ms)

## Implementation of RAN4 requirement in a stable way and relevant for field deployment

Regarding the point raised by the chair on ensuring test method verifies the RAN4 requirement in a stable way and is relevant for field deployment,

The without Beamlock procedure provided in this paper satisfies all the RAN4 requirement and more. Although RAN4 did not specify how the 1ms worth of PRACH shall be collected by the SS, this procedure enforces a stricter criteria of requiring consecutive PRACH worth 1ms

So, once a UE meets the conformance requirement, one can be certain it will work in the real field deployment as well which is the primary goal of this Work Item.

Proposal 1: Endorse the approach described in this paper for TC 6.6.3 without UE beamlock_idle function.

## 3.	Conclusion

Proposal 1: Endorse the approach described in this paper for TC 6.6.3 without UE beamlock_idle function.

## 4.		References

[1]		R4-2317764, LS on testability issues of beam correspondence requirement for initial access, Huawei

[2]		R5-243159, Discussion on FR2 beam correspondence for initial access, Huawei

[3]	TS 38.101-2 v18.8.0

[4]	R5-245297, UE beam correspondence for initial access, Qualcomm