3GPP TSG-RAN5 Meeting #108	R5-254065

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:	5.3.19.10

Source:	Keysight Technologies UK Ltd

Title:	Discussion on FR1 MUs for 4Tx test cases

Document for:	Discussion and Endorsement

# Introduction

This contribution aims to perform MU and TT analysis for the FR1 4Tx test cases being added under the work plan NR_ENDC_RF_FR1_enh2-UEConTest.

# Discussion

## Frequency range

In previous meeting, MU/TT were already defined for multiple 4Tx UL MIMO and TxD test cases, by leveraging the MU/TT already defined for the equivalent 2Tx UL MIMO or TxD test cases.

Doing just references to other test cases rather than defining explicitly the MU and TTs values has the risk that updates in the referred test cases will automatically propagate to the referring test cases even if not analysed in detail.

MU/TT agreed in last meeting were assuming f ≤ 6GHz. While not explicitly analysed, MU/TT for 4Tx test cases shall be assumed FFS for f > 6GHz.

Proposal 1. While not explicitly analysed, MU/TT for 4Tx test cases shall be assumed FFS for f > 6GHz.

## Power classes

According to current 4Tx test cases defined, their applicability is limited to “all types of NR Power Class 1.5 UE release 18 and forward that support 4-layer codebook based UL MIMO” or “all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx Tx diversity”. While no higher power classes are considered, any MU/TT analysis shall be assumed valid only for UEs with power up to PC1.5 maximum output power.

Proposal 2. While not explicitly analysed, MU/TT for 4Tx test cases shall be assumed FFS for UE powers greater than PC1.5 maximum output power.

## TX Tests analysis

#### High power tests (in-channel)

In previous meeting the MU/TT for the 4Tx MOP and MPR tests was already defined same as corresponding MOP and MPR SISO tests. In these 4TX UL-MIMO/TxD tests, the requirement applies to the sum of the mean power of the UE at each antenna connector. Despite the 6dB SNR degradation due to combination of the signal from 4 measurements, one per antenna connector, the influence of noise is still very low and for that reason the MU/TT limits already defined for the MOP SISO test 6.2.1 can be leveraged.

Other 4Tx test cases with pending MU/TT definition and measuring also a relatively high power are the A-MPR test, the configured output power test, the absolute power tolerance test and the aggregate power tolerance test. In those test cases, lower power to measure is about -13.2dBm/100MHz (absolute power test). In such a case, after analysing the performance of our test system, we conclude that MU/TT definition for them can be leveraged from the MU/TT defined for the equivalent SISO tests.

Proposal 3. For the 4Tx A-MPR tests 6.2D.3_2 and 6.2G.3_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.2.3 for the sum of power at each of UE antenna connector.

Proposal 4. For the 4Tx configured transmit power tests 6.2D.4_2 and 6.2G.4_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.2.4 for the sum of power at each of UE antenna connector.

Proposal 5. For the 4Tx absolute power tolerance tests 6.3D.4.1_2 and 6.3G.4.1_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.3.4.2 for the sum of power at each of UE antenna connector.

Proposal 6. For the 4Tx aggregate power tolerance tests 6.3D.4.3_2 and 6.3G.4.3_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.3.4.4 for the sum of power at each of UE antenna connector.

#### Low power tests (in-channel)

- In this subsection the minimum output power tests, the transmit (ON/)OFF power test cases and the relative power control tests will be analyzed.

- In the case of OFF power, the requirement is defined per antenna connector with the same limit as defined for single antenna test. As a result, the MU/TT for this measurement can be leveraged directly from the OFF power SISO test 6.3.2.

- To measure the ON power in the ON/OFF power tests / time mask tests, the requirement applies to the sum of the mean power of the UE at each antenna connector. The ON power is the same as defined and tested in the 4Tx MOP test case.

- Proposal 7. For the OFF power measurement in the 4Tx tests 6.3D.2_2, 6.3D.3_2, 6.3G.2_1, 6.3G.3.1_1, 6.3G.3.2_1, 6.3G.3.3_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.3.2 for each antenna connector.

- Proposal 8. For the ON power measurement in the 4Tx tests 6.3D.3_2, 6.3G.3.1_1 and 6.3G.3.3_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 4Tx MOP tests 6.2D.1_2 and 6.2G.1_1 for UL MIMO and TxD respectively.

- Regarding this latest proposal, while doing the analysis it has been observed that there is an error in the MU definition of the 2Tx TxD test case 6.3G.3.3 for SRS time mask. As per test definition, expected ON power will be the same as defined in 6.2G.1. However, current MU definition does not make any differentiation for the ON power, what should be corrected and defined same as defined for general ON/OFF time mask TxD test 6.3G.3.1.

Proposal 9. For the 2Tx TxD SRS time mask test 6.3G.3.3, define MU same as defined for general ON/OFF time mask TxD test 6.3G.3.1.

- For the test cases measuring UL power set to or close to the minimum output power and the requirement applies to the sum of power at each of UE antenna connector, more time is required to do the MU and TT evaluation.

#### Channel quality tests

- Let’s start the analysis with the EVM, carrier leakage, in-band emissions and EVM equalizer tests. In the corresponding 2Tx tests, with some minor differences between UL MIMO and TxD, in general de MU is defined same as defined for the equivalent SISO applying the requirement for each antenna for the channel quality measurement. In addition, in these test cases, UL power control is used to set the UL power to certain values so the MU applicable to set the UL power is defined by referring to the MU defined in the corresponding 2Tx MOP test, minimum output power test or relative output power test depending on power levels required in each test case.

- In the case of the 4Tx channel quality tests, for the channel quality measurements, MU and TT can be defined the same as defined in the equivalent 2Tx tests, that is, same as defined for the equivalent SISO applying the requirement for each antenna for the channel quality measurement. However, regarding the channel power measurement MU, the corresponding 4Tx MOP test, minimum output power test or relative output power test, depending on power levels required in each test case, should be referred instead.

- The mentioned exceptions between UL MIMO and TxD are mainly for the EVM TxD measurement where the final value is obtained weighing the EVM measured at each connector with the power measured at it.

Proposal 10. For the quality measurement in the 4Tx channel quality tests 6.4D.2.1_2, 6.4D.2.2_2, 6.4D.2.3_2, 6.4D.2.4_2, 6.4G.2.1_1¸ 6.4G.2.2_1, 6.4G.2.3_1 and 6.4G.2.4_1, for f ≤ 6GHz and CBW ≤ 100MHz,  define MU/TT same as defined for equivalent SISO tests in section 6.4.2 for each antenna connector applying for the test case 6.4G.2.1_1 the required clarifications aligned to those for test 6.4G.2.1.

Proposal 11. For the power measurements in the 4Tx channel quality tests 6.4D.2.1_2, 6.4D.2.2_2, 6.4D.2.3_2, 6.4D.2.4_2, 6.4G.2.1_1¸ 6.4G.2.2_1, 6.4G.2.3_1 and 6.4G.2.4_1, for f ≤ 6GHz and CBW ≤ 100MHz,  define MU/TT same as defined for the corresponding 4Tx maximum, minimum and relative power tests depending on the case, applying for the test case 6.4G.2.1_1 the required clarifications aligned to those for test 6.4G.2.1.

- Regarding this latest proposal, while doing the analysis it has been observed that there is an error in the MU definition of the 2Tx TxD channel quality tests, because they refer to the SISO relative power test instead of the corresponding UL TxD test, what should be corrected because the power control is applied to the sum of power at each of UE antenna connector.

Proposal 12. For the power measurements in the 2Tx TxD channel quality tests, where applicable, define MU for the relative uplink power measurement same as defined in test case 6.3G.4.2.

- In addition, we have observed that the MU definition for the 2Tx TxD relative power control test 6.3G.4.2 is not correct and should be the same as defined for the 2Tx UL MIMO relative power control test 6.3D.4.2.

Proposal 13. For the 2Tx TxD relative power control test 6.3G.4.2, define MU same as defined for the 2Tx UL MIMO relative power control test 6.3D.4.2.

- For the particular test to measure the time alignment error for UL MIMO, the minimum requirements are defined per antenna pairs, therefore the MU and TT defined for the 2Tx UL MIMO test can be leveraged to define the MU and TT in the equivalent 4Tx test for each antenna pair.

- Proposal 14. For the 4Tx Time alignment error for UL MIMO test 6.4D.3_2, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.4.D.3 for each antenna pair.

#### Out of band and spurious emissions tests

- In all these test cases (SEM, ACLR, general spurious, additional spurious…) the requirements apply to the to the sum of the mean power of the UE at each antenna connector.  The powers to measure are remarkable low in absolute or relative terms and the configuration of the test equipment is more complex compared to an in-channel power measurement.  For those reasons, more time is required to analyze the MU and TT for these test cases.

#### Transmit intermodulation tests

- In both UL MIMO and TxD scenarios, the transmit intermodulation requirements are specified at each transmit antenna connector. Therefore, current MU and TTs defined for the 1Tx (SISO) tests can be leveraged for the 4Tx tests as already leveraged for the 2Tx tests for each antenna connector.

- Proposal 15. For the transmit intermodulation 4Tx tests 6.5D.4_2 and 6.5G.4_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.5.4 for each antenna connector.

Proposal 16

## RX Tests analysis

- In the Rx test cases, typically the UE power is set to maximum power or very close to, for example set to Pmax – 4dB. Pending 4Tx Rx tests to be defined are the wideband intermodulation UL MIMO and TxD tests. In those conditions, it is proposed to define MU and TT same as defined for test 7.8.2 except for the UL power measurement uncertainty that shall be defined same as corresponding 4Tx MOP test.

- Proposal 17. For the wideband intermodulation 4Tx tests 7.8D.2_2 and 7.8G.1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for test 7.8.2 except for the UL power measurement uncertainty that shall be defined same as corresponding 4Tx MOP tests 6.2D.1_2 and 6.2G.1_1 respectively.

# Conclusion

The following observations and proposals were made in this contribution.

Proposal 1. While not explicitly analysed, MU/TT for 4Tx test cases shall be assumed FFS for f > 6GHz.

Proposal 2. While not explicitly analysed, MU/TT for 4Tx test cases shall be assumed FFS for UE powers greater than PC1.5 maximum output power.

Proposal 3. For the 4Tx A-MPR tests 6.2D.3_2 and 6.2G.3_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.2.3 for the sum of power at each of UE antenna connector.

Proposal 4. For the 4Tx configured transmit power tests 6.2D.4_2 and 6.2G.4_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.2.4 for the sum of power at each of UE antenna connector.

Proposal 5. For the 4Tx absolute power tolerance tests 6.3D.4.1_2 and 6.3G.4.1_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.3.4.2 for the sum of power at each of UE antenna connector.

Proposal 6. For the 4Tx aggregate power tolerance tests 6.3D.4.3_2 and 6.3G.4.3_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.3.4.4 for the sum of power at each of UE antenna connector.

Proposal 7. For the OFF power measurement in the 4Tx tests 6.3D.2_2, 6.3D.3_2, 6.3G.2_1, 6.3G.3.1_1, 6.3G.3.2_1, 6.3G.3.3_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.3.2 for each antenna connector.

Proposal 8. For the ON power measurement in the 4Tx tests 6.3D.3_2, 6.3G.3.1_1 and 6.3G.3.3_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 4Tx MOP tests 6.2D.1_2 and 6.2G.1_1 for UL MIMO and TxD respectively.

Proposal 9. For the 2Tx TxD SRS time mask test 6.3G.3.3, define MU same as defined for general ON/OFF time mask TxD test 6.3G.3.1.

Proposal 10. For the quality measurement in the 4Tx channel quality tests 6.4D.2.1_2, 6.4D.2.2_2, 6.4D.2.3_2, 6.4D.2.4_2, 6.4G.2.1_1¸ 6.4G.2.2_1, 6.4G.2.3_1 and 6.4G.2.4_1, for f ≤ 6GHz and CBW ≤ 100MHz,  define MU/TT same as defined for equivalent SISO tests in section 6.4.2 for each antenna connector applying for the test case 6.4G.2.1_1 the required clarifications aligned to those for test 6.4G.2.1.

Proposal 11. For the power measurements in the 4Tx channel quality tests 6.4D.2.1_2, 6.4D.2.2_2, 6.4D.2.3_2, 6.4D.2.4_2, 6.4G.2.1_1¸ 6.4G.2.2_1, 6.4G.2.3_1 and 6.4G.2.4_1, for f ≤ 6GHz and CBW ≤ 100MHz,  define MU/TT same as defined for the corresponding 4Tx maximum, minimum and relative power tests depending on the case, applying for the test case 6.4G.2.1_1 the required clarifications aligned to those for test 6.4G.2.1.

Proposal 12. For the power measurements in the 2Tx TxD channel quality tests, where applicable, define MU for the relative uplink power measurement same as defined in test case 6.3G.4.2.

Proposal 13. For the 2Tx TxD relative power control test 6.3G.4.2, define MU same as defined for the 2Tx UL MIMO relative power control test 6.3D.4.2.

Proposal 14. For the 4Tx Time alignment error for UL MIMO test 6.4D.3_2, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.4.D.3 for each antenna pair.

Proposal 15. For the transmit intermodulation 4Tx tests 6.5D.4_2 and 6.5G.4_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.5.4 for each antenna connector.

Proposal 17. For the wideband intermodulation 4Tx tests 7.8D.2_2 and 7.8G.1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for test 7.8.2 except for the UL power measurement uncertainty that shall be defined same as corresponding 4Tx MOP tests 6.2D.1_2 and 6.2G.1_1 respectively.

# References

- TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v19.0.0 (2025-06)

- TS 38.101-1, “User Equipment (UE); Radio transmission and reception; Part 1: Range 1 Standalone”, v18.10.0 (2025-06)