3GPP TSG-RAN5 Meeting #108	R5-255111

# Bengaluru, IN, 25 - 29 Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5148 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.3.3.5 Steering of Roaming in SNPN / After registration / Security check successful / Access for localized services in SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2-NGRAN_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-8-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new test case 6.3.3.5 Steering of Roaming in SNPN / After registration / Security check successful / Access for localized services in SNPN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 6.3.3.5 Steering of Roaming in SNPN / After registration / Security check successful / Access for localized services in SNPN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case cannot run normally |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0621 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-255003. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

#### 6.3.3.5	Steering of Roaming in SNPN / After registration / Security check successful / Access for localized services in SNPN

6.3.3.5.1	Test Purpose (TP)

(1)

with { UE supports access to an SNPN providing access for localized services in SNPN and has registered onto a SNPN }

ensure that {

when { SOR Transparent container included in DL NAS TRANSPORT message contains steering of roaming information and indicates ACK has been requested and security check is successful }

then { UE sends an SOR transparent container with ACK in UL NAS TRANSPORT message and indicates ME support of SOR-SNPN-SI-LS }

}

6.3.3.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.122 clause C.6. Unless otherwise stated these are Rel-18 requirements.

[TS 23.122, clause C.6]

The stage-2 flow for the steering of UE in SNPN after registration is indicated in figure C.6.1. The UE is registered on an SNPN which can be the subscribed SNPN or a non-subscribed SNPN. The AMF is located in the selected SNPN. The UDM is located in the HPLMN or subscribed SNPN.

…

Figure C.6.1: Procedure for providing SOR-SNPN-SI (if any) and SOR-SNPN-SI-LS (if any) after registration

For the steps below, security protection is described in 3GPP TS 33.501 [66].

1)	The SOR-AF to the UDM: Nudm_ParameterProvision_Update request is sent to the UDM to trigger the update of the UE with the SOR-SNPN-SI,if any, the SOR-CMCI, if any, the "Store the SOR-CMCI in the ME" indicator, if any, and the SOR-SNPN-SI-LS, if any, for a UE identified by SUPI.

2)	The UDM to the AMF: The UDM notifies the changes of the user profile to the affected AMF by the means of invoking Nudm_SDM_Notification service operation. The Nudm_SDM_Notification service operation contains the steering of roaming information that needs to be delivered transparently to the UE over NAS within the Access and Mobility Subscription data. If the subscribed SNPN or HPLMN decided that the UE is to acknowledge successful security check of the received steering of roaming information, the Nudm_SDM_Notification service operation also contains an indication that the UDM requests an acknowledgement from the UE as part of the steering of roaming information. If the SOR-SNPN-SI was obtained, the UDM shall include the SOR-SNPN-SI in to the SOR information. If the SOR-CMCI was obtained, the UDM shall include the SOR-CMCI into the steering of roaming information and shall requests an acknowledgement from the UE as part of the steering of roaming information. If the "Store the SOR-CMCI in the ME" indicator was obtained, the UDM shall include the "Store the SOR-CMCI in the ME" indicator If the SOR-SNPN-SI-LS was obtained, the UDM shall include the SOR-SNPN-SI-LS into the SOR information;

NOTE 4:	The UDM cannot provide the SOR-SNPN-SI SOR-CMCI, or SOR-SNPN-SI-LS to the AMF which does not support receiving SOR transparent container (see 3GPP TS 29.503 [78]).

3)	The AMF to the UE: the AMF sends a DL NAS TRANSPORT message to the served UE. The AMF includes in the DL NAS TRANSPORT message the steering of roaming information received from the UDM.

4)	Upon receiving the steering of roaming information, the UE shall perform a security check on the steering of roaming information included in the DL NAS TRANSPORT message to verify that the steering of roaming information is provided by the subscribed SNPN or HPLMN, and

i)	if the security check is successful, then:

a)	if the steering of roaming information contains the SOR-SNPN-SI, the ME shall replace the credentials holder controlled prioritized list of preferred SNPNs for the selected entry of the "list of subscriber data" or the selected PLMN subscription with the received credentials holder controlled prioritized list of preferred SNPNs, if any, the ME shall replace the credentials holder controlled prioritized list of GINs for the selected entry of the "list of subscriber data" or the selected PLMN subscription with the received credentials holder controlled prioritized list of GINs, if any, and the ME shall delete the SNPNs identified by the credentials holder controlled prioritized list of preferred SNPNs or the SNPN(s) stored along with GIN(s) identified by the credentials holder controlled prioritized list of GINs from the list of "temporarily forbidden SNPNs" and the list of "permanently forbidden SNPNs", if they are present in these lists. If the SOR information contains the SOR-SNPN-SI-LS, the ME shall replace the "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN" for the selected entry of the "list of subscriber data" or the selected PLMN subscription with the received "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN", if any, and the ME shall replace the "credentials holder controlled prioritized list of preferred GINs for access for localized services in SNPN" for the selected entry of the "list of subscriber data" or the selected PLMN subscription with the received "credentials holder controlled prioritized list of preferred GINs for access for localized services in SNPN", if any, and delete the SNPNs identified by the "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN" or the SNPN(s) stored along with GIN(s) identified by the "credentials holder controlled prioritized list of preferred GINs for access for localized services in SNPN" from the list of "temporarily forbidden SNPNs for access for localized services in SNPN" and the list of "permanently forbidden SNPNs for access for localized services in SNPN", if they are present in these lists;

b)	if the UDM has requested an acknowledgement from the UE in the DL NAS TRANSPORT message, the UE sends an UL NAS TRANSPORT message to the serving AMF with an SOR transparent container including the UE acknowledgement and the UE shall set:

-	the "ME support of SOR-CMCI" indicator to "supported"; and

-	the "ME support of SOR-SNPN-SI-LS" indicator to "supported" if the UE supports access to an SNPN providing access for localized services in SNPN.

c)	if the UE is in automatic network selection mode, the selected SNPN is a non-subscribed SNPN and the UE decides to perform SNPN selection, then:

-	if the UE is configured with the SOR-CMCI or received the SOR-CMCI over N1 NAS signalling, the UE shall apply the actions in clause C.4; or

-	the UE shall wait until it moves to idle mode or 5GMM-CONNECTED mode with RRC inactive indication (see 3GPP TS 24.501 [64]) before attempting to obtain service on a higher priority SNPN as specified in clause 4.9.3.

d)	if the selected SNPN is a non-subscribed SNPN and the UE has an established emergency PDU session then the UE may attempt to perform the SNPN selection subsequently after the emergency PDU session is released, if the UE is in automatic network selection mode.

e)	if the UDM has not requested an acknowledgement from the UE, then steps 4 is skipped; and

ii)the security check is not successful:

a)	step 5 is skipped; and

b)	if the selected SNPN is a non-subscribed SNPN and the UE is in automatic network selection mode, then:

A)	if the UE has a stored SOR-CMCI, then:

-	if there are ongoing PDU sessions or services, the UE shall apply the actions in clause C.4.2, and the current SNPN is considered as lowest priority; or

-	if there are no ongoing PDU sessions or services, the UE shall release the current N1 NAS signalling connection locally and attempt to obtain service on a higher priority SNPN as specified in clause 4.9.3, with an exception that the current SNPN is considered as lowest priority;

B)	if the UE does not have a stored SOR-CMCI, then:

-	if there are ongoing PDU sessions or services, the UE shall wait until it moves to idle mode or 5GMM-CONNECTED mode with RRC inactive indication (see 3GPP TS 24.501 [64]) before attempting to obtain service on a higher priority SNPN as specified in clause 4.9.3, with an exception that the current SNPN is considered as lowest priority. If the selected SNPN is a non-subscribed SNPN and the UE has an established emergency PDU session, then the UE shall attempt to perform the SNPN selection after the emergency PDU session is released; or

-	if there are no ongoing PDU sessions or services, the UE shall release the current N1 NAS signalling connection locally and attempt to obtain service on a higher priority SNPN as specified in clause 4.9.3, with an exception that the current SNPN is considered as lowest priority.

NOTE 5:	When the UE is in the manual mode of operation and the current chosen non-subscribed SNPN is part of the user controlled prioritized list of preferred SNPNs, the UE stays on the current chosen non-subscribed SNPN.

6.3.3.5.3	Test description

6.3.3.5.3.1	Pre-test conditions

System Simulator:

-	1 SNPN cell: NR Cell 11 is configured broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-26 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 11.

-	NR Cell 11 is set to “Serving Cell”.

UE:

-	The UE is in Automatic SNPN selection mode.

-	The UE is provisioned with a “list of subscriber data” to allow access to SNPN identified by NR cell 11.

Preamble:

-	Ensure that the UE has cleared the Registered SNPN. And the UE is in state Switched OFF (state 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

6.3.3.5.3.2	Test procedure sequence

Table 6.3.3.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE | - | - | - | - |
| 2 | Whole registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 11, with “connected without release” | - | - | - | - |
| 3 | The SS transmits an DLInformationTransfer message containing steering of roaming information indicating that acknowledgment is requested from the UE for successful reception. | <-- | NR RRC: DLInformationTransfer 5G MM: DL NAS TRANSPORT | - | - |
| 4 | Check: Does the UE transmit an ULInformationTransfer message carrying acknowledgement and indication of support of localized services in SNPN? | --> | NR RRC: ULInformationTransfer 5G MM: UL NAS TRANSPORT | 1 | P |

6.3.3.5.3.3	Specific message contents

Table 6.3.3.5.3.3-1: DL NAS TRANSPORT Message (Step 3, Table 6.3.3.5.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.7.1-11 with condition 5GS_SOR_CMCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Payload container | Present | The SOR transparent container in the payload container IE carries steering of roaming information. |  |
| Payload container contents |  |  |  |
| SOR Transparent Container | Present | The SOR transparent container carries steering of roaming information. |  |
| SOR header |  |  |  |
| List indication value | ‘0’B | no list of preferred PLMN/access technology combinations is provided |  |
| Acknowledgement (ACK) value | ‘1’B | Acknowledgement is requested |  |
| Additional parameters (AP) value | ‘1’B | Additional parameters is included |  |
| Length of PLMN ID and access technology list | Not present |  |  |
| PLMN ID and access technology list | Not present |  |  |
| SOR-CMCI indicator (SI) value | ‘1’B | SOR-CMCI present |  |
| SOR-SNPN-SI-LS indicator (SSSLI) value | 0 | SOR-SNPN-SI-LS absent |  |
| SOR-CMCI | Present |  |  |
| Length of SOR-CMCI contents |  | Length value of SOR-CMCI rule1 |  |
| SOR-CMCI rule 1 |  |  |  |
| Length of SOR-CMCI rule contents |  | Length value of SOR-CMCI rule contents |  |
| Tsor-cm timer value | ‘00100001’B | 60s |  |
| Criterion type | ‘11111111’B | match all |  |
| Criterion value | Not Present |  |  |

Table 6.3.3.5.3.3-2: UL NAS TRANSPORT Message (Step 4, Table 6.3.3.5.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.7.1-10 with condition 5GS_SOR_CMCI |  |  |  |
| --- | --- | --- | --- |
| ME support of SOR-CMCI indicator (MSSI) value | ‘1’B | SOR-CMCI supported by the ME |  |
| ME support of SOR-SNPN-SI-LS indicator (MSSNPNSILS) value | ‘1’B | SOR-SNPN-SI-LS supported by the ME |  |

<End of Changed Section>