3GPP TSG-RAN5 Meeting #108	R5-254049r2

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.544 | CR | 0037 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Test Equipment Reference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Spirent Communications, ETS-Lindgren |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_MIMO_OTA-UEConTest |  |  |  |  |  | Date: |  |  | 08-08-2025 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause E.19 incorrectly references the wrong test equipment |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the test equipment reference from BS simulator to channel emulator |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case execution can lead to confusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | N.1, N.4 (New) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-254049r1: saved with docx extension R5-254049r2: defined MU element ‘Throughput measurement: output level step Resolution’ in Annex N.4 |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

# N.1	General

Editor’s notes: Table N.1-1 is based on a similar table in RAN4 TR 37.977. There are a number of inconsistencies (examples are listed below) and errors in the source table that are still remaining in Table N.1-1. They should be fixed once RAN4 fixes the corresponding table in TR 37.977.

-	NOTEs 1, 3 & 4 are not relevant to MPAC; the NOTEs should be removed.

-	Item 19 is not relevant to MPAC and should be removed.

-	Items 9 and 10 seem to be related to SIR that is out of scope of the WI; these may be removed from the table.

- Individual uncertainty contributions in the TRMS measurements are discussed and evaluated in this Annex. A technique for calculating the total measurement uncertainty is also presented.

- An important part of a standard measurement procedure is the identification of uncertainty sources and the evaluation of the overall measurement uncertainty. There are various individual uncertainty sources in the measurement procedure that introduce a certain uncertainty contribution to the final measurement result. The approach in this standard test procedure is that the test laboratories are not limited to using some specific instruments and antenna positioners, for example. However, a limit is set for the maximum overall measurement uncertainty.

- The TRMS measurement procedure can be considered to include two stages. In Stage 1 the actual measurement of the MIMO OTA throughput of the Device Under Test (DUT) is performed. In Stage 2 the calibration of the absolute level of the DUT measurement results is performed.

- The uncertainty contributions related to TRMS are listed in Tables N.1-1. The uncertainty contributions are analysed in the following paragraphs.

- The calculation of the uncertainty contribution is based on the ISO Guide to the expression of uncertainty in measurement. Each individual uncertainty is expressed by its Standard Deviation (termed here as ‘standard uncertainty’) and represented by symbol U. The uncertainty contributions can be classified to two categories: Type-A uncertainties, which are statistically determined e.g. by repeated measurements, and Type-B uncertainties, which are derived from existing data e.g. data sheets. Several individual uncertainties are common in Stage 1 and Stage 2 and therefore cancel.

- The procedure of forming the uncertainty budget in measurement is:

    - 1)	Compile lists of individual uncertainty contributions for measurements in both Stage 1 and Stage 2.

- 2)	Determine the standard uncertainty of each contribution by

- a)	Determining the distribution of the uncertainty (Gaussian, U-shaped, rectangular, etc.)

- b)	Determining the maximum value of each uncertainty (unless the distributions is Gaussian)

- c)	Calculating the standard uncertainty by dividing the uncertainty by  if the distribution is U-shaped, and by  if the distribution is rectangular.

- 3)	Convert the units into decibel, if necessary.

- 4)	Combine all the standard uncertainties by the Root of the Sum of the Squares (RSS) method.

- 5)	Combine the total uncertainties in Stage 1 and Stage 2 also by the RSS method: .

6)	Multiply the result by an expansion factor of 1.96 to derive expanded uncertainty at 95% confidence level: 1.96 * .

Table N.1-1: Measurement uncertainty budget for Multiprobe Anechoic method

|  | Description of uncertainty contribution | Details in | MPAC |  |  |
| --- | --- | --- | --- | --- | --- |
| # | Stage 1-DUT measurement |  | Example Value [dB] | Prob Distr | Std Uncertainty [dB] |
| 1 | Mismatch of transmitter chain (i.e. between fixed measurement antenna and base station simulator) | TS 34.114, E.1-E.2 | 0.00 | u-shape | 0.00 |
| 2 | Insertion loss of transmitter chain | TS 34.114, E.3-E.5 | 0.00 | rect | 0.00 |
| 3 | Influence of the fixed measurement antenna cable | TS 34.114, E.6 | 0.00 | rect | 0.00 |
| 4 | Uncertainty of the absolute antenna gain of the fixed measurement antenna | TS 34.114, E.7 | 0.00 | rect | 0.00 |
| 5 | Base station simulator: uncertainty of the absolute output level | TS 34.114, E.17 [TS 36.521-1 F.1.3] | 1.00 | rect | 0.58 |
| 6 | Throughput measurement: output level step resolution | N.4 | 0.25 | rect | 0.14 |
| 7 | Statistical uncertainty of throughput measurement | TS34.114, E.19 | FFS (negligible and partially included in repeatability) |  |  |
| 8 | Fading channel emulator output uncertainty (if used) - absolute output power - output signal stability - output stability with temperature | N.2 | 1.5dB 0.5dB 0.4dB | normal (output power) rect (stability) | 0.84 |
| 9 | AWGN flatness within LTE band | TBD (NOTE 4) |  |  | FFS |
| 10 | Signal-to noise ratio uncertainty, averaged over downlink transmission Bandwidth | TBD (NOTE 4) |  |  | FFS |
| 11 | Channel model implementation (NOTE 2) | TBD | FFS |  | FFS |
| 12 | Chamber statistical ripple and repeatability | TS 34.114, E.26.A | N/A |  | 0.00 |
| 13 | Additional power loss in EUT chassis | TS 34.114, E.26.B | N/A |  | 0.00 |
| 14 | Quality of the quiet zone | TS 34.114, E.10 | 0.50 | std | 0.50 |
| 15 | Measurement Distance - VSWR - Chamber Standing Wave | TS 34.114, E.9 | 0.00 |  | 0.00 |
| 16 | DUT sensitivity drift | TS 34.114, E.21 | 0.20 | rect | 0.12 |
| 17 | Uncertainty related to the use of the phantoms: |  |  |  | 0.00 |
|  | a) Uncertainty of dielectric properties and shape of the hand phantom | TR 25.914, A.12.3 |  |  | 0.00 |
|  | b) Uncertainty related to the use of laptop ground plane phantom | TR 25.914, A.12.4 |  |  | 0.00 |
| 18 | Random uncertainty (repeatability) | TS 34.114, E.14 | 0.20 | rect | 0.12 |
| 19 | Uncertainty associated with the stirring method and number of subframes (NOTE 3) |  | N/A |  | 0.00 |
|  | Stage 2-Calibration measurement |  |  |  | 0.00 |
| 20 | Uncertainty of network analyzer - Receiver and Source VNA - Receiver VNA Calibration prior to measurement | TS 34.114, E.15 | 0.50 | rect | 0.29 |
| 21 | Mismatch of transmitter chain | TS 34.114, E.1-E.2 | 0.20 | u-shape | 0.14 |
| 22 | Insertion loss of transmitter chain | TS 34.114, E.3-E.5 | 0.00 |  | 0.00 |
| 23 | Mismatch in the connection of calibration antenna | TS 34.114, E.1 | 0.00 | rect | 0.00 |
| 24 | Influence of the calibration antenna feed cable | TS 34.114, E.6 |  |  | 0.00 |
| 25 | Influence of the transmitter antennas/probes cables | TS 34.114, E.6 | 0.00 | rect | 0.00 |
| 26 | Uncertainty of the absolute gain of the transmitter antennas/probes | TS 34.114, E.7 | 0.00 | rect | 0.00 |
| 27 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | TS 34.114, E.16 | 0.50 | std | 0.50 |
| 28 | Chamber statistical ripple and repeatability | TS 34.114, E.26.A |  |  | 0.00 |
| 29 | Phase Centre Offset (when using horn to calibrate) | TS 34.114, E.9 | 0.00 | rect | 0.00 |
| 30 | Quality of the quiet zone (Range Ref. Antenna) | TS 34.114, E.10 | 0.50 | rect | 0.29 |
|  | External Amplifiers |  |  |  | 0.00 |
| 31 | Stability | N.3.1 | 0.30 | rect | 0.17 |
| 32 | Linearity | N.3.2 | 0.10 | rect | 0.06 |
| 33 | Noise Figure | N.3.3 | 0.30 | rect | 0.17 |
| 34 | Mismatch | N.3.4 | 0.00 | rect | 0.00 |
| 35 | Gain | N.3.5 | 0.00 | rect | 0.00 |

NOTE 1:	0dB if fading for RTS is done in baseband; same as RC&CE and MPAC if fading is not in baseband

NOTE 2:	assumption is that MU set to 0dB with channel model validation pass/fail limits (FFS) that have negligible impact on TP FOM; MU for channel model validation is FFS

NOTE 3:	Analysis of the element associated with stirring method and number of subframes is based on existing harmonization test campaign data and can be further augmented by additional measurements. The following combinations of stirring modes and number of subframes have been identified as common use cases with the following standard uncertainties (different combinations require separate validation):

A: stepped stirring mode with 20k SF per stirring state: 0dB

B: stepped stirring mode with 400 SF per stirring state: 0.22dB

C: continuous stirring mode with 20k SF per sample: FFS

D: continuous stirring mode with 400 SF per sample: FFS

Until MU elements for continuous stirring modes have been defined, the test plan shall only consider stepped stirring approach

NOTE 4:	As the applicability of SIR to MIMO OTA performance evaluation is FFS, the measurement uncertainty treatment for SIR related items will remain FFS. When the applicability of SIR is confirmed, the measurement uncertainty treatment defined in 3GPP TS 36.521-1 [11] Table F.1.4-1 for line item 8.2.1.3.1 should be considered along with the related test system constraints. Any adjustments to the test system limits or uncertainty definitions necessary for MIMO OTA performance testing should be applied.

# N.2	Fading channel emulator output uncertainty

In the case where a fading channel emulator is used, the outputs of the fading channel emulator are used to drive signals through the amplifiers, if necessary, to the probe antennas in the MIMO OTA tests either as an absolute level or as a relative level. The receiving device used is a UE. Generally, an uncertainty contribution comes from the absolute level accuracy, non-linearity, and output signal stability of the fading channel emulator. All of the contributions should be considered. However, the applicability of each contribution is contingent on the specific measurement implementation and calibration procedure.

This uncertainty should be determined from the manufacturer’s data sheet expressed in dB with a rectangular distribution, unless otherwise informed (see clause 5.1.2 in [33]). Furthermore, the uncertainty of the non-linearity and the output signal stability of the device may be included in the absolute level uncertainty. If not, the non-linearity, output signal stability and output signal stability versus temperature should be determined from the manufacturer’s data sheet. Once determined, the non-linearity uncertainty and the output signal stability uncertainties should be combined with the output uncertainty using the RSS method.

# N.3	External Amplifiers Uncertainty Terms

Any components in the setup can potentially introduce measurement uncertainty. It is then needed to determine the uncertainty contributors associated with the use of such components. For the case of external amplifiers, the following uncertainties should be considered but the applicability is contingent to the measurement implementation and calibration procedure.

## N.3.1	Stability

An uncertainty contribution comes from the output level stability of the amplifier. Even if the amplifier is part of the system for both measurement and calibration, the uncertainty due to the stability shall be considered. This uncertainty can be either measured or determined by the manufacturers’ data sheet for the operating conditions in which the system will be required to operate.

## N.3.2	Linearity

An uncertainty contribution comes from the linearity of the amplifier since in most cases calibration and measurements are performed at two different input/output power levels. This uncertainty can be either measured or determined by the manufacturers’ data sheet.

## N.3.3	Noise Figure

When the signal goes into an amplifier, noise is added so that the SNR at the output is reduced with regard to the SNR of the signal at the input. This added noise introduces error on the signal which affects the Error Rate of the receiver thus the EVM (Error Vector Magnitude). An uncertainty can be calculated through the following formula:

εEVM=20∙log⁡1+10-SNR20

where SNR is the signal to noise ratio in dB at the signal level used during the sensitivity measurement.

## N.3.4	Mismatch

If the external amplifier is used for both stages, measurement and calibration the uncertainty contribution associated with it can be considered systematic and constant -> 0dB. If it is not the case, the mismatch uncertainty at its input and output shall be either measured or determined by the method described in [3].

## N.3.5	Gain

If the external amplifier is used for both stages, measurement and calibration the uncertainty contribution associated with it can be considered systematic and constant -> 0dB. If it is not the case, this uncertainty shall be considered.

## N.4	Throughput measurement: output level step resolution

When output power of the channel emulator is swept to reach the TP target, used power step resolution creates this uncertainty. Output power step used in the TP measurement is divided by factor 2 to obtain the uncertainty with rectangular distribution.

<<< END OF CHANGES >>>