3GPP TSG-RAN5 Meeting #108	R5-254177

# Bangalore, India, 25th Aug 2025 – 29th Aug 2025

# Source:	Qualcomm Incorporated

# Title:	Duplicate conditions and their usage in test case selection expression

# Document for:	Discussion

## 1.	Introduction

3GPP TS 38.523-2 have duplicate conditions and these duplicates conditions are used for the same test case selection expression. This may create confusion to automation entity, readability and needs to be cleaned up for better clarity in the specification and remove ambiguity.

## 2.	Discussion

Below data\tables outlines the duplicate condition and how to remove redundancy to ensure no impact for the test case selection for a given DUT with specific set of feature support.

| Condition | Test case Selection Expression | Comment | Duplicate | Test case Selection Expression | Comment |
| --- | --- | --- | --- | --- | --- |
| C67 | IF A.4.1-3/2 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and Intra-Band Contiguous CA and EN-DC with 2 NR DL carriers | C44A | IF A.4.1-3/2 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and intra-band contiguous CA and EN-DC with 2 NR DL carriers |
| C68 | IF A.4.1-3/2 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and Intra-Band Non-Contiguous CA and EN-DC with 2 NR DL carriers | C46A | IF A.4.1-3/2 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and intra-band non-contiguous CA and EN-DC with 2 NR DL carriers |
| C69 | IF A.4.1-3/2 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and Inter-Band CA and EN-DC with 2 NR DL carriers | C45A | IF A.4.1-3/2 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A | UEs supporting EN-DC and inter-band contiguous CA and EN-DC with 2 NR DL carriers |
| C144 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.4-1/6 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and logged measurements in RRC_IDLE and RRC_INACTIVE | C32A | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.4-1/6 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and logged MDT |
| C159 | IF A.4.1-5/2 AND [10] A.4.1-1/5 THEN R ELSE N/A | UEs supporting 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment | C29 | IF A.4.1-5/2 AND [10] A.4.1-1/5 THEN R ELSE N/A | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| C397 | IF A.4.1-5/1 AND A.4.4-1/31 THEN R ELSE N/A | UEs supporting 5G Core and MT-SDT via Configured Grant Type 1 | C385 | IF A.4.1-5/1 AND A.4.4-1/31 THEN R ELSE N/A | UEs supporting 5G Core and MT SDT via Configured Grant Type 1 in RRC_INACTIVE state |

| Condition | Dependent Test Cases as per 38.523-2 | TTCN Implementation | Identified Duplicate Condition | Dependent Test Cases as per 38.523-2 | Associated TCs to Identical duplicate condition (Also used by TTCN) | Proposal 1 |
| --- | --- | --- | --- | --- | --- | --- |
| C29 | 17 Tests | 17 Tests | C159 | 10.3.1.1 | None | Delete C159 and change condition for TC10.3.1.1 as C29 |
| C32a | 8.1.6.2.3, 8.1.6.2.4 | 8.1.6.2.3, 8.1.6.2.4 | C144 | 8.1.6.2.2 | 8.1.6.2.2 | Delete C144 and change condition for TC8.1.6.2.2 as C32a |
| C44A | 7.1.1.7.1.1.ENDC | 7.1.1.7.1.1.ENDC | C67 | 8.2.4.1.1.1, 8.2.4.2.1.1 | 8.2.4.2.1.1, 8.2.4.1.1.1 | Delete C44A and change condition for TC 7.1.1.7.1.1.ENDC as C67 |
| C45A | 7.1.1.7.1.2.ENDC | 7.1.1.7.1.2.ENDC | C69 | 8.2.4.1.1.3, 8.2.4.2.1.3 | 8.2.4.1.1.3, 8.2.4.2.1.3 | Delete C45A and change condition for TC 7.1.1.7.1.2.ENDC as C69 |
| C46A | 7.1.1.7.1.3.ENDC | 7.1.1.7.1.3.ENDC | C68 | 8.2.4.1.1.2, 8.2.4.2.1.2 | 8.2.4.1.1.2, 8.2.4.2.1.2 | Delete C46A and change condition for TC 7.1.1.7.1.3.ENDC as C68 |
| C385 | 8.1.5.13.4 | 8.1.5.13.4 | C397 | 7.1.1.13.8 | 7.1.1.13.8 | Delete C397 and change condition for TC7.1.1.13.8 as C385 |

## 3.  Proposals/Way Forward

Proposal -1, TS 38.523-2

- Void C159 and change condition for TC10.3.1.1 as C29 – May change C29- WLAN Access

- Void C144 and change condition for TC8.1.6.2.2  as C32a

- Void C44A and change condition for TC 7.1.1.7.1.1.ENDC  as C67

- Void C45A and change condition for TC 7.1.1.7.1.2.ENDC   as C69

- Void C46A and change condition for TC 7.1.1.7.1.3.ENDC   as C68

- Void C397 and change condition for  TC7.1.1.13.8 as C385

Proposal -2

Investigate future checks either in TTCN or by other ways to avoid introducing duplicate conditions. There may be more such conditions which may need to be identified.

## 4.	References

[1]	TS 38.523-2		User Equipment (UE) conformance specification; Part 2: Applicability of protocol test cases (Release 19)

[2]	R5-254514	Removing Duplicate Conditions for test case selection expression