3GPP TSG-RAN5 Meeting #108	R5-254300

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4049 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of TT for test case 14.2.2.3.1. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TT for test case 14.2.2.3.1 is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added TT for 14.2.2.3.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TT for 14.2.2.3.1 will be still missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.903 CR 1060 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT, Depends on RAN4 CR R4-2509846 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

##### 14.2.2.3.1	NR SA FR1-FR1 RRC connection release with redirection for Satellite Access

Editor's Note: This test is incomplete in the following aspects:

- Message contents need to be updated

- Applicability needs to be updated

- Annex E and F need to be updated

14.2.2.3.1.1	Test purpose

This test is to verify RRC connection release with redirection from NR to NR requirements for Satellite Access specified in clause 14.2.2.3.0.

14.2.2.3.1.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.2.2.3.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 14.2.2.3.0.

The normative reference for this requirement is TS 38.133 [6] clause A.14.2.2.3.1.

14.2.2.3.1.4	Test description

14.2.2.3.1.4.1	Initial conditions

Supported test configurations are shown in table 14.2.2.3.1.4.1-1. The time delay is tested by using the parameters in table 14.2.2.3.1.4.1-3, and 14.2.2.3.1.5-1.

Table 14.2.2.3.1.4.1-1: Redirection from NR to NR test configurations

| Configuration | Description |
| --- | --- |
| 14.2.2.3.1-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.2.2.3.1-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.2.2.3.1.4.1-2.

Table 14.2.2.3.1.4.1-2: Initial conditions for NR SA FR1-FR1 RRC connection release with redirection for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.4.2.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 14.2.2.3.1.4.3.

2.	The power levels and settings for Cell 1 and Cell 2 are set according to Annex C.1.2 and C.1.3. Both Cell 1 and Cell 2 are satellite access cells.

3.	The test parameters are given in Table 14.2.2.3.1.4.1-3.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.2.2.3.1.4.1-3: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

14.2.2.3.1.4.2	Test Procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2. Cell 1 and Cell 2 belong to different tracking areas.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 14.2.2.3.1.5-1. T1 starts.

3.	SS shall transmit an RRCRelease during period T1.

4.	The SS shall switch the power setting from T1 to T2 as specified in Table 14.2.2.3.1.5-1. When the last TTI containing the RRCRelease message is sent to UE, T2 starts.

5.	If the UE transmits the PRACH to Cell 2 less than 2240 ms from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

6.	After T2 expires, the UE shall be switched off. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell and Cell 2 shall be powered OFF.

7.	The SS shall set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

8.	Repeat step 2-7 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

14.2.2.3.1.4.3	Message Contents

FFS

14.2.2.3.1.5	Test Requirement

Table 14.2.2.3.1.5-1 defines the primary level settings for Redirection from NR to NR test case for satellite access.

Table 14.2.2.3.1.5-1: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1, 2 |  | FDD |  |  |  |  |  |  |
| SSB Configuration |  | Config 1, 2 |  | SSB.1 FR1 |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1, 2 |  | TRS.1.1 FDD |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 2 |  | SR.1.1 FDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1, 2 |  | CR.1.1 FDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05+TT |  | -64.59 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 2240 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH,

Where:

TRRC_procedure_delay = 110 ms in the test.

Tidentify-NR = 680 ms in the test.

TSI-NR = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 [13] for the target NR cell.

TRACH = 170 ms in the test.

This gives a total of 2240 ms.

## <<< END OF CHANGES >>>