3GPP TSG-RAN5 Meeting #108	R5-254352

Bengaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4054 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Beam Failure detection and link recovery test case for a UE operating on a cell with 3 MHz BW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | # NR_FR1_lessthan_5MHz_BW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with 3 MHz BW has been missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with 3 MHz BW has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case would be missing in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.5.8(new), E.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.5	Signaling characteristics

### 6.5.5	Link recovery procedures

-------- Start of change 1--------

-------- Unchanged Clauses Skipped --------

#### 6.5.5.8	NR SA FR1 SSB-based Beam Failure Detection and Link Recovery in non-DRX mode for a UE operating on a cell with 3 MHz BW

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test tolerance analysis and measurement uncertainty are missing

6.5.5.8.1	Test purpose

The purpose of this test is to verify that the UE supporting support-3MHz-ChannelBW-r18 properly detects SSB-based beam failure in the set q0 configured for a serving cell operating on a less than 5 MHz bandwidth, and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5.

6.5.5.8.2	Test applicability

This test applies to all types of NR UE from Release 18 onwards for a UE operating on a cell with 3 MHz channel bandwidth.

6.5.5.8.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.5.8 and 8.5.

6.5.5.8.4	Test description

The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively. Figure 6.5.5.8.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate SSB based beam failure. Figure 6.5.5.8.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery

Figure 6.5.5.8.4-1: SNR and L1-RSRP variation for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

6.5.5.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.5.8.4.1-1.

Table 6.5.5.8.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth |

Configure the test equipment and the DUT according to the parameters in Table 6.5.5.8.4.1-2.

Table 6.5.5.8.4.1-2: Initial conditions for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode for PCell configured with 3 MHZ BW

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1. Message contents are defined in clause 6.5.5.8.4.3.

2. Single Cell is used, which is NR FR1 PCell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

3. General test parameters as specified in Table 6.5.5.1.4.1-3 with config 1 apply except those specified in Table 6.5.5.8.4.1-3.

4. Downlink signals for NR cell are initially set up according to Annex C.1.2 and C.1.3.

Table 6.5.5.8.4.1-3: General test parameters for FR1 in-sync testing in non-DRX mode

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| BWchannel |  | Config 1 | MHz | 3: NPRB,c = 15 |  |
| SSB Configuration |  | Config 1 |  | SSB.13 FR1 |  |
| PRACH Configuration |  | Config 1 |  | 1 |  |
| Beam failure detection transmission parameters | Number of Control OFDM symbols |  |  | 3 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Mapping from REG to CCE |  |  | Non-Distributed |  |
| T1 |  |  | s | 1 | During this time the UE shall be fully synchronized to Cell 1 |
| T2 |  |  | s | 5.17 |  |
| T3 |  |  | s | 3.24 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.94 |  |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

6.5.5.8.4.2	Test Procedure

Same as 6.5.5.1.4.2.

6.5.5.8.4.3	Message Contents

Same as 6.5.5.1.4.3.

6.5.5.8.5	Test Requirement

Cell specific test parameters as specified in Table 6.5.5.8.5-1 apply except those specified in Table 6.5.5.1.5-1. Test requirements specified in clause 6.5.5.1.5 apply to this test.

Table 6.5.5.8.5-1: Cell specific test parameters for FR1 PCell

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_SSB of set q0 | Config 1 | dB | 6+TT | -2+TT | -11+TT | -11+TT | -11+TT |

----- skipped unchanged clauses ------

-------- Start of change 2--------

# E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for NR/5GC FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 | NR SA FR1 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | NR Cell 6 | NR Cell 23 |  |  |  |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |
| … |  |  |  |  |  |  |
| 6.5.1.14 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW | NR Cell 1 |  |  |  |  |
| 6.5.1.15 | NR SA FR1 radio link monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.1.16 | NR SA FR1 Radio Link Monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.5.2.2 | SA FR1 interruptions at NR SRS carrier based switching | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| … |  |  |  |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 6.5.5.5 | NR SA FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.5.5.6 | NR SA FR1 SCell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.5.5.7 | NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX | NR Cell 1 |  |  |  |  |
| 6.5.5.8 | NR SA FR1 SSB-based Beam Failure Detection and Link Recovery in non-DRX mode for a UE operating on a cell with 3 MHz BW | NR Cell 1 |  |  |  |  |
| … |  |  |  |  |  |  |
| 6.7.9.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | NR Cell 1 |  |  |  |  |
| 6.7.9.3 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |

----- End of change 2------