3GPP TSG-RAN5 Meeting #108	R5-254502

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

# Source:	Qualcomm Incorporated

# Title:	Discussion on support of NR RedCap in P-Access-Network-Info header field

# Document for:	Discussion

## 1.	Introduction

With [1], 3GPP CT1 has introduced support for the NR RedCap RAT type by defining the value '3GPP-NR-REDCAP' in the access-class and access-type parameters of P-Access-Network-Info header in [2]. This enhancement enables IMS networks to identify and differentiate traffic associated with NR RedCap UEs, which is critical for functions such as charging. These parameter values are currently not specified in [3] and affects test cases in [4] and [5] when run on a UE supporting NR RedCap.

## 2.	Discussion

P-Access-Network-Info header is checked across multiple SIP messages, as specified in default messages such as A.1.1, A.2.1, A.3.1, etc., in [3]. UE including access-class or access-type parameter of P-Access-Network-Info header with value as '3GPP-NR-REDCAP' would fail all IMS test cases in [4] and [5]. Hence, all impacted default message contents of [3] needs update.

However, as these changes were introduced late in [2], some NR RedCap UEs may still not adhere to the requirement of adding “3GPP-NR-REDCAP” value in access-type and access-class parameters of P-Access-Network-Info header. It would be prudent to provide a grace period (TBD) for such UEs to comply with the requirement. During this period, UEs including access-class parameter with “3GPP-NR” or access-type parameter with value “3GPP-NR-TDD” or “3GPP-NR-FDD” instead of “3GPP-NR-REDCAP” shall not fail existing test cases.

## 3.  Proposals

Proposal 1: Update default message contents of [3] to allow inclusion of '3GPP-NR-REDCAP' in the access-class and access-type parameter of P-Access-Network-Info header.

Proposal 2: A grace period (TBD) to be granted for inclusion of '3GPP-NR-REDCAP' in the P-Access-Network-Info header.

RAN5 CR [6] has been raised at this meeting to handle both the proposals.

## 4.	References

[1]	C1-241218	Support of NR RedCap within P-Access-Network-Info header field

[2]	TS 24.229	IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3

[3]	TS 34.229-1	Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification

[4]	TS 34.229-5	Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)

[5]	TS 38.523-1	5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[6]	R5-254501	Correction to P-Access-Network-Info header for RedCap UEs