3GPP TSG-RAN5 Meeting #108	R5-254519

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5487 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall only mode test case 11.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 11.3.9 [“eCall Only mode / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is not supported / emergency call over IMS”] needs addition as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 11.3.9 – “eCall Only mode / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is not supported / emergency call over IMS”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.9 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1510 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 11.3.9	eCall Only mode / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is not supported / emergency call over IMS

11.3.9.1	Test Purpose (TP)

(1)

with { UE in eCall Only mode is in the state EMM-DEREGISTERED.eCALL-INACTIVE }

ensure that {

when { when EPS supports IMS voice over PS, emergency service and does not support eCall over IMS and UE is requested to make a manual eCall }

then { UE sends RRC Connection Request message with establishmentCause set to ‘emergency’ and establishes the normal emergency call }

}

11.3.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause 4.4.3.1.1, TS 24.229 clause 5.1.6.11.2 and TS 23.167 Annex H.6. Unless otherwise stated these are Rel-19 requirements.

[TS 23.122 clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and, for bullets i, ii, iii, iv, v, allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present);

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order) or stored in the ME (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

NOTE 1:	High quality signal is defined in the appropriate AS specification.

v)	other PLMN/access technology combinations in order of decreasing signal quality.

…

When following the above procedure the following requirements apply:

…

l)	In i to vii, if the MS is in eCall only mode, the MS shall not consider PLMN/access technology combinations which do not advertise support for eCall over IMS, unless such PLMNs are available in GERAN or UTRAN.

NOTE 6:	As an implementation option, an MS in eCall only mode that was not able to select any PLMN according to l) can perform a second iteration of i to v with no restriction.

[TS 24.229 clause 5.1.6.11.2]

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

- the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

NOTE 0:	In some countries, the type of emergency service is required, due to national regulations, so a Request-URI with ecall sub-type is needed to avoid failure in routing, e.g. when the IP-CAN indicates that eCall over IMS is not supported.

[TS 23.167 Annex H.6]

This clause details the domain priority and selection (see clause 7.3) for a UE that attempts to make an eCall over IMS session using E-UTRAN or NG-RAN radio access networks based on the availability of the CS or PS domains and the network support for IMS emergency, eCall over IMS and IMS voice over PS.

The following table (Table H.2) defines these rules based on the UE (last 2 columns) for different initial conditions (first 4 columns) when an eCall over IMS session is initiated.

Table H.2: Domain Selection Rules for eCall over IMS session attempts for E-UTRAN or NG-RAN radio access networks

|  | PS Available | VoIMS | EMS | ECL | First eCall Attempt | Second eCall Attempt |
| --- | --- | --- | --- | --- | --- | --- |
| A | Y | Y | Y | Y | PS | PS on another PS RAT if available with EMS=Y and ECL=Y or CS if available |
| B | Y | Y | Y | N | CS if available | PS (UE establishes IMS emergency session) (NOTE 2) |
| C | Y | Y or N | N | N | CS if available | PS on another PS RAT if available with EMS=Y or EMS unknown (NOTE 2) |
| D | Y | N | Y | Y | PS or CS if available | CS if first attempt in PS PS if first attempt in CS (NOTE 2) |
| E | Y | N | Y | N | CS if available | PS (UE establishes IMS emergency session) (NOTE 2) |
| F | N |  | - | - | CS if available |  |
| VoIMS = Voice over IMS over PS sessions support as indicated by IMS Voice over PS session supported indication as defined in TS 23.401 [28] and TS 23.502 [49]. For the purposes of this table, a UE in limited service state shall assume that Voice over IMS over PS sessions is supported when EMS=Y or ECL=Y. EMS = IMS Emergency Services supported as indicated by Emergency Service Support indicator as defined in TS 23.401 [28] and TS 23.501 [48] and TS 23.502 [49]. A UE in limited service state shall use the ims-EmergencySupport broadcast bit (TS 36.331 [55]; TS 38.331 [56]) instead of the Emergency Service Support indicator. ECL = eCall Over IMS support as indicated by the eCall support indicator defined in TS 23.401 [28] and TS 23.501 [48]. NOTE 1: As an implementation option, when the first attempt uses PS and fails for reasons other than related to IMS, the second attempt may use PS with a different 3GPP RAT. In this case the UE, can make a third attempt using CS. NOTE 2: Fallback to the PS domain with ECL=N shall only occur after the UE has attempted but failed to select another PLMN in the PS domain with ECL=Y or in the CS domain according to PLMN selection rules for a UE in eCall only mode in clause 4.4.3.1.1 of TS 23.122 [41]. |  |  |  |  |  |  |

11.3.9.3	Test description

11.3.9.3.1	Pre-test conditions

System Simulator:

-	Cell A is configured according to Table 6.3.2.2-1 in [18] and is the serving cell.

UE:

-	The eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

11.3.9.3.2	Test procedure sequence

Table 11.3.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | A manual eCall is initiated. (Note 1) | - | - | - | - |
| 3 | Check: Does the UE transmit a RRCConnectionRequest message with 'establishmentCause' set to ' emergency'? | --> | RRCConnectionRequest | 1 | P |
| 4-16 | Steps 3-15 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 17 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 17-24 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 take place if requested by the UE. | - | - | - | - |
| 17 | Step 16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 is performed. | - | - | - | - |
| 18 | Check: Does the UE transmit a PDN connectivity REQUEST message to request an additional PDN, with 'Request type' set to 'emergency (‘0100’B)’? | --> | PDN CONNECTIVITY REQUEST | 1 | P |
| 19-24 | Steps 10 to 15 of the generic test procedure for IMS Emergency call establishment in EUTRA: in EUTRA: Normal Service (TS 36.508 subclause 4.5A.4.3-1) take place. (Note 2) | - | - | - | - |
| 25-28 | Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33 using condition A9 for the BYE at step 1. | - | - | - | - |
| 29 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| Note 1: The request to originate a manual eCall may be performed by MMI or AT command. Note 2: The UE shall set the "Request URI and To header" to "urn:service:sos.ecall.manual" and shall not include the initial MSD in the SIP INVITE message. |  |  |  |  |  |

11.3.9.3.3	Specific message contents

Table 11.3.9.3.3-1: SystemInformationBlockType1 (preamble and all steps)

| Derivation Path: 36.508, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| eCallOverIMS-Support-r14 | Not present |  |  |
| } |  |  |  |

Table 11.3.9.3.3-2: RRCConnectionRequest (step 3, Table 11.3.9.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | emergency |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.3.9.3.3-3: Message ATTACH ACCEPT (step 14, Table 11.3.9.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach result | 001 | "EPS only" |  |
| Location area identification | Not present | SS doesn’t provide LAI |  |
| MS identity | Not Present | SS doesn’t provide TMSI |  |
| EMM cause | 00010010 | #18 "CS domain not available" | combined_EPS_IMSI |

| Condition | Explanation |
| --- | --- |
| combined_EPS_IMSI | This condition applies if the UE is configured to initiate combined EPS/IMSI attach or if explicitly specified. |

<End of modified section>