3GPP TSG-RAN5 Meeting #108	R5-254572

# Bengaluru, India, Aug.25th – 29rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5122 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to V2X SIG test case 12.1.9.1.1 - UM RLC segmentation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add the reference of SL-RLC-Config for the sake of clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reference of SL-RLC-Config is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion are caused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.9.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

12.1.9.1.1	PC5-only operation / RLC / unacknowledged mode / RLC SDU segmentation

12.1.9.1.1.1	Test Purpose (TP)

(1)

with { UE has established unicast link with peer UE and has established an unicast SL DRB with RLC-UM and sl-SN-FieldLengthUM = size6 }

ensure that {

when { A RLC SDU arrives and transmission opportunity indicated by MAC is not enough to transmit entire RLC SDU. }

then { UE segments the RLC SDU, sends a UM PDU with SI = 01 to contain the first segment, sends a UM PDU with SI = 10 to contain the last segment, and sends UM PDU(s) with SI = 11 to contain other segment(s) }

}

(2)

with { UE has established unicast link with peer UE and has established an unicast SL DRB with RLC-UM and sl-SN-FieldLengthUM = size6 }

ensure that {

when { Subsequent RLC SDUs arrives and transmission opportunity indicated by MAC is not enough to transmit each RLC SDU entirely }

then { UE increases SN by 1 for each UM SDU, and all UM PDUs corresponding to the same RLC SDU use the same SN }

}

(3)

with { UE has established unicast link with peer UE and has established an unicast SL DRB with RLC-UM and sl-SN-FieldLengthUM = size6 }

ensure that {

when { UE finishes sending all UM PDUs with SN = 63. A RLC SDU arrives and transmission opportunity indicated by MAC is not enough to transmit the entire RLC SDU }

then { UE includes SN = 0 in each UM PDU corresponding to the RLC SDU }

}

12.1.9.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, subclause 5.2.2.1.1 and 6.2.2.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.322, subclause 5.2.2.1.1]

When submitting a UMD PDU to lower layer, the transmitting UM RLC entity shall:

-	if the UMD PDU contains a segment of an RLC SDU, set the SN of the UMD PDU to TX_Next;

-	if the UMD PDU contains a segment that maps to the last byte of an RLC SDU, then increment TX_Next by one.

[TS 38.323, subclause 6.2.2.3]

UMD PDU consists of a Data field and an UMD PDU header. The UMD PDU header is byte aligned.

…

An UM RLC entity is configured by RRC to use either a 6 bit SN or a 12 bit SN. For groupcast and broadcast of NR sidelink communication, only 6 bit SN length is configured. An UMD PDU header contains the SN field only when the corresponding RLC SDU is segmented. An UMD PDU carrying the first segment of an RLC SDU does not carry the SO field in its header. The length of the SO field is 16 bits.

…

Figure 6.2.2.3-2: UMD PDU with 6 bit SN (No SO)

…

Figure 6.2.2.3-4: UMD PDU with 6 bit SN and with SO

…

12.1.9.1.1.3	Test description

12.1.9.1.1.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 is as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.1.9.1.1.3.1-1.

- The UE uses GNSS as the synchronization reference source.

Table 12.1.9.1.1.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [4] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 SL-PreconfigurationNR included in V2X data policy over PC5 is defined in Table 12.1.9.1.1.3.3-1 |  |

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.1.9.1.1.3.2	Test procedure sequence

Table 12.1.9.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger UE to close UE test loop mode E. (TRANSMIT). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). NOTE: The UE sends multiple packets continuously. | - | - | - | - |
| 2 | Check: Does the UE transmit UMD PDU containing the first segment of RLC SDU#0 (SN = 0, SI field = 01)? (NOTE 1) | --> | (RLC SDU #0, first segment) | 1 | P |
| 3 | Check: Does the UE transmit UMD PDU containing the second segment of RLC SDU#0 (SN = 0, SI field = 11) and including SO field? (NOTE 1) | --> | (RLC SDU #0, second segment) | 1 | P |
| 4 | Check: Does the UE transmit UMD PDU containing the last segment of RLC SDU#0 (SN = 0, SI field = 10) and including SO field? (NOTE 1) | --> | (RLC SDU #0, last segment) | 1 | P |
| - | EXCEPTION: Steps 5 to 7 are executed 63 times, the initial value of k = 1, it is incremented by one for each iteration. | - | - | - | - |
| 5 | Check: Does the UE transmit UMD PDU containing the first segment of RLC SDU#k (SN = k, SI field = 01)? (NOTE 1) | --> | (RLC SDU #k, first segment) | 2 | P |
| 6 | Check: Does the UE transmit UMD PDU containing the second segment of RLC SDU#k (SN = k, SI field = 11) and including SO field? (NOTE 1) | --> | (RLC SDU #k, second segment) | 2 | P |
| 7 | Check: Does the UE transmit UMD PDU containing the last segment of RLC SDU#k (SN = k, SI field = 10) and including SO field? (NOTE 1) | --> | (RLC SDU #k, last segment) | 2 | P |
| 8 | Check: Does the UE transmit UMD PDU containing the first segment of RLC SDU#64 (SN = 0, SI field = 01)? (NOTE 1) | --> | (RLC SDU #64, first segment) | 3 | P |
| 9 | Check: Does the UE transmit UMD PDU containing the second segment of RLC SDU#64 (SN = 0, SI field = 11) and including SO field? (NOTE 1) | --> | (RLC SDU #64, second segment) | 3 | P |
| 10 | Check: Does the UE transmit UMD PDU containing the last segment of RLC SDU#64 (SN = 0, SI field = 10) and including SO field? (NOTE 1) | --> | (RLC SDU #64, last segment) | 3 | P |
| 11 | Trigger UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| Note 1: The MCS is selected to ensure that a RLC SDU will be split into 3 segments The RLC SDU size is calculated as follows: RLC SDU size = 300 (SDAP SDU size given in 38.509 Table 5.3.4.3.1-1) +40 (Ipv6 header size) + 1 (SDAP header size) + 4 (PDCP header size) = 345 Bytes. TBS size = 157 Bytes based on IMCS = 7, sl-LengthSymbols = 14, sl-RB-Number = 10, sl-TimeResourcePSCCH = 2 and sl-PSSCH-DMRS-TimePatternList = {2,4} . Size of the data field in RLC PDU using header with SN but no SO = 157 (TBS size) – 4 (SL-SCH subheader size) – 3 (subPDU subheader size) - 1 (UM RLC header with SN but no SO size) = 149 Bytes. Size of the data field in RLC PDU using header with SN and SO = 157 (TBS size) – 4 (SL-SCH subheader size) – 3 (subPDU subheader size) - 3 (UM RLC header with SN and SO size) = 147 Bytes. Then each RLC SDU will be split into 3 RLC PDUs because RLC SDU size > Size of the data field in RLC PDU using header with SN but no SO + Size of the data field in RLC PDU using header with SN and SO, and < Size of the data field in RLC PDU using header with SN but no SO + 2 * Size of the data field in RLC PDU using header with SN and SO |  |  |  |  |  |

12.1.9.1.1.3.3	Specific message contents

Table 12.1.9.1.1.3.3-1: SL-PreconfigurationNR (Pre-configuration, UE under test)

| Derivation Path: TS 38.508-1 [4], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |  |
| sl-PreconfigFreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { |  | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-BWP-List-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-ConfigCommon-r16 { |  | 1 entry |  |  |
| SL-BWP-ConfigCommon-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-BWP-PoolConfigCommon-r16 |  | SL-BWP-PoolConfigCommon | Table 12.1.9.1.1.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-RLC-BearerPreConfigList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { |  | 1 entry |  |  |
| SL-RLC-BearerConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-RLC-Config-r16 |  | SL-RLC-Config specified in TS 38.508-1[4] Table 4.6.6-28 with condition UM |  |  |
| sl-MAC-LogicalChannelConfig-r16 SEQUENCE { |  |  |  |  |
| sl-HARQ-FeedbackEnabled-r16 |  | disabled |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-UE-SelectedPreConfig-r16 SEQUENCE { |  |  |  |  |
| sl-PSSCH-TxConfigList-r16 SEQUENCE (SIZE (1..maxPSSCH-TxConfig-r16)) OF SL-PSSCH-TxConfig-r16 { |  | 1 entry |  |  |
| SL-PSSCH-TxConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-ParametersBelowThres-r16 SEQUENCE { |  |  |  |  |
| sl-MinMCS-PSSCH-r16 |  | 7 | To ensure that only IMCS = 7 is used (R = 0.5136, Qm = 2) |  |
| sl-MaxMCS-PSSCH-r16 |  | 7 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.1.9.1.1.3.3-2: SL-BWP-PoolConfigCommon (Table 12.1.9.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-4 with RXPOOL and SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-PoolConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 { |  | 1 entry |  |  |
| SL-ResourcePool-r16[1] |  | SL-ResourcePool | entry 1 Table 12.1.9.1.1.3.3-3 |  |
| } |  |  |  |  |
| sl-TxPoolSelectedNormal-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { |  | 1 entry |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-ResourcePool-r16 |  | SL-ResourcePool | Table 12.1.9.1.1.3.3-3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.1.9.1.1.3.3-3: SL-ResourcePool (Table 12.1.9.1.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PSFCH-Config-r16 |  | Not present | PSFCH is not configured in resource pool |  |
| sl-NumSubchannel-r16 |  | 1 | To ensure there is only 1 subchannel in resource pool |  |
| sl-RB-Number-r16 |  | 10 |  |  |
| sl-TimeResource-r16 |  | 1111111111 |  |  |
| } |  |  |  |  |

<End of Change 1>