3GPP TSG-RAN5 Meeting #108	R5-254578

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3577 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DCI format updates for SSSG switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Physical layer parameters for DCI formats 0_1 and 1_1 need update for SSSG switching feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Physical layer parameters for DCI formats 0_1 and 1_1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCI formats would remiain undefined for SSSG switching. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1.1.2, 4.3.6.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

###### 4.3.6.1.1.2	Physical layer parameters for DCI format 0_1

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

Default physical layer parameters for DCI format 0_1 are specified in table 4.3.6.1.1.2-1.

Table 4.3.6.1.1.2-1: Physical layer parameters for DCI format 0_1

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Carrier indicator | Not present | - |  |
| UL/SUL indicator | Not present (0 bit for UEs not configured with SUL in the cell) | - |  |
|  | 1 | 1 | SUL |
| Identifier for DCI formats | Indicating an UL DCI format | “0” |  |
| Bandwidth part indicator | Not present (indicating active BWP, not present in case of only one BWP-Id as per Table 4.6.3-8) | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PUSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| Frequency hopping flag | Not present | - |  |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission / retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| Transform precoder indicator | 1 bit. A value of "1" indicates that transform precoder is disabled, i.e. CP-OFDM waveform is used. | “1” | DWS |
|  | Not present | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| 1st downlink assignment index | Dependent on test parameters |  |  |
| 2nd downlink assignment index | Not present (0 bit if one HARQ-ACK sub-codebook) | - |  |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| SRS resource indicator | Not present | - |  |
| Precoding information and number of layers | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 2 | “10” | 2TX_UL_MIMO or ULFPTx_Mode1 |
|  | 11 | “1011” | 4TX_UL_MIMO or (ULFPTx_Mode1_4Tx and 2L_UL_MIMO) |
|  | 4 | “100” | ULFPTx_Mode1_4Tx and 1L_UL_MIMO |
|  | Dependent on UE reported full power PMI and higher layer parameter codebookSubset | 4 or 5 bit as per Table 7.3.1.1.2-3 in TS 38.212 | ULFPTx_Mode2_4Tx and 1L_UL_MIMO |
|  | Dependent on UE reported full power PMI | 1 bit as per Table 7.3.1.1.2-5 in TS 38.212 | ULFPTx_Mode2 |
|  | Dependent on UE reported full power PMI | 4 bit as per Table 7.3.1.1.2-2 in TS 38.212 | ULFPTx_Mode2_4Tx and 2L_UL_MIMO |
|  | 0 or 1 | 1 bit | ULFPTx_ModeFull |
| Antenna ports | Port 0 (NOTE 2) | “000” |  |
|  |  | “00” | TRANSFORM _PRECODER _ENABLED |
| SRS request | No aperiodic SRS resource set triggered as per Table 7.3.1.1.2-24 in TS 38.212 (no SUL configured) | “00” |  |
| CSI request | Not present | - |  |
| CBG transmission information | Not present | - |  |
| PTRS-DMRS association | DMRS port 0 | “00” | PTRS_UL_CONFIG |
|  | Not present | - |  |
| beta_offset indicator | Not present (0 bit if the higher layer parameter dynamic in uci-on-PUSCH is not configured) | - |  |
| DMRS sequence initialization | = 0 (ScramblingID0 is not present as per Table 4.6.3-50) | “0” |  |
|  | Not present | - | TRANSFORM _PRECODER _ENABLED |
| UL-SCH indicator | Dependent on test parameters 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. | - |  |
| ChannelAccess-CPext-CAPC | Not present (0 bit) | - |  |
|  | 2 as per Table 7.3.1.1.1-4A in TS 38.212 | "10" | SharedSpectrum AND pc_ul_Semi_StaticChAccess_r16 |
| Open-loop power control parameter set indication | Not present | - |  |
| Priority indicator | Not present | - |  |
| Invalid symbol pattern indicator | Not present | - |  |
| Minimum applicable scheduling offset indicator | Not present | - |  |
| SCell dormancy indication | Not present | - |  |
| Sidelink assignment index | Dependent on test parameters 2 bits |  | SL_Mode1 |
| PDCCH monitoring adaptation indication | 1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2; | "0" | SSSG AND pc_sssg_Switching_1BitInd_r17 AND NOT pc_sssg_Switching_2BitInd_r17 |
|  | 2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2; | "00" | SSSG AND pc_sssg_Switching_2BitInd_r17 |
|  | Not present (0 bit) | - |  |
| NOTE 1: codebookSubset = nonCoherent, 2 layers,TPMI = 0 as specified in TS 38.212 [27] Table 7.3.1.1.2-4 NOTE 2: Bitsize depends on transform precoder being enabled/disabled (PUSCH_Config, Table 4.6.3-118) and on dmrs-Type and maxLength (DMRS-UplinkConfig, Table 4.6.3-51); 3 bits (transform precoder disabled) or 2 bits (transform precoder enabled) for DMRS type 1 and len1 |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| 4TX_UL_MIMO | UL-MIMO test cases with 4 Tx antenna ports |
| PTRS_UL_CONFIG | When PTRS Uplink is configured |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (PUSCH_Config, Table 4.6.3-118) |
| SUL | On the SUL carrier when supplementary carrier is configured |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 and 2 Tx antenna ports |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 and 2 Tx antenna ports |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power and 2 Tx antenna ports |
| SL_Mode1 | For SL mode 1 transmission test cases. |
| SharedSpectrum | Operation with shared spectrum channel access |
| 2L_UL_MIMO | UL-MIMO test cases with 2 layers |
| 1L_UL_MIMO | UL-MIMO test cases with 1 layer |
| ULFPTx_Mode1_4Tx | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 and 4 Tx antenna ports |
| ULFPTx_Mode2_4Tx | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 and 4 Tx antenna ports |
| DWS | When dynamic waveform switching is enabled for DCI format 0_1 |
| SSSG | Test cases configured with searchSpaceGroupIdList-r17 |

<End of modified section>

<Start of modified section>

###### 4.3.6.1.2.2	Physical layer parameters for DCI format 1_1

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

Default physical layer parameters for DCI format 1_1 are specified in table 4.3.6.1.2.2-1.

Table 4.3.6.1.2.2-1: Physical layer parameters for DCI format 1_1

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Carrier indicator | Not present | - |  |
| Identifier for DCI formats | Indicating a DL DCI format | “1” |  |
| Bandwidth part indicator | Not present | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| VRB-to-PRB mapping | Not present (0 bits, vrb-ToPRB-Interleaver = Not present as per Table 4.6.3-100) | - |  |
| PRB bundling size indicator | Not present (semi-static PRB_bundling) | - |  |
| Rate matching indicator | Not present | - |  |
| ZP CSI-RS trigger | Not present | - |  |
| Modulation and coding scheme (TB1) | Dependent on test parameters | - |  |
| New data indicator (TB1) | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version (TB1) | Dependent on test parameters | - |  |
| Modulation and coding scheme (TB2) | Dependent on test parameters | - |  |
| New data indicator (TB2) | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version (TB2) | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| Downlink assignment index | Dependent on test parameters | - |  |
| TPC command for scheduled PUCCH | 0 dB (accumulated TPC) as per Table 7.2.1-1 in TS 38.213 [22] | “01” |  |
| PUCCH resource indicator | PUCCH-ResourceId[7] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213 [22]) | “110” |  |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 |  |  |
|  | µ=0 (SCS=15kHz): K1=7 | “101” |  |
|  | µ=1 (SCS=30kHz): K1=5 | “011” |  |
|  | µ=3 (SCS=120kHz): K1=8 | “110” |  |
|  | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For 60KHz SCS K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 7 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,39} For 120KHz SCS K1 = 4 if mod(i,5) = 0 K1 =3 if mod(i,5) = 1 K1 =7 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,79} |  | RF_FR2_DL |
|  | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For TDD K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} For FDD K1 = 2 |  | RF_FR1_15kHz |
|  | For TDD corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 K1 = 8 if mod(i,10) = 0 K1 = 7 if mod(i,10) = 1 K1 = 6 if mod(i,10) = 2 K1 = 5 if mod(i,10) = 3 K1 = 4 if mod(i,10) = 4 K1 = 3 if mod(i,10) = 5 K1 = 2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} For FDD K1 = 2 |  | RF_FR1_30kHz |
|  | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For TDD K1 = 13 if mod(i,20) = 2 K1 = 12 if mod(i,20) = 3 K1 = 11 if mod(i,20) = 4 K1 = 10 if mod(i,20) = 5 K1 = 9 if mod(i,20) = 6 K1 = 8 if mod(i,20) = 7 K1 = 7 if mod(i,20) = 8 K1 = 6 if mod(i,20) = 9 K1 = 6 if mod(i,20) = 10 K1 = 6 if mod(i,20) = 11 K1 = 6 if mod(i,20) = 12 K1 = 6 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} For FDD K1 = 2 |  | RF_FR1_60kHz |
| Antenna port(s) | DMRS port 0 as per Table 7.3.1.2.2-1 in TS 38.212 [27] (dmrs-Type = DMRS type 1 and maxLength = len1 as per Table 4.6.3-50) | “0000” |  |
|  |  | “0011” | RF |
| Transmission configuration indication | Not present (0 bits, tci-PresentInDCI = Not present as per Table 4.6.3-28) | - |  |
| SRS request | No aperiodic SRS resource set triggered as per Table 7.3.1.1.2-24 in TS 38.212 [27] (no SUL configured) | “00” |  |
| CBG transmission information | Not present | - |  |
| CBG flushing out information | Not present | - |  |
| DMRS sequence initialization | fix length of 1 bit; '0'B for DMRS-DownlinkConfig.scramblingID0 (or physCellId if scramblingID0 is not present); see Table 4.6.3-50 | “0” |  |
| ChannelAccess-CPext | 2 as per Table 7.3.1.1.1-4A in TS 38.212 | "10" | SharedSpectrum AND pc_ul_Semi_StaticChAccess_r16 |
|  | Not present (0 bit) | - |  |
| PDCCH monitoring adaptation indication | 1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2; | "0" | SSSG AND pc_sssg_Switching_1BitInd_r17 AND NOT pc_sssg_Switching_2BitInd_r17 |
|  | 2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2; | "00" | SSSG AND pc_sssg_Switching_2BitInd_r17 |
|  | Not present (0 bit) | - |  |

| Condition | Explanation |
| --- | --- |
| RF_FR1_15kHz | RF testing in TS 38.521-x for FR1. SCS is set to 15kHz. |
| RF_FR1_30kHz | RF testing in TS 38.521-x for FR1. SCS is set to 30kHz. |
| RF_FR1_60kHz | RF testing in TS 38.521-x for FR1. SCS is set to 60kHz. |
| RF_FR2_DL | RF testing in TS 38.521-x for FR2. SCS is set to 60kHz or 120kHz |
| RF | RF testing in TS 38.521-x for FR1 and FR2. |
| SharedSpectrum | Operation with shared spectrum channel access |
| SSSG | Test cases configured with searchSpaceGroupIdList-r17 |

<End of modified section>