3GPP TSG-RAN5 Meeting #108	R5-254625

# Bengaluru, India, Aug 25th – 29th 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0641 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability of FR1 time mask for switching test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Time mask for switching across three uplink bands and time mask for switching across four uplink bands are complete, so the RF conformance test cases needed to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RF conformance test case 38.522 TC 4.1.1 is updated. - Test case conditions and selection criteria 38.522 TC 4.0 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WP will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0, 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Hard number for yellow highlighted “Cmm01” “Emm01” is left to MCC implementation |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

Table 4.0-1: Applicability of conformance test cases conditions

| C001 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A |  |
| --- | --- |
| C001a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7/3 THEN R ELSE N/A |  |
| C001b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A |  |
| <Unchanged contents are omitted> |
| C409 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.3.2-1/226 AND A.4.3.2-1/32 AND A.4.3.2-1/228 THEN R ELSE N/A |
| C410 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.3.2-1/226 AND A.4.3.2-1/41 THEN R ELSE N/A |
| Cmm01 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 and A.4.3.2A.1-2/2 and A.4.3.2-1/127a THEN R ELSE N/A |
| Cmm02 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND ((A.4.1-4A/1 or A.4.1-4A/2) AND A.4.1-4A/5) and A.4.3.2A.1-2/3 and A.4.3.2-1/127a THEN R ELSE N/A |
| Cmm03 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 and A.4.3.2A.1-2/3 and A.4.3.2-1/127a THEN R ELSE N/A |
| NOTE 1: Cxxxx applicability is defined for enhanced type 1 receiver for NR related tests (A.4.3.9-1/1). NOTE 2: Cxxxy applicability is defined for alternative additional DMRS position for co-existence with LTE CRS related tests (A.4.3.2-1/20). NOTE 3: Cxxxz applicability is defined for modified MPR behaviour related test (A.4.3.2-1/25). NOTE 4: Cxxxw applicability is defined for mpr Power Boost related test (A.4.3.2-1/38). |  |

### <Unchanged Skipped>

Table 4.0-3: Tested CA/DC or V2X Configuration Selection Criteria

| Code | Tested CA/DC Configuration Selection Criteria | Comment |
| --- | --- | --- |
| E001 | DL_2CC(A.4.3.2A.2.1-3) AND A.4.3.2B.2.0-1/1 AND NOT UL(A.4.3.2A.2.1-2) | All supported intra-band contiguous CA Configurations with 2 carriers in DL but no CA in UL |
| E002 | DL_2CC(A.4.3.2A.4.1-3) AND A.4.3.2B.2.0-1/1 AND NOT UL(A.4.3.2A.4.1-2) | All supported inter-band CA Configurations with 2 carriers in DL but no CA in UL |
| E003 | UL_2CC(A.4.3.2B.2.1-2) AND A.4.3.2B.2.0-2/1 | All supported Intra-band contiguous EN-DC configurations in FR1 (2UL CCs) |
| <Unchanged contents are omitted> |  |  |
| E049 | 3bandsULTxSwitching(Table A.4.3.2C.3-3) | All supported FR1 SUL operation with inter-band CA with ULTxSwitchingBandPair-r18 capability |
| E050 | Table A.4.3.2E.2-2 | All supported NR V2X inter-band concurrent operation |
| E051 | Table A.4.3.2E.2-2 OR Table A.4.3.2E.2-1 | All supported NR V2X inter-band concurrent operation and NR V2X intra-band concurrent operation |
| E052 | Table A.4.3.2E.2-1 | All supported NR V2X intra-band concurrent operation |
| E053 | A.4.3.2E.3-3 | All supported inter-band V2X operation in FR1 |
| Emm01 | 3BandsULTxSwitching (Table A.4.3.2A.4.2-3) | All supported FR1 inter-band CA configurations with ULTxSwitchingBandPair-r18 capability |
| Emm02 | 4BandsULTxSwitching (Table A.4.3.2A.4.3-3) | All supported FR1 inter-band CA configurations with ULTxSwitchingBandPair-r18 capability |
| <Unchanged contents are omitted> |  |  |
| NOTE 1: UL(table_index) includes all supported CA Configurations where at least one UL CA configuration was declared in column "Supported CA Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8]. NOTE 2: UL_nCC(table_index) includes all supported CA or DC Configurations where at least one n-carrier UL CA or DC configuration was declared in column "Supported CA Bandwidth Class(es) in UL" or “Supported EN-DC Bandwidth Class(es) in UL” in Table table_index in TS 38.508-2 [8]. NOTE 3: UL_NR_nCC(table_index) includes all supported DC Configurations where at least one DC configuration with n-carrier NR UL CA configuration was declared in column "Supported EN-DC Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8]. NOTE 4: DL_nCC(table_index) includes all supported n-carrier CA/DC Configurations in Table table_index in TS 38.508-2 [8]. NOTE 5: DL_NR_nCC(table_index) includes all supported DC Configurations with n-carrier NR DL CA configuration in Table table_index in TS 38.508-2 [8]. NOTE 6: ULTxSwitching(table_index) includes all supported CA/DC/SUL Configurations where at least one uplink band pair was declared in column “Supported ULTxSwitching Band Pair" in Table table_index in TS 38.508-2 [8]. 2Tx_ULTxSwitching(table_index) includes all supported CA/DC/SUL Configurations where at least one uplink band pair was declared in column “Supported 2Tx-2Tx ULTxSwitching Band Pair" in Table table_index in TS 38.508-2 [8]. 3bandsULTxSwitching(table_index) includes all supported CA/DC/SUL Configurations where at least one uplink band pair was declared in column “Supported R18 dynamic UL Tx switching band pair” in Table table_index in TS 38.508-2 [8]. 4bandsULTxSwitching(table_index) includes all supported CA/DC/SUL Configurations where at least one uplink band pair was declared in column “Supported R18 dynamic UL Tx switching band pair” in Table table_index in TS 38.508-2 [8]. |  |  |

### <Unchanged Skipped>

<End of Change 1>

<Start of Change 2>

4.1	RF conformance test cases

NOTE:	To determine applicability of a test case, supported CBW and SCS in the RF-Parameters IE (see TS 38.331 [11]) which conveys RF related capabilities for NR operation is taken into account.

### 4.1.1	FR1 standalone conformance test cases

Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-1 [1]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power | Rel-15 | C001l | UEs supporting 5GS FR1 and not supporting RedCap and not supporting eRedCap | D001a | PC1 PC2 PC3 | TC 6.2.1 is skipped if TC 6.2G.1 is executed. |
| 6.2.2 | UE maximum output power reduction | Rel-15 | C001 | UEs supporting 5GS FR1 | D001a | PC1 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5.2.4.1 is executed. Skip TC 6.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.3 or 6.5B.2.3.3.1 has been executed. TC 6.2.2 is skipped if TC 6.2G.2 is executed. |
| <Unchanged contents are omitted> |  |  |  |  |  |  |  |
| 6.3A.3.4 | Time mask for switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors (3UL CA) | Rel-17 | C051d | UEs supporting 5GS FR1 and inter-band/intra-band CA (3UL CA) and dynamic 1Tx-2Tx UL Tx switching. | E019b |  |  |
| 6.3A.3.5 | Time mask for switching between two uplink bands with two transmit antenna connectors (3UL CA) | Rel-17 | C051f | UEs supporting 5GS FR1 and inter-band/intra-band CA (3UL CA) and dynamic 2Tx-2Tx UL Tx switching. | E019c |  |  |
| 6.3A.3.6 | Time mask for switching across three uplink bands | Rel-18 | Cmm1 | UEs supporting 5GS FR1 and inter-band CA and dynamic UL Tx switching across three bands. | Emm01 |  |  |
| 6.3A.3.6_1 | Time mask for switching across three uplink bands (more than 3CC) | Rel-18 | Cmm2 | UEs supporting 5GS FR1 and inter-band + intra-band CA and dynamic UL Tx switching across three bands. | Emm01 |  |  |
| 6.3A.3.7 | Time mask for switching across four uplink bands | Rel-18 | Cmm3 | UEs supporting 5GS FR1 and inter-band CA and dynamic UL Tx switching across four bands. | Emm02 |  |  |
| 6.3A.4.1.1 | Absolute power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) and not supporting Tx Diversity | E015 |  |  |
| <Unchanged contents are omitted> |  |  |  |  |  |  |  |
| 7.9A.1 | Spurious emissions for CA (2DL CA) | Rel-15 | C005 | UEs supporting 5GS FR1 and inter-band 2DL CA with a DL-only band | E002 |  |  |
| 7.9J | Spurious emissions for ATG | Rel-18 | C001n | UEs supporting 5GS FR1 ATG | D028 |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-1. NOTE 2: The test case is optional for Rel-17 RedCap UE implementing SUL. NOTE 3: Uplink configurations CA_nA-nB, DC_A_nB, DC_B_nA, SUL_A-nD and SUL_B-nC can be noted as same UL CA configuration, where band nC and nD are the corresponding SUL bands with same UL frequency range of band (n)A and (n)B |  |  |  |  |  |  |  |

<End of Change 2>