3GPP TSG-RAN5 Meeting #108	R5-254647

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3486 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of delta TIBc and MOP for Rel16 NR CA configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE maximum output power of the following NR CA configurations is not complete. CA_ n5A-n79A, CA_n3A-n79A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding requirement about UE maximum output power of these NRCA configurations in Table 6.2A.1.1.5-1 and Table 6.2A.1.0.3-1 - Adding ΔTIB,c for the following NR CA configuration in Table 6.2A.4.0.2.3-1. CA_n3A-n79A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE maximum output power for these NRCA configuration will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.0, 6.2A.1.1, 6.2A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.2A.1	UE maximum output power for CA

6.2A.1.0	Minimum conformance requirements

6.2A.1.0.1	Void

6.2A.1.0.2	Void

6.2A.1.0.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in Table 6.2.1.3-1 apply for power class 3 and other power classes if indicated in clause 5.5A.3.

For inter-band carrier aggregation with two uplink contiguous carrier assigned to one NR band, the transmitter power requirements specified in clause 6.2A.1.0.4 apply.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the transmitter power requirements specified in clause 6.2A.1.0.5 apply.

For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.0.3-1.

Table 6.2A.1.0.3-1: UE Power Class for uplink inter-band CA (two bands)

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n12A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n14A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n14A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n14A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n14A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n20A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n30A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n30A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n40A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n66A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n66A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n71A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n71A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: Void. NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports a PC2 uplink CA configuration with single carrier for each individual band and a composite of supporting PC3 within an NR TDD or FDD band and PC2 within a second NR TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. |  |  |  |  |  |  |  |  |

-------- Skip sections not changed -------

6.2A.1.1.5	Test requirement

The maximum output power for CA, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.1.1.5-1 for Inter-band 2 UL CA configuration. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.1.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n5A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n28A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n12A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n14A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n30A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n2A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n5A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n8A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n28A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n40A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n3A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n7A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n30A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n48A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n7A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n14A-n30A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n14A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n14A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n20A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n48A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n24A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n25A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n25A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n25A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n26A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n26A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n40A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n28A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n30A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n30A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n40A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n41A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n41A-n79A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n70A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n48A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n66A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n71A-n77A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| CA_n71A-n78A |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: Void. NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1.3-1 is allowed to reduce the lower tolerance limit by 1.5dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The TT for 2UL CA Maximum Output Power is in the maximum TT among all UL CCs. For TT of each UL CC refers to Table 6.2A.1.1.5-2. |  |  |  |  |  |  |  |  |

-------- Skip sections not changed -------

6.2A.4	Configured output power for CA

-------- Skip sections not changed -------

6.2A.4.0.2.3	ΔTIB,c for Inter-band CA

Table 6.2A.4.0.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination |  | NR Band | ΔTIB,c (dB) |  |
| --- | --- | --- | --- | --- |
| CA_n1-n3 |  | n1 | 0.3 |  |
|  |  | n3 | 0.3 |  |
| CA_n1-n5 |  | n1 | 0.3 |  |
|  |  | n5 | 0.3 |  |
| CA_n1-n8 |  | n1 | 0.3 |  |
|  |  | n8 | 0.3 |  |
| CA_n1-n28 |  | n1 | 0.3 |  |
|  |  | n28 | 0.6 |  |
| CA_n1-n41 |  | n1 | 0.5 |  |
|  |  | n41 | 0.5 |  |
| CA_n1-n77 |  | n1 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n1-n78 |  | n1 | 0.3 |  |
|  |  | n78 | 0.8 |  |
| CA_n2-n5 |  | n2 | 0.3 |  |
|  |  | n5 | 0.3 |  |
| CA_n2A-n12A |  | n2 | 0.3 |  |
|  |  | n12 | 0.3 |  |
| CA_n2-n14 |  | n2 | 0.3 |  |
|  |  | n14 | 0.3 |  |
| CA_n2-n30 |  | n2 | 0.5 |  |
|  |  | n30 | 0.3 |  |
| CA_n2-n48 |  | n2 | 0.6 |  |
|  |  | n48 | 0.8 |  |
| CA_n2-n66 |  | n2 | 0.5 |  |
|  |  | n66 | 0.5 |  |
| CA_n2-n77 |  | n2 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n3-n5 |  | n3 | 0.3 |  |
|  |  | n5 | 0.3 |  |
| CA_n3-n8 |  | n3 | 0.3 |  |
|  |  | n8 | 0.3 |  |
| CA_n3-n28 |  | n3 | 0.3 |  |
|  |  | n28 | 0.3 |  |
| CA_n3-n40 |  | n3 | 0.5 |  |
|  |  | n40 | 0.5 |  |
| CA_n3-n41 |  | n3 | 0.5 |  |
|  |  | n41 | 0.34 |  |
|  |  |  | 0.85 |  |
| CA_n3-n77 |  | n3 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n3-n78 |  | n3 | 0.6 |  |
|  |  | n78 | 0.8 |  |
| CA_n3-n79 |  | n3 | 0.3 |  |
|  |  | n79 | 0.8 |  |
| CA_n5-n30 |  | n5 | 0.3 |  |
|  |  | n30 | 0.3 |  |
| CA_n5-n48 |  | n5 | 0.3 |  |
|  |  | n48 | 0.3 |  |
| CA_n5-n66 |  | n5 | 0.3 |  |
|  |  | n66 | 0.3 |  |
| CA_n5-n77 |  | n5 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n5-n78 |  | n5 | 0.6 |  |
|  |  | n78 | 0.8 |  |
| CA_n8-n78 |  | n8 | 0.6 |  |
|  |  | n78 | 0.8 |  |
| CA_n14-n30 |  | n14 |  | 0.3 |
|  |  | n30 |  | 0.3 |
| CA_n14-n66 |  | n14 |  | 0.3 |
|  |  | n66 |  | 0.3 |
| CA_n14-n77 |  | n14 |  | 0.5 |
|  |  | n77 |  | 0.8 |
| CA_n20-n78 |  | n20 | 0.6 |  |
|  |  | n78 | 0.8 |  |
|  |  | n24 | 0.3 |  |
| CA_n24-n41 |  | n41 | 0.44 |  |
|  |  |  | 0.95 |  |
| CA_n24-n48 |  | n24 | 0.6 |  |
|  |  | n48 | 0.8 |  |
| CA_n24-n77 |  | n24 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n25-n66 |  | n25 | 0.5 |  |
|  |  | n66 | 0.5 |  |
| CA_n25-n77 |  | n25 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n28-n40 |  | n28 | 0.3 |  |
|  |  | n40 | 0.3 |  |
| CA_n28-n41 |  | n28 | 0.3 |  |
|  |  | n41 | 0.3 |  |
| CA_n28-n77 |  | n28 | 0.5 |  |
|  |  | n77 | 0.8 |  |
| CA_n28-n78 |  | n28 | 0.5 |  |
|  |  | n78 | 0.8 |  |
| CA_n28-n79 |  | n28 | 0.5 |  |
|  |  | n79 | 0.8 |  |
| CA_n30-n66 |  | n30 | 0.5 |  |
|  |  | n66 | 0.8 |  |
| CA_n30-n77 |  | n30 | 0.3 |  |
|  |  | n77 | 0.8 |  |
| CA_n39-n41 |  | n39 | 02 |  |
|  |  | n41 | 02 |  |
|  |  | n39 | 0.5 |  |
|  |  | n41 | 0.5 |  |
| CA_n40-n77 |  | n77 | 0.5 |  |
| CA_n41-n771 |  | n41 | 0.3 |  |
|  |  | n77 | 0.8 |  |
| CA_n41-n79 |  | n41 | 0.3 |  |
|  |  | n79 | 0.8 |  |
| CA_n48-n66 |  | n48 | 0.8 |  |
|  |  | n66 | 0.6 |  |
| CA_n66-n70 |  | n66 | 0.5 |  |
|  |  | n70 | 0.5 |  |
| CA_n66-n71 |  | n66 | 0.3 |  |
|  |  | n71 | 0.3 |  |
| CA_n66-n77 |  | n66 | 0.6 |  |
|  |  | n77 | 0.8 |  |
| CA_n66-n78 |  | n66 | 0.6 |  |
|  |  | n78 | 0.8 |  |
| CA_n70-n71 |  | n70 | 0.3 |  |
|  |  | n71 | 0.6 |  |
| CA_n71-n77 |  | n71 | 0.5 |  |
|  |  | n77 | 0.8 |  |
| CA_n71-n78 |  | n71 | 0.5 |  |
|  |  | n78 | 0.8 |  |
| CA_n77-n79 |  | n77 | 0.5 |  |
|  |  | n79 | 0.5 |  |
|  |  | n78 | 0.5 |  |
| CA_n78-n79 |  |  | 1.56 |  |
|  |  | n79 | 0.5 |  |
|  |  |  | 1.56 |  |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 6: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |

[End of change]