3GPP TSG-RAN5 Meeting #108 	R5-254737

# Bengaluru, India, 25th Aug. 2025 - 29th Aug. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3590 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test frequencies of n105 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are errors of test frequencies for n105 in clause 4.3.1.1.1.105. - According to Annex C.2, carrier centre frequencies for asymmetric channel bandwidth combinations of FDD bands are determined by the descriptions in Annex C.2.3.2, which basically use the parameter ΔFTx-Rx. However, according to Annex C.2.3.1, ΔFTx-Rx is not applied when it is explicitly stated in TS 38.101-1 Table 7.3.2-3. In case of n105, ΔFTx-Rx shall not be applied due to this exception, but some of the current carrier centre frequencies are calculated by applying ΔFTx-Rx. - According to Annex C.3.2, FSSref for the selected GSCN shall be whithn FSSref_Min <= FSSref <= FSSref_Max. However, some of FSSref for the currente GSCN is higher than FSSref_Max. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 4.3.1.1.1.105, - Carrier centre frequencies and affected parameters for asymmetric channel bandwidth were recalculated without applying ΔFTx-Rx. - GSCN values and affected parameters were recalculated to make FSSref to be within FSSref_Min <= FSSref <= FSSref_Max. Editorial correction to text style of the clause title of 4.3.1.1.1.105. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR band n105 will not be tested with correct test frequencies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.105 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

4.3.1.1.1.105	Reference test frequencies for NR operating band n105

Table 4.3.1.1.1.105-1: Test frequencies for NR operating band n105 and SCS 15 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 614.5 | 122900 | 612.25 | 122450 | 0 | 15 | 1537 | 122930 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 632 | 126400 | 611.39 | 122278 | 102 |  | 1580 | 126490 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 649.5 | 129900 | 556.53 | 111306 | 504 |  | 1623 | 129870 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 665.5 | 133100 | 663.25 | 132650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 590.03 | 118006 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 700.5 | 140100 | 697.17 | 139434 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low | 617 | 123400 | 612.32 | 122464 | 0 | 15 | 1535 | 122890 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 632 | 126400 | 608.96 | 121792 | 102 |  | 1574 | 126010 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 647 | 129400 | 551.6 | 110320 | 504 |  | 1610 | 128890 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 668 | 133600 | 663.32 | 132664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 587.6 | 117520 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 698 | 139600 | 692.24 | 138448 | 6 |  | - | - | - | - | - | - |
| 15/15 | 79 | Downlink | Low | 619.5 | 123900 | 612.39 | 122478 | 0 | 15 | 1536 | 122910 | 0 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 632 | 126400 | 606.53 | 121306 | 102 |  | 1568 | 125530 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 644.5 | 128900 | 546.67 | 109334 | 504 |  | 1600 | 127970 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 670.5 | 134100 | 663.39 | 132678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 585.17 | 117034 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 695.5 | 139100 | 687.31 | 137462 | 6 |  | - | - | - | - | - | - |
| 20/20 | 106 | Downlink | Low | 622 | 124400 | 612.46 | 122492 | 0 | 15 | 1537 | 122930 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 632 | 126400 | 604.1 | 120820 | 102 |  | 1562 | 125050 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 642 | 128400 | 541.74 | 108348 | 504 |  | 1587 | 126990 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 673 | 134600 | 663.46 | 132692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 582.74 | 116548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 693 | 138600 | 682.38 | 136476 | 6 |  | - | - | - | - | - | - |
| 20/25 | 133 | Downlink | Low | 624.5 | 124900 | 612.53 | 122506 | 0 | 15 | 1535 | 122890 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 632 | 126400 | 601.67 | 120334 | 102 |  | 1556 | 124570 | 8 | 1 | 2 (4) | 107 |
|  |  |  | High | 639.5 | 127900 | 536.81 | 107362 | 504 |  | 1574 | 126010 | 0 | 0 | 2 (4) | 508 |
|  | 106 | Uplink | Low | 675.5 | 135100 | 665.96 | 133192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 582.74 | 116548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 679.88 | 135976 | 6 |  | - | - | - | - | - | - |
| 20/30 | 160 | Downlink | Low | 627 | 125400 | 612.6 | 122520 | 0 | 15 | 1536 | 122910 | 10 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 632 | 126400 | 599.24 | 119848 | 102 |  | 1550 | 124090 | 10 | 1 | 2 (4) | 107 |
|  |  |  | High | 637 | 127400 | 531.88 | 106376 | 504 |  | 1564 | 125090 | 10 | 1 | 2 (4) | 509 |
|  | 106 | Uplink | Low | 678 | 135600 | 668.46 | 133692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 582.74 | 116548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 677.38 | 135476 | 6 |  | - | - | - | - | - | - |
| 20/35 | 188 | Downlink | Low | 629.5 | 125900 | 612.58 | 122516 | 0 | 15 | 1537 | 122930 | 6 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 632 | 126400 | 596.72 | 119344 | 102 |  | 1544 | 123610 | 6 | 2 | 2 (4) | 108 |
|  |  |  | High | 634.5 | 126900 | 526.86 | 105372 | 504 |  | 1551 | 124110 | 6 | 2 | 2 (4) | 510 |
|  | 106 | Uplink | Low | 680.5 | 136100 | 670.96 | 134192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 582.74 | 116548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 674.88 | 134976 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.105-2: Test frequencies for NR operating band n105 and SCS 30 kHz

| UL/DL Bandwidth combination [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 24 | Downlink | Low | 617 | 123400 | 612.68 | 122536 | 0 | 15 | 1541 | 123370 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 632 | 126400 | 590.96 | 118192 | 102 |  | 1580 | 126490 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 647 | 129400 | 461.24 | 92248 | 504 |  | 1616 | 129370 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 668 | 133600 | 663.68 | 132736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 497.24 | 99448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 698 | 139600 | 691.52 | 138304 | 6 |  | - | - | - | - | - | - |
| 15/15 | 38 | Downlink | Low | 619.5 | 123900 | 612.66 | 122532 | 0 | 15 | 1542 | 123390 | 22 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 632 | 126400 | 588.44 | 117688 | 102 |  | 1571 | 125770 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 644.5 | 128900 | 456.22 | 91244 | 504 |  | 1606 | 128450 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 670.5 | 134100 | 663.66 | 132732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 494.72 | 98944 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 695.5 | 139100 | 686.5 | 137300 | 6 |  | - | - | - | - | - | - |
| 20/20 | 51 | Downlink | Low | 622 | 124400 | 612.82 | 122564 | 0 | 15 | 1543 | 123410 | 18 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 632 | 126400 | 586.1 | 117220 | 102 |  | 1565 | 125290 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 642 | 128400 | 451.38 | 90276 | 504 |  | 1593 | 127470 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 673 | 134600 | 663.82 | 132764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 492.38 | 98476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 693 | 138600 | 681.66 | 136332 | 6 |  | - | - | - | - | - | - |
| 20/25 | 65 | Downlink | Low | 624.5 | 124900 | 612.8 | 122560 | 0 | 15 | 1541 | 123370 | 6 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 632 | 126400 | 583.58 | 116716 | 102 |  | 1559 | 124810 | 10 | 0 | 0 (5) | 214 |
|  |  |  | High | 639.5 | 127900 | 446.36 | 89272 | 504 |  | 1580 | 126490 | 22 | 0 | 2 (7) | 1022 |
|  | 51 | Uplink | Low | 675.5 | 135100 | 666.32 | 133264 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 492.38 | 98476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 679.16 | 135832 | 6 |  | - | - | - | - | - | - |
| 20/30 | 78 | Downlink | Low | 627 | 125400 | 612.96 | 122592 | 0 | 15 | 1542 | 123390 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 632 | 126400 | 581.24 | 116248 | 102 |  | 1553 | 124330 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 637 | 127400 | 441.52 | 88304 | 504 |  | 1567 | 125330 | 6 | 0 | 0 (5) | 1018 |
|  | 51 | Uplink | Low | 678 | 135600 | 668.82 | 133764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 492.38 | 98476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 676.66 | 135332 | 6 |  | - | - | - | - | - | - |
| 20/35 | 92 | Downlink | Low | 629.5 | 125900 | 612.94 | 122588 | 0 | 15 | 1543 | 123410 | 10 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 632 | 126400 | 578.72 | 115744 | 102 |  | 1547 | 123850 | 14 | 0 | 0 (5) | 214 |
|  |  |  | High | 634.5 | 126900 | 436.5 | 87300 | 504 |  | 1554 | 124350 | 14 | 0 | 0 (5) | 1018 |
|  | 51 | Uplink | Low | 680.5 | 136100 | 671.32 | 134264 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 683 | 136600 | 492.38 | 98476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 685.5 | 137100 | 674.16 | 134832 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<End of change>>