3GPP TSG-RAN5 Meeting #108	R5-254769

# Bengaluru, India, 25 – 29 Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3592 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test frequencies for V2X intra-band concurrent operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test frequency is missing for V2X intra-band concurrent operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test frequency for V2X intra-band concurrent combination V2X_79B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases can’t be executed properly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

4.3.1.8	Test frequencies for NR Sidelink operating bands

4.3.1.8.1	Test frequencies for NR Sidelink operating bands in FR1

4.3.1.8.1.1	Reference test frequencies for NR Sidelink operating band n38

FFS

4.3.1.8.1.2	Reference test frequencies for NR Sidelink operating band n47

Table 4.3.1.8.1.2-1: Test frequencies for NR operating band n47 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | frequencyInfoSL [ARFCN] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 52 | Low | 5860.005 | 790667 | 5855.325 | 790355 | 0 | 790355 | 790421 | 790673 | 790913 |
|  |  | Mid | 5889.99 | 792666 | 5794.59 | 786306 | 504 | 792354 | 792420 | 792672 | 792912 |
|  |  | High | 5919.99 | 794666 | 5914.23 | 794282 | 6 | 794354 | 794420 | 794672 | 794912 |
| 20 | 106 | Low | 5865 | 791000 | 5855.46 | 790364 | 0 | 790364 | 790430 | 791006 | 791570 |
|  |  | Mid | 5889.99 | 792666 | 5789.73 | 785982 | 504 | 792030 | 792096 | 792672 | 793236 |
|  |  | High | 5914.995 | 794333 | 5904.375 | 793625 | 6 | 793697 | 793763 | 794339 | 794903 |
| 30 | 160 | Low | 5870.01 | 791334 | 5855.61 | 790374 | 0 | 790374 | 790440 | 791340 | 792228 |
|  |  | Mid | 5889.99 | 792666 | 5784.87 | 785658 | 504 | 791706 | 791772 | 792672 | 793560 |
|  |  | High | 5910 | 794000 | 5894.52 | 792968 | 6 | 793040 | 793106 | 794006 | 794894 |
| 40 | 216 | Low | 5875.005 | 791667 | 5855.565 | 790371 | 0 | 790371 | 790437 | 791673 | 792897 |
|  |  | Mid | 5889.99 | 792666 | 5779.83 | 785322 | 504 | 791370 | 791436 | 792672 | 793896 |
|  |  | High | 5904.99 | 793666 | 5884.47 | 792298 | 6 | 792370 | 792436 | 793672 | 794896 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.1.2-2: Test frequencies for NR operating band n47 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | frequencyInfoSL [ARFCN] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 24 | Low | 5860.005 | 790667 | 5855.685 | 790379 | 0 | 790379 | 790511 | 790679 | 790823 |
|  |  | Mid | 5889.99 | 792666 | 5704.23 | 780282 | 504 | 792378 | 792510 | 792678 | 792822 |
|  |  | High | 5919.99 | 794666 | 5913.51 | 794234 | 6 | 794378 | 794510 | 794678 | 794822 |
| 20 | 51 | Low | 5865 | 791000 | 5855.82 | 790388 | 0 | 790388 | 790520 | 791000 | 791480 |
|  |  | Mid | 5889.99 | 792666 | 5699.37 | 779958 | 504 | 792054 | 792186 | 792666 | 793146 |
|  |  | High | 5914.995 | 794333 | 5903.655 | 793577 | 6 | 793721 | 793853 | 794333 | 794813 |
| 30 | 78 | Low | 5870.01 | 791334 | 5855.97 | 790398 | 0 | 790398 | 790530 | 791346 | 792138 |
|  |  | Mid | 5889.9 | 792666 | 5694.51 | 779634 | 504 | 791730 | 791862 | 792678 | 793470 |
|  |  | High | 5910 | 794000 | 5893.8 | 792920 | 6 | 793064 | 793196 | 794012 | 794804 |
| 40 | 106 | Low | 5875.005 | 791667 | 5855.925 | 790395 | 0 | 790395 | 790527 | 791679 | 792807 |
|  |  | Mid | 5889.99 | 792666 | 5689.47 | 779298 | 504 | 791394 | 791526 | 792678 | 793806 |
|  |  | High | 5904.99 | 793666 | 5883.75 | 792250 | 6 | 792394 | 792526 | 793678 | 794806 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.1.2-3: Test frequencies for NR operating band n47 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | frequencyInfoSL [ARFCN] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 11 | Low | 5860.005 | 790667 | 5856.045 | 790403 | 0 | 790403 | 790667 | 790667 | 790667 |
|  |  | Mid | 5889.99 | 792666 | 5523.15 | 768210 | 504 | 792402 | 792666 | 792666 | 792666 |
|  |  | High | 5919.99 | 794666 | 5911.71 | 794114 | 6 | 794402 | 794666 | 794666 | 794666 |
| 20 | 24 | Low | 5865 | 791000 | 5856.36 | 790424 | 0 | 790424 | 790688 | 791024 | 791312 |
|  |  | Mid | 5889.99 | 792666 | 5518.47 | 767898 | 504 | 792090 | 792354 | 792690 | 792978 |
|  |  | High | 5914.995 | 794333 | 5902.035 | 793469 | 6 | 793757 | 794021 | 794357 | 794645 |
| 30 | 38 | Low | 5870.01 | 791334 | 5856.33 | 790422 | 0 | 790422 | 790686 | 791358 | 791982 |
|  |  | Mid | 5889.99 | 792666 | 5513.43 | 767562 | 504 | 791754 | 792018 | 792690 | 793314 |
|  |  | High | 5910 | 794000 | 5892 | 792800 | 6 | 793088 | 793352 | 794024 | 794648 |
| 40 | 51 | Low | 5875.005 | 791667 | 5856.645 | 790443 | 0 | 790443 | 790707 | 791667 | 792627 |
|  |  | Mid | 5889.99 | 792666 | 5508.75 | 767250 | 504 | 791442 | 791706 | 792666 | 793626 |
|  |  | High | 5904.99 | 793666 | 5882.31 | 792154 | 6 | 792442 | 792706 | 793666 | 794626 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.8.1.3	Reference test frequencies for NR Sidelink operating band n79

Table 4.3.1.8.1.3-1: Test frequencies for NR operating band n79 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | frequencyInfoSL [ARFCN] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 52 | Low | 4405.005 | 693667 | 4400.325 | 693355 | 0 | 693355 | 693421 | 693673 | 693913 |
|  |  | Mid | 4699.995 | 713333 | 4695.315 | 713021 | 504 | 713021 | 713087 | 713339 | 713579 |
|  |  | High | 4995 | 733000 | 4990.32 | 732688 | 6 | 732688 | 732754 | 733006 | 733246 |
| 20 | 106 | Low | 4410 | 694000 | 4400.46 | 693364 | 0 | 693364 | 693430 | 694006 | 694570 |
|  |  | Mid | 4699.995 | 713333 | 4690.455 | 712697 | 504 | 712697 | 712763 | 713339 | 713903 |
|  |  | High | 4989.99 | 732666 | 4980.45 | 732030 | 6 | 732030 | 732096 | 732672 | 733236 |
| 30 | 160 | Low | 4415.01 | 694334 | 4400.61 | 693374 | 0 | 693374 | 693440 | 694340 | 695228 |
|  |  | Mid | 4699.995 | 713333 | 4685.595 | 712373 | 504 | 712373 | 712439 | 713339 | 714227 |
|  |  | High | 4984.995 | 732333 | 4970.595 | 731373 | 6 | 731373 | 731439 | 732339 | 733227 |
| 40 | 216 | Low | 4420.005 | 694667 | 4400.565 | 693371 | 0 | 693371 | 693437 | 694673 | 695897 |
|  |  | Mid | 4699.995 | 713333 | 4680.555 | 712037 | 504 | 712037 | 712103 | 713339 | 714563 |
|  |  | High | 4980 | 732000 | 4960.56 | 730704 | 6 | 730704 | 730770 | 732006 | 733230 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.1.3-2: Test frequencies for NR operating band n79 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | frequencyInfoSL [ARFCN] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 24 | Low | 4405.005 | 693667 | 4400.685 | 693379 | 0 | 693379 | 693511 | 693679 | 693823 |
|  |  | Mid | 4699.995 | 713333 | 4514.235 | 700949 | 504 | 713045 | 713177 | 713345 | 713489 |
|  |  | High | 4995 | 733000 | 4988.52 | 732568 | 6 | 732712 | 732844 | 733012 | 733156 |
| 20 | 51 | Low | 4410 | 694000 | 4400.82 | 693388 | 0 | 693388 | 693520 | 694000 | 694480 |
|  |  | Mid | 4699.995 | 713333 | 4509.375 | 700625 | 504 | 712721 | 712853 | 713333 | 713813 |
|  |  | High | 4989.99 | 732666 | 4978.65 | 731910 | 6 | 732054 | 732186 | 732666 | 733146 |
| 30 | 78 | Low | 4415.01 | 694334 | 4400.97 | 693398 | 0 | 693398 | 693530 | 694346 | 695138 |
|  |  | Mid | 4699.995 | 713333 | 4504.515 | 700301 | 504 | 712397 | 712529 | 713345 | 714137 |
|  |  | High | 4984.995 | 732333 | 4968.795 | 731253 | 6 | 731397 | 731529 | 732345 | 733137 |
| 40 | 106 | Low | 4420.005 | 694667 | 4400.925 | 693395 | 0 | 693395 | 693527 | 694679 | 695807 |
|  |  | Mid | 4699.995 | 713333 | 4499.475 | 699965 | 504 | 712061 | 712193 | 713345 | 714473 |
|  |  | High | 4980 | 732000 | 4958.76 | 730584 | 6 | 730728 | 730860 | 732012 | 733140 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.1.3-3: Test frequencies for NR operating band n79 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | frequencyInfoSL [ARFCN] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 11 | Low | 4405.005 | 693667 | 4401.045 | 693403 | 0 | 693403 | 693667 | 693667 | 693667 |
|  |  | Mid | 4699.995 | 713333 | 4333.155 | 688877 | 504 | 713069 | 713333 | 713333 | 713333 |
|  |  | High | 4995 | 733000 | 4986.72 | 732448 | 6 | 732736 | 733000 | 733000 | 733000 |
| 20 | 24 | Low | 4410 | 694000 | 4401.36 | 693424 | 0 | 693424 | 693688 | 694024 | 694312 |
|  |  | Mid | 4699.995 | 713333 | 4328.475 | 688565 | 504 | 712757 | 713021 | 713357 | 713645 |
|  |  | High | 4989.99 | 732666 | 4977.03 | 731802 | 6 | 732090 | 732354 | 732690 | 732978 |
| 30 | 38 | Low | 4415.01 | 694334 | 4401.33 | 693422 | 0 | 693422 | 693686 | 694358 | 694982 |
|  |  | Mid | 4699.995 | 713333 | 4323.435 | 688229 | 504 | 712421 | 712685 | 713357 | 713981 |
|  |  | High | 4984.995 | 732333 | 4966.995 | 731133 | 6 | 731421 | 731685 | 732357 | 732981 |
| 40 | 51 | Low | 4420.005 | 694667 | 4401.645 | 693443 | 0 | 693443 | 693707 | 694667 | 695627 |
|  |  | Mid | 4699.995 | 713333 | 4318.755 | 687917 | 504 | 712109 | 712373 | 713333 | 714293 |
|  |  | High | 4980 | 732000 | 4957.32 | 730488 | 6 | 730776 | 731040 | 732000 | 732960 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.8.2	Test frequencies for concurrent NR sidelink operation

4.3.1.8.2.1	Reference test frequencies for Inter-band concurrent NR sidelink configurations

For inter-band concurrent NR sidelink configurations as listed in this clause, the following apply:

For the NR Uu band, test frequencies as specified in clause 4.3.1.1 for FR1 are used.

For the NR PC5 band, test frequencies as specified in clause 4.3.1.8.1 for FR1 are used.

Table 4.3.1.8.2.1-1: Test frequencies for Inter-band concurrent NR sidelink operating bands

| Inter-band concurrent NR sidelink configuration | Operating band for NR uplink | Operating band for NR downlink | Operating band for NR sidelink | Applicable for protocol testing |
| --- | --- | --- | --- | --- |
| V2X_n71-n47 | n71 | n71 | n47 | Yes |

4.3.1.8.2.2	Reference test frequencies for Intra-band concurrent NR sidelink configurations

4.3.1.8.2.2.1	V2X Intra-band concurrent combination n79

4.3.1.8.2.2.1.1	V2X_n79B

Table 4.3.1.8.2.2.1.1-1: V2X Intra-band concurrent operation V2X_n79B, SCS 15 kHz (NR CC)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 216 | Downlink& | Low | 4420.02 | 694668 | 4400.58 | 693372 | 0 | 30 | 8480 | 694176 | 0 | 43 | 0 (4) | 47 |
|  |  | Uplink | High | 4980 | 732000 | 4869.84 | 724656 | 504 |  | 8864 | 731040 | 0 | 4 | 0 (4) | 512 |
| 50 | 270 | Downlink& | Low | 4425 | 695000 | 4400.7 | 693380 | 0 | 30 | 8480 | 694176 | 4 | 42 | 0 (4) | 46 |
|  |  | Uplink | High | 4974.99 | 731666 | 4859.97 | 723998 | 504 |  | 8864 | 731040 | 10 | 58 | 0 (4) | 566 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-5 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The High range frequency can’t cover the two RBs at the right edge of band due to the limit of GSCN range in TS 38.101-1 table 5.4.3.3-1. Note 4: This frequency is obtained applying certain exceptions over general rules defined in annex C.3.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.2.2.1.1-2: V2X Intra-band concurrent operation V2X_n79B, SCS 15 kHz (SL CC)

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | frequencyInfoSL [ARFCN] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 52 | Low | 4445.01 | 696334 | 4440.33 | 696022 | 0 | 696022 | 696088 | 696340 | 696580 |
|  |  | High | 4995 | 733000 | 4990.32 | 732688 | 6 | 732688 | 732754 | 733006 | 733246 |
| 40 | 216 | Low | 4470 | 698000 | 4450.56 | 696704 | 0 | 696704 | 696770 | 698006 | 699230 |
|  |  | High | 4980 | 732000 | 4960.56 | 730704 | 6 | 730704 | 730770 | 732006 | 733230 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.2.2.1.1-3: V2X Intra-band concurrent operation V2X_n79B, SCS 30 kHz (NR CC)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 106 | Downlink& | Low | 4420.02 | 694668 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | Uplink | High | 4980 | 732000 | 4779.48 | 718632 | 504 |  | 8864 | 731040 | 0 | 3 | 0 (0) | 1014 |
| 100 | 273 | Downlink& | Low | 4450.02 | 696668 | 4400.88 | 693392 | 0 | 30 | 8480 | 694176 | 16 | 18 | 1 (4) | 44 |
|  |  | Uplink | High | 4950 | 730000 | 4719.42 | 714628 | 504 |  | 8832 | 727968 | 20 | 37 | 1 (4) | 1090 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-5 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The High range frequency can’t cover the two RBs at the right edge of band due to the limit of GSCN range in TS 38.101-1 table 5.4.3.3-1. Note 4: This frequency is obtained applying certain exceptions over general rules defined in annex C.3.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.2.2.1.1-4: V2X Intra-band concurrent operation V2X_n79B, SCS 30 kHz (SL CC)

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | frequencyInfoSL [ARFCN] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 52 | Low | 4445.01 | 696334 | 4440.69 | 696046 | 0 | 696046 | 696178 | 696346 | 696490 |
|  |  | High | 4995 | 733000 | 4988.52 | 732568 | 6 | 732712 | 732844 | 733012 | 733156 |
| 40 | 216 | Low | 4520.01 | 701334 | 4500.93 | 700062 | 0 | 700062 | 700194 | 701346 | 702474 |
|  |  | High | 4980 | 732000 | 4958.76 | 730584 | 6 | 730728 | 730860 | 732012 | 733140 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

Additional parameters used in this annex are defined in Table C.5.1-2.

Table C.5.1-2: Definition of additional parameters used in section C.5.2

| SCSSL_Carrier | subcarrier spacing for the carrier (sl-SCS-SpecificCarrierList-r16): FR1: 15kHz, 30kHz or 60kHz according to TS 38.101-1 [7] Table 5.3.5-1 |
| --- | --- |
| CRBsize | Common Resource Block size = 12 * SCSSL_Carrier. |
| FSL_Low | Lowest frequency of the frequency range of the V2X band as defined in clause 5.2E.1 of TS 38.101-1 [7]. |
| FSL_High | Highest frequency of the frequency range of the V2X band as defined in clause 5.2E.1 of TS 38.101-1 [7]. |
| ΔFSL_Raster | Frequency raster of the band as defined in clause 5.4E.2.3 of TS 38.101-1 [7]. |
| BWSL | Bandwidth of V2X operation frequency range of the band. |
| CBWSL | UE V2X operation channel bandwidth (MHz) of the carrier according to section 5.3E.1 of TS 38.101-1 [7]. |
| BWSL_SSB | Bandwidth of the SSB. BWSL_SSB = 11 * SCSSL_Carrier * 12 |

< End of Changes >