3GPP TSG-RAN5 Meeting #108			R5-254882

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	5.3.4.9

Source: 	Qualcomm Inc

Title: 	Discussion on distance based CHO test cases for NR-NTN

Document for:	Discussion and Endorsement

## 1.	Introduction

TS 38.533 [1] introduced distance based conditional handover test case for NR-NTN based on the requirements defined in TS 38.133 [2]. This paper discusses the feasibility of implementing this test case using the available test framework.

## 2.	Discussion

Pasting the RAN4 requirements for TC 14.2.1.5 NR SA FR1 Distance-based Conditional Handover for Satellite Access.

14.2.1.5.1	Test Purpose: This test is to verify the requirement for intra-frequency SAN distance-based conditional handover from FR1 to FR1 specified in clause 14.2.1.0.2.

14.2.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.2.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.14.2.1.5.

14.2.1.5.4	Test description

The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. During T1, the UE is configured to measure intra-frequency neighbour cell. The RRC message implying distance-based handover to cell 2 with Event CondEvent D1 shall be sent to UE, at a time earlier than TRRC (10ms) before the beginning of T2.

Starting T2, cell 2 becomes detectable and offset better than cell 1 and location condition event condEventD1-r17 is fulfilled.

Table 14.2.1.5.4.1-3: General test parameters for NR SA FR1 Distance-based Conditional Handover for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | One NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| Satellite configuration | Config 1 |  | RMC in [A.x] | For GSO satellite configuration |
|  | Config 2 |  | RMC in [A.x] | For NGSO satellite configuration |
| UE position (N,S, H) at T1 start |  |  | [(0, 0, 0)] | Set by AT command |
| UE moving speed |  | km/h | [(108, 0, 0)] | Set by AT command |
| referenceLocation1-r17.condEventD1-r17 |  | m | [(-700, 0, 0)] | Reference location for serving cell |
| referenceLocation2-r17.condEventD1-r17 |  | m | [(1300, 0, 0)] | Reference location for target cell |
| distanceThreshFromReference1-r17.condEventD1-r17 |  | 50m | [20] | D1-1 Location condition is fulfilled at T2 |
| distanceThreshFromReference2-r17.condEventD1-r17 |  | 50m | [20] | D1-2 Location condition is fulfilled at T2 |
| hysteresis-r17.condEventD1-r17 |  | 10m | 0 |  |
| timeToTrigger-r17.condEventD1-r17 |  | s | 0 |  |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 12 |  |
| T2 |  | s | 6 |  |

The RAN5 defined test procedure is as follows

14.2.1.5.4.2	Test procedure

The test scenario comprises of two satellite access NR FDD intra-frequency cells as given in table 14.2.1.5.4.1-3. Both handover delay and interruption length are tested.

Time interval T1 needs to start immediately after the UE position and moving speed are configured via AT commands.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell. Establish SRB2 and DRB. Set Cell 2 physical cell identity to the initial physical cell identity.

2.	Set the parameters according to T1 in Table 14.2.1.5.5-1. Propagation conditions are set according to Annex C clause C.2.2. The SS shall configure UE position and moving speed. Immediately after that, T1 starts.

3.	The SS shall transmit an RRCReconfiguration message with conditionalReconfiguration-r16 on Cell 1 to configure distance-based CHO condition on the UE.

4.	The UE shall transmit an RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.2.1.5.5-1. T2 starts.

6.	If the UE transmits the PRACH preambles to Cell 2 less than 872 ms from the beginning of time period T2, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	After T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for the next iteration of the test procedure loop.

9.	Once the connection is released, the SS shall switch off and then on the UE and then proceed with step 1.

10.	Repeat steps 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

The RAN4 intention for this test is during T1, UE is camped on cell1 and UE is configured to measure the neighbor satellite.  10ms before T2 starts, SS is expected to send a RRC message implying distance based handover to cell2 with event D1. At the Start of T2, UE is expected to start moving at 108 Kmph (30 m/s).

Currently there is no mechanism in RAN5 spec to configure UE with its moving speed. Even if a new method such as using AT command or a conformance only test function is defined, such methods still pose several open questions such as

- How much time will it take for the SS to signal the UE its artificially configured moving speed and receive the acknowledgement from the UE

- What would be ‘epoch’ time when UE is expected to start applying the moving speed

- In these tests, UE is expected to be pre-configured with its UE location. What happens to that pre-configured location when UE starts moving

- Can RAN4 defined T1 and T2 timelines be met by the above method to configure UE with its moving speed.

Given the fixed satellite ephemeris sent across SIB19 and UE expected to disable its satellite trajectory prediction, this makes the test case highly unrealistic and an unnecessary burden on both Test equipment and UE to meet the requirements.

Once the Rel19 satellite propagator model is introduced for these requirements as part of the Rel19 WI [3], it can accommodate UE mobility and make the test case testable.

Due to the above highlighted testability issue, we propose to make the distance based conditional handover test case for NR-NTN untestable until the Rel19 satellite propagator model is introduced in both core and test requirements.

Proposal 1: Mark the distance based conditional handover test cases for NR-NTN untestable until the Rel19 satellite propagator model is introduced in both core and test requirements.

## 3.	Conclusion

Proposal 1: Mark the distance based conditional handover test cases for NR-NTN untestable until the Rel19 satellite propagator model is introduced in the core and test requirements.

## 4.		References

[1]		TS 38.533 v18.7.0

[2]		TS 38.133 v17.18.1

[3]		Rel19 WI UID 1032086, WIC NR_IoT_NTN_req_test_enh-Perf,    Perf. Part: Enhanced requirements and conductive test methodology for NR NTN and IoT NTN