3GPP TSG-RAN5 Meeting #108	R5-254923

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5485 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Inter-RAT cell reselection test case 6.2.3.36 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the outcome of RAN5#107 discussion paper R5-253041, there is industry interest for test coverage related to protection against improper reselection to GERAN/UTRAN feature. Hence, new test case needs to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new IRAT reselection E-UTRA - UTRAN test case 6.2.3.36 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage will not exist for protection against improper reselection to GERAN/UTRAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.36 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1509 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-254923 is the final version of R5-254513. |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 6.2.3.36	Inter-RAT cell reselection from E-UTRA RRC_IDLE to UTRA_Idle / Protection against improper reselection to GERAN/UTRAN

6.2.3.36.1	Test Purpose (TP)

(1)

with { UE supporting protection against improper reselection to GERAN/UTRAN in E-UTRA RRC_IDLE state }

ensure that {
  when { UE detects an inter-RAT UTRA cell having higher reselection priority than the E-UTRA serving cell }

then { UE does not reselect to the UTRA cell }

}

(2)

with { UE in E-UTRA RRC_IDLE state }

ensure that {
  when { UE supporting protection against improper reselection to GERAN/UTRAN detects the cell re-selection criteria are not met for the cell which belongs to the lower priority inter-RAT UTRA cell }

then { UE does not reselect to the UTRA cell }

}

6.2.3.36.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1 and 5.2.4.5. Unless otherwise stated these are Rel-18 requirements.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease or RRCEarlyDataComplete message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities (i.e. cellReselectionPriority and/or cellReselectionSubPriority) provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling, deprioritisationReq received in RRCConnectionReject and altFreqPriorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values).

…

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

…

[TS 36.304, clause 5.2.4.5]

If threshServingLowQ is provided in SystemInformationBlockType3 and more than 1 second has elapsed since the UE camped on the current serving cell and if the measurements are not performed using RSS as specified in [10], cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN, NR or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3 and more than 1 second has elapsed since the UE camped on the current serving cell and if the measurements are not performed using RSS as specified in [10], cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN, NR or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

If the UE supports the protection against improper reselection to GERAN/UTRAN then:

-	When the UE evaluates serving cell conditions for the purpose of cell reselection to GERAN/UTRAN FDD/TDD by applying the reselection priorities provided in AS security protected dedicated signalling with a GERAN/UTRAN FDD/TDD frequency as lower priority compared to the current E-UTRAN frequency, or when the UE applies GERAN/UTRAN FDD/TDD priorities provided in system information, the UE shall:

-	treat GERAN and/or UTRAN FDD and/or UTRAN TDD frequencies as the lower priority compared to E-UTRAN;

-	set the value of ThreshServing, LowP to 6 dB if the value received in the system information is higher than 6 dB;

-	set the value of Q-RxLevMin to -116 dBm if the value received in SIB1 is higher than -116 dBm;

-	set the values of Pcompensation and Qoffsettemp to 0.

The UE shall not perform cell reselection to NR or UTRAN FDD cells for which the cell selection criterion S is not fulfilled.

…

6.2.3.36.3	Test description

6.2.3.36.3.1		Pre-test conditions

System Simulator:

-	Cell 1 is E-UTRAN cell.

-	Cell 7 and Cell 9 are UTRAN cells.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell 1.

UE:

-	None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.2.3.36.3.2	Test procedure sequence

Tables 6.2.3.36.3.2-1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.36.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 7 | Cell 9 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |  |  |
|  | CPICH_Ec | dBm/3.84 MHz | - | “Off” | “Off” |  |
|  | P-CCPCH | dBm/1.28 MHz | - | “Off” | “Off” |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | - | The power level values are assigned to satisfy SrxlevCell 7 > ThreshCell 7,high. |
|  | CPICH_Ec | dBm/3.84 MHz | - | -65 | “Off” |  |
|  | P-CCPCH | dBm/1.28 MHz | - | -67 | “Off” |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -100 | - | - | The power level values are assigned to satisfy SrxlevCell 9 > ThreshCell 9,low and SrxlevCell 1 < ThreshCell 1, low. |
|  | CPICH_Ec | dBm/3.84 MHz | - | “Off” | -65 |  |
|  | P-CCPCH | dBm/1.28 MHz | - | “Off” | -67 |  |
| Note: Srxlev is calculated in the UE. |  |  |  |  |  |  |

Table 6.2.3.36.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 2 | The SS changes Cell 7 power level according to the row "T1" in table 6.2.3.36.3.2-1. | - | - | - | - |
| 3 | Check: Does the UE send RRC CONNECTION REQUEST on Cell 7 within the next 60s? | - | RRC CONNECTION REQUEST | 1 | F |
| 4 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 1 | - |
| 5 | The SS changes Cell 1, Cell 7 and Cell 9 power levels according to row "T2" in table 6.2.3.36.3.2-1. (Note 1) | - | - | - | - |
| 6 | Check: Does the UE send RRC CONNECTION REQUEST on Cell 9 within the next 60s? | - | RRC CONNECTION REQUEST | 2 | F |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.2 indicate that the UE is in E-UTRA RRC_IDLE state on Cell 1? | - | - | 2 | - |
| Note 1: Power levels are such that low priority inter RAT cell reselection criteria is satisfied for UEs not supporting protection against improper reselection to GERAN/UTRAN. |  |  |  |  |  |

6.2.3.36.3.3	Specific message contents

Table 6.2.3.36.3.3-1:  SystemInformationBlockType3 cell 1 (preamble and all steps, Table 6.2.3.36.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | Not present |  |  |
| threshServingLow | 10 (20dB) |  |  |
| cellReselectionPriority | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.36.3.3-2: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.36.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 7 |  |  |
| cellReselectionPriority[n] | 5 | Higher priority than E-UTRA |  |
| threshX-High[n] | 2 (4 dB) |  |  |
| threshX-Low[n] | 1 (2 dB) |  |  |
| carrierFreq[m] | Same downlink UARFCN as used for Cell 9 |  |  |
| cellReselectionPriority[m] | 3 | Lower priority than E-UTRA |  |
| threshX-High[m] | 2 (4 dB) |  |  |
| threshX-Low[m] | 1 (2 dB) |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 7 |  |  |
| cellReselectionPriority[n] | 5 | Higher priority than E-UTRA |  |
| threshX-High[n] | 2 (4 dB) |  |  |
| threshX-Low[n] | 1 (2 dB) |  |  |
| carrierFreq[m] | Same downlink UARFCN as used for Cell 9 |  |  |
| cellReselectionPriority[m] | 3 | Lower priority than E-UTRA |  |
| threshX-High[m] | 2 (4 dB) |  |  |
| threshX-Low[m] | 1 (2 dB) |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

<End of modified section>