3GPP TSG-RAN5 Meeting #108	R5-254958

# Bengaluru, IN, 25 - 29 Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3599 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of SOR-SNPN-SI-LS IE for Rel-18 eNPN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2-NGRAN_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SOR-SNPN-SI-LS are missing in the TS 38.508-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce SOR-SNPN-SI-LS IE to 38.508-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some of the related R18 NPN test case cannot be implenmented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

### 4.7.1	Contents of 5GMM messages

<Unchanged contents omitted>

#### –	UL NAS transport

Table 4.7.1-10: UL NAS TRANSPORT

| Derivation Path: 24.501 clause 8.2.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| UL NAS TRANSPORT message identity |  | ‘0110 0111’B |  |  |
| Payload container type |  | Set according to specific message content |  |  |
| Payload container type |  | ‘0001’B | N1 SM information | INITIAL_PDU_REQUEST |
| Payload container type |  | ‘0100’B | SOR transparent container | 5GS_SOR_CMCI |
| Spare half octet |  | '0000'B |  |  |
| Payload container |  | Set according to specific message content |  |  |
| Payload container |  | Present | The SOR transparent container in the payload container IE carries steering of roaming information. | 5GS_SOR_CMCI |
| Length of payload container contents |  |  | 2 octet |  |
| Payload container contents |  |  |  |  |
| SOR transparent container |  | Present | The SOR transparent container carries steering of roaming information. |  |
| Length of SOR transparent container contents |  |  | 2 octet |  |
| SOR header |  |  | 1 octet |  |
| SOR data type |  | 1 | The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information. |  |
| ME support of SOR-CMCI indicator (MSSI) value |  | Not checked |  |  |
| ME support of SOR-CMCI indicator (MSSI) value |  | 1 | SOR-CMCI supported by the ME |  |
| ME support of SOR-SNPN-SI indicator (MSSNPNSI) value |  | Not checked |  |  |
| MS support of SOR-SNPN-SI-LS indicator (MSSNPNSILS) value |  | Not checked |  |  |
| SOR-MAC-IAUSF |  | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| Payload container |  | PDU SESSION ESTABLISHMENT REQUEST message |  | INITIAL_PDU_REQUEST |
| PDU session ID |  | Not checked |  |  |
| PDU session ID |  | Same PDU session ID as defined in the PDU SESSION ESTABLISHMENT REQUEST message in the Payload container |  | INITIAL_PDU_REQUEST |
| Old PDU session ID |  | Not checked |  |  |
| Request type |  | Not checked |  |  |
| Request type |  | ‘001’B | initial request | INITIAL_PDU_REQUEST |
| S-NSSAI |  | Not checked |  |  |
| DNN |  | Not checked | (NOTE 1) |  |
| Additional information |  | Not checked |  |  |
| MA PDU session information |  | Not checked |  |  |
| Release assistance indication |  | Not checked |  |  |
| Non-3GPP access path switching indication |  | Not checked |  |  |
| Alternative S-NSSAI |  | Not checked |  |  |
| Payload container information |  | Not checked |  |  |
| NOTE 1: Although the contents of the IE is not required to be verified for PASS/FAIL purposes, the provided information shall be taken into account e.g. for the building the content of messages and allowing for specific UE behaviour as specified in Table 4.8.4-1. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| INITIAL_PDU_REQUEST | The UL NAS TRANSPORT message is used to transport a PDU SESSION ESTABLISHMENT REQUEST message to establish a new PDU session. |
| 5GS_SOR_CMCI | Used in Rel-17 SOR-CMCI test cases (TS 38.523-1 [12]) |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

#### –	DL NAS transport

Table 4.7.1-11: DL NAS TRANSPORT

| Derivation Path: 24.501 clause 8.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| DL NAS TRANSPORT message identity |  | ‘0110 1000’B |  |  |
| Payload container type |  | Set according to specific message content |  |  |
| Payload container type |  | ‘0001’B | N1 SM information | 5GSM_MESSAGE |
| Payload container type |  | ‘0100’B | SOR transparent container | 5GS_SOR_CMCI |
| Spare half octet |  | '0000'B |  |  |
| Payload container |  | Set according to specific message content |  |  |
| Payload container |  | 5GSM message |  | 5GSM_MESSAGE |
| Payload container |  | Present | The SOR transparent container in the payload container IE carries steering of roaming information. | 5GS_SOR_CMCI |
| Length of payload container contents |  |  | 2 octet |  |
| Payload container contents |  |  |  |  |
| SOR transparent container |  | Present | The SOR transparent container carries steering of roaming information. |  |
| Length of SOR transparent container contents |  |  | 2 octet |  |
| SOR header |  |  | 1 octet |  |
| SOR data type |  | 0 | The SOR transparent container carries steering of roaming information. |  |
| List indication value |  | 1 | List of preferred PLMN/access technology combinations is provided |  |
| List type |  | 1 | The list type is a "PLMN ID and access technology list" |  |
| Acknowledgement (ACK) value |  | 1 | Acknowledgement requested |  |
| Additional parameters (AP) value |  | 0 | Additional parameters not included |  |
| Additional parameters (AP) value |  | 1 | Additional parameters included |  |
| SOR-MAC-IAUSF |  | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| CounterSOR |  | Value generated as per TS 33.501 Cl 6.14.2.3 |  |  |
| Length of PLMN ID and access technology list |  |  | 1 octet |  |
| PLMN ID and access technology list |  |  | Values determined by test implementation |  |
| SOR-CMCI indicator (SI) value |  | 0 | SOR-CMCI absent |  |
| SOR-CMCI indicator (SI) value |  | 1 | SOR-CMCI present |  |
| Store SOR-CMCI in ME indicator (SSCMI) value |  |  | Value determined by test implementation |  |
| SOR-SNPN-SI indicator (SSSI) value |  | 0 | subscribed SNPN or HPLMN indication that 'no change of the SOR-SNPN-SI stored in the UE is needed and thus no SOR-SNPN-SI is provided' |  |
| SOR-SNPN-SI-LS indicator (SSSLI) value |  | 0 | SOR-SNPN-SI-LS absent |  |
| SOR-CMCI |  | Present |  |  |
| Length of SOR-CMCI contents |  |  | Length value determined by test implementation |  |
| SOR-CMCI rule 1 |  |  | Value determined by test implementation |  |
| Length of SOR-CMCI rule contents |  |  | Length value determined by test implementation |  |
| Tsor-cm timer value |  | 00100001 | 60s |  |
| Criterion type |  |  | Value determined by test implementation |  |
| Criterion value |  |  | Value determined by test implementation |  |
| SOR-SNPN-SI |  | Not Present |  |  |
| SOR-SNPN-SI-LS |  | Not Present |  |  |
| PDU session ID |  | Not Present |  |  |
| PDU session ID |  | Set to the same value as the PDU session ID of the 5GSM message in the Payload container. |  | 5GSM_MESSAGE |
| Additional information |  | Not Present |  |  |
| 5GMM cause |  | Not Present |  |  |
| Back-off timer value |  | Not Present |  |  |
| Lower bound timer value |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| 5GSM_MESSAGE | The DL NAS TRANSPORT message is used to transport a 5GSM message |
| 5GS_SOR_CMCI | Used in Rel-17 SOR-CMCI test cases (TS 38.523-1 [12]) |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

<End of Changed Section>