Title:	Discussion on test point selection for NS_06 A-MPR in FR1, including PC1 for n14 and n85, and PC2 for n14

Document for:	Discussion and approval

Background

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for A-MPR in FR1.

Discussion

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] and for FR1. In general for A-MPR the emissions minimum requirements are SEM and/or Spurious. Specifically, for NS_06 only SEM requirements apply.

NS_06 could be configured on bands n12, n13, n14, n85 and n110, and relevant emission requirements are specified for additional spectrum emission.

| NS_06 | 6.5.2.3.4 | n12 | 3, 5, 10, 15 |  | Clause 6.2.3.32 |
| --- | --- | --- | --- | --- | --- |
|  |  | n13, n14 | 5, 10 |  |  |
|  |  | n85 | 3, 5, 10, 15 |  |  |
|  |  | n110 | 3 |  |  |

The AMPR requirement is specified as below in TS 38.101-1 v19.2.0:

For power class 3 operation on bands n12, n13, n14, n85 and n110, no A-MPR is applicable.

For power class 1 operation on band n14, no A-MPR is applicable.

For power class 1 operation on band n85 A-MPR = 8.5 dB if

( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )

or

( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz ).

For power class 2 operation on bands n13, n14 and n85, the PC2 A-MPR requirements for NS_06 are defined below in Table 6.2.3.32-1.

Table 6.2.3.32-1: A-MPR for NS_06 (Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.0 |
|  | QPSK | ≤ 1.5 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3.0 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 |
|  | 64 QAM | ≤ 4.0 |
|  | 256 QAM | ≤ 6.5 |

For power class 3 on bands n12, n13, n14, n85, n110, and power class 1 on band n14, no A-MPR is applicable, which means that the MPR requirement should be used instead of A-MPR.

For power class 1 operation on band n85, A-MPR is specified. The requirements are very similar to that of NS_35. Only operating bands and channel bandwidths differentiate between NS_35 and NS_06, which means that the proposals approved for NS_35 should be applicable to NS_06.

For power class 2 operation on band n13, n14 and n85, A-MPR is specified. The specific uplink RB allocation needs to be considered.

Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

Test Frequencies

The selection of test frequencies can be based on the agreed proposals for SEM [2], i.e. Low and High frequency.

PROPOSAL 2: Select Low range and High range test frequencies for NS_06 A-MPR in FR1.

Test Channel Bandwidth

Minimum requirements for NS_06 applies for all channel bandwidths. Select channel bandwidth to test as for SEM [2]

PROPOSAL 3: Select Lowest and Highest test channel bandwidth for NS_06 A-MPR in FR1.

  - Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. lowest and highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_06 A-MPR in FR1.

  - UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms). For NS_06 power class 1 for n14 one modulation (DFT-s-OFDM Pi / 2 BPSK w Pi / 2 BPSK DMRS) is added to the other 9 modulation types.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_06 A-MPR in FR1.

  - Uplink RB allocation when no AMPR is applicable

For power class 3 on bands n12, n13, n14, n85, n110, and power class 1 on band n14, no A-MPR is applicable, the selection of UL RB allocation can be based on the agreed proposals for SEM [2], i.e. Edge_1RB_left, Edge_1RB_right and Outer_Full.

PROPOSAL 6: The test uplink RB allocations are Edge_1RB_left, Edge_1RB_right and Outer_Full when no AMPR is applicable, i.e. power class 3 on bands n12, n13, n14, n85, n110, and power class 1 on band n14.

  - Uplink RB allocation for NS_06 PC1 on band n85

NS_06 can be signalled for a UE supporting band n85, where FUL_low = 698 MHz and where FUL_high = 716 MHz. The Maximum transmission bandwidth for NR is shown in figure 1 from [7].

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Figure 1: Maximum transmission badwidth.

The following channel bandwidths (BWChannel) are supported for NR band n85 in [7]. 3 MHz CBW is optional in Rel-18 version of the specification.

Figure 2: Channel bandwiths for n85.

The A -MPR requirement for NS_06 PC1 on band n85 is shown in clause 2. RB allocations are calculated in Table 1 and Table 2.

Table 1: A-MPR regions for ( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )

| CBW | SCS | NRB | 0.20 ∙ NRB | LCRB | RBstart, high | RBstart, low |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | 15 | 3 | 3 | 12 | 0 |
| 5 | 15 | 25 | 5 | 5 | 20 | 0 |
| 10 | 15 | 52 | 10.4 | 10 | 15 | 0 |
| 15 | 15 | 79 | 15.8 | 15 | 64 | 0 |
| 10 | 30 | 24 | 4.8 | 4 | 20 | 0 |
| 15 | 30 | 38 | 7.6 | 7 | 31 | 0 |

Select RB allocations RBstart, low and RBstart, high fulfilling Table 1 based on the agreed proposals for SEM [2].

- Edge_1RB_Left is selected for Low frequency, A-MPR apply

- Edge_1RB_Right is selected for High frequency, A-MPR apply

- Outer_Full is selected for both Low and High, no A-MPR.

Table 2: A-MPR regions for ( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz )

| CBW | SCS | NRB | 1.5 ∙ CBW + 5 MHz | NRB / 2 | RBstart | 5 ∙ ¦ RBstart + 0.5 – NRB / 2 ¦ ∙ 12 ∙ SCS |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | 15 | 9.5 | 7.5 | 15 | 7.2 |
| 5 | 15 | 25 | 12.5 | 12.5 | 25 | 11.7 |
| 10 | 15 | 52 | 20 | 26 | 48 | 20.25 |
| 15 | 15 | 79 | 27.5 | 39.5 | 70 | 27.9 |
| 10 | 30 | 24 | 20 | 12 | 23 | 20.7 |
| 15 | 30 | 38 | 27.5 | 19 | 34 | 27.9 |

Lowest possible RB allocations RBstart are shown in Table 2. Considering the higher edge of the band as worst case it would be sufficient to test requirement with Edge_1RB_Right allocation only.

PROPOSAL 7: For NS_06 power class 1 operation on band n85, the uplink RB allocation is according to existing test configuration.

  - Uplink RB allocation for NS_06 PC2 on band n13, n14 and n85

For power class 2 operation on bands n13, n14 and n85, the PC2 A-MPR requirements for NS_06 are defined below in Table 6.2.3.32-1.

Table 6.2.3.32-1: A-MPR for NS_06 (Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.0 |
|  | QPSK | ≤ 1.5 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3.0 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 |
|  | 64 QAM | ≤ 4.0 |
|  | 256 QAM | ≤ 6.5 |

Since there is no AMPR value for edge allocation and inner full allocation, MPR requirements should still apply.

There is no specific AMPR RB allocation range specified, the generic UL RB allocation for SEM could be followed, i.e. Edge_1RB_left, Edge_1RB_right and Outer_Full.

PROPOSAL 7a: For NS_06 power class 2 operation on band n13, n14 and n85, the uplink RB allocations are Edge_1RB_left, Edge_1RB_right and Outer_Full.

  - Number of Test Points

The number of test points for NS_06 AMPR is equal to the number of test points is 144 for power class 3 and power class 2 and 160 for power class 1 for n14.

For NS_06 power class 1 operation on band n85, the number of test points is 144.

  - Exceptions for A-MPR UL-MIMO test

The requirements of A-MPR for UL-MIMO is the same as the requirements of single antenna A-MPR test case 6.2.3, therefore the test point selection in proposal 1~6 can be reused without any change.

CP-OFDM is mandatory in case UL-MIMO transmission with two-layer data, only CP-OFDM test points selected in proposal 5 are valid for UL-MIMO.

PROPOSAL 8: Test points selected in proposals 1~6 can be reused by A-MPR UL-MIMO testing, and only CP-OFDM test points selected in proposal 5 are reused for UL-MIMO.

Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_06 A-MPR in FR1 test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

PROPOSAL 2: Select Low range and High range test frequencies for NS_06 A-MPR in FR1.

PROPOSAL 3: Select Lowest and Highest test channel bandwidth for NS_06 A-MPR in FR1.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_06 A-MPR in FR1.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_06 A-MPR in FR1

PROPOSAL 6: For NS_06 A-MPR base the uplink RB allocation on the agreed proposal for SEM for power classes 1.5, 2 and 3 and power class 1 for n14.

PROPOSAL 7: For NS_06 power class 1 operation on band n85, the uplink RB allocation is according to existing test configuration.

PROPOSAL 7a: For NS_06 power class 2 operation on band n13, n14 and n85, the uplink RB allocations are Edge_1RB_left, Edge_1RB_right and Outer_Full.

PROPOSAL 8: Test points selected in proposals 1~6 can be reused by A-MPR UL-MIMO testing, and only CP-OFDM test points selected in proposal 5 are reused for UL-MIMO.

References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- R5-186256, “Update test points analysis for multiple FR1 test cases”, (Huawei, HiSilicon, Keysight, CAICT)

- TS 38.101-1 v15.5.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Rel-15)

- R5-194169, “Discussion on test point selection for NS_35 A-MPR in FR1, Ericsson, Huawei, Hisilicon, RAN5#83, Reno, Athens, May.2019

- TS 38.101-1 v18.6.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Rel-18)