3GPP TSG-RAN5 Meeting #109	R5-255598

# Dallas, Texas, USA, 17 – 21 Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3530 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Receiver requirements for CA_n5A-n29A and CA_n5A-n25A-n77(2A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing Receiver requirements for CA_n5A-n29A and CA_n5A-n25A-n77(2A) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Receiver requirements for CA_n5A-n29A and CA_n5A-n25A-n77(2A) - CA_n5A-n25A-n77A into Table 7.3A.0.3.2.3-1 - CA_n5A-n25A and CA_n5A-n29A into Table 7.3A.1.5-1 - CA_n5A-n25A-n77A into Table 7.3A.2.5-1 - CA_n5A-n25A-n77A confgurations into Table 7.3A.2_1.4.1-1 - CA_n5A-n25A-n77A requirements into Table 7.3A.2_1.5-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Receiver requirements for CA_n5A-n29A and CA_n5A-n25A-n77(2A) will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.3.2.3, 7.3A.1.5, 7.3A.2.5, 7.3A.2_1.4.1, 7.3A.2_1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 38.905 CR 1087 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Test point analysis in R5-255593 (CR 1087) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

7.3A.0.3.2.3	ΔRIB,c for three bands

Table 7.3A.0.3.2.3-1: ΔRIB,c due to CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n3-n28 | - | - | 0.2 |
| CA_n1-n3-n77 | 0.2 | 0.2 | 0.5 |
| CA_n1-n3-n78 | 0.2 | 0.2 | 0.5 |
| CA_n1-n5-n78 | 0.2 | 0.2 | 0.5 |
| CA_n1-n8-n78 | - | 0.2 | 0.5 |
| CA_n1-n28-n77 | 0.2 | 0.2 | 0.5 |
| CA_n1-n28-n78 | - | 0.2 | 0.5 |
| CA_n1-n41-n77 | 0.2 | - | 0.5 |
| CA_n1-n78-n79 | - | 0.5 | - |
| CA_n2-n5-n48 | 0.2 | - | 0.5 |
| CA_n2-n5-n66 | 0.3 | - | 0.3 |
| CA_n2-n5-n77 | 0.2 | 0.5 | 0.5 |
| CA_n2-n14-n66 | 0.3 | - | 0.3 |
| CA_n2-n14-n77 | 0.2 | 0.2 | 0.5 |
| CA_n2-n48-n66 | 0.3 | 0.5 | 0.3 |
| CA_n2-n48-n77 | 0.2 | 0.5 | 0.5 |
| CA_n2-n66-n77 | 0.2 | 0.2 | 0.5 |
| CA_n3-n5-n78 | 0.2 | 0.2 | 0.5 |
| CA_n3-n8-n78 | 0.2 | 0.2 | 0.5 |
| CA_n3-n28-n41 | - | - | 01 / 0.52 |
| CA_n3-n28-n77 | 0.2 | 0.2 | 0.5 |
| CA_n3-n28-n78 | - | 0.2 | 0.5 |
| CA_n3-n41-n77 | 0.2 | 01 / 0.52 | 0.5 |
| CA_n5-n25-n77 | 0.2 | 0.2 | 0.5 |
| CA_n5-n48-n66 | - | 0.5 | 0.2 |
| CA_n5-n48-n77 | 0.2 | 0.5 | 0.5 |
| CA_n5-n66-n77 | 0.2 | 0.2 | 0.5 |
| CA_n14-n66-n77 | 0.2 | 0.5 | 0.5 |
| CA_n25-n66-n77 | 0.3 | 0.3 | 0.5 |
| CA_n25-n66-n78 | 0.3 | 0.3 | 0.5 |
| CA_n28-n40-n71 | 0.7 | - | 0.7 |
| CA_n28-n40-n77 | - | - | 0.5 |
| CA_n28-n41-n77 | 0.2 | - | 0.5 |
| CA_n28-n41-n79 | 0.2 | 0.5 | 0.5 |
| CA_n28-n71-n77 | 0.7 | 0.7 | 0.5 |
| CA_n28-n78-n79 | 0.2 | 0.5 | - |
| CA_n40-n71-n77 | - | 0.2 | 0.5 |
| CA_n41-n66-n71 | 0.51 / 12 | 0.5 | - |
| CA_n48-n66-n70 | 0.5 | 0.2 | 0.2 |
| CA_n48-n66-n71 | 0.2 | 0.2 | 0.2 |
| CA_n48-n66-n77 | 0.5 | 0.2 | 0.5 |
| CA_n48-n70-n71 | 0.2 | 0.2 | 0.2 |
| CA_n66-n71-n77 | 0.2 | 0.2 | 0.5 |
| CA_n66-n71-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Void. NOTE 4: Void. NOTE 5: FFS. NOTE 6: FFS. NOTE 7: Void. NOTE 8: Void. NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

-------- Skipped unchanged clauses --------

7.3A.1.5	Test requirement

For 2DL carrier aggregation, test parameters are specified in table 7.3A.1.4.1-1, 7.3A.1.4.1-2 and 7.3A.1.4.1-3. For the CA configurations listed in table 7.3A.1.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference power level specified in table 7.3.2.5-1a and table 7.3.2.5-1b for non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2a and 7.3.2.5-2b for non-SDL carrier for 4 Rx antenna port, in table 7.3.2.5-2e and table 7.3.2.5-2f for non-SDL carrier for 8 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous 2 DL CA, the test requirement for shall be increased by ΔRIBNC given in Table 7.3A.0.2.2-1 for the SCC. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.1.5-1: Reference sensitivity requirement for 2DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
|  | CA_n5B | - |
|  | CA_n41C | - |
|  | CA_n48B | - |
|  | CA_n48C | - |
| Intra-band contiguous 2DL CA | CA_n66B | - |
|  | CA_n77C | - |
|  | CA_n78B | - |
|  | CA_n78C | - |
|  | CA_n79C | - |
|  | CA_n48(2A) | - |
|  | CA_n2(2A) | - |
|  | CA_n66(2A) | - |
| Intra-band non-contiguous 2DL CA | CA_n77(2A) | - |
|  | CA_n78(2A) | - |
|  | CA_n71(2A) | - |
|  | CA_n1A-n3A | - |
|  | CA_n1A-n5A | - |
|  | CA_n1A-n8A | - |
|  | CA_n1A-n26A | - |
|  | CA_n1A-n28A | - |
|  | CA_n1A-n41A | - |
|  | CA_n1A-n77A | - |
|  | CA_n1A-n78A | - |
|  | CA_n1A-n79A | - |
|  | CA_n2A-n5A | - |
|  | CA_n2A-n12A | - |
|  | CA_n2A-n14A | - |
|  | CA_n2A-n30A | - |
|  | CA_n2A-n48A | - |
|  | CA_n2A-n66A | - |
|  | CA_n2A-n77A | - |
|  | CA_n3A-n5A | - |
|  | CA_n3A-n8A | - |
|  | CA_n3A-n28A | - |
|  | CA_n3A-n41A | - |
|  | CA-n3A-n77A | - |
| Inter-band 2DL CA | CA_n3A-n78A | - |
|  | CA_n3A-n79A | - |
|  | CA_n5A-n25A | - |
|  | CA_n5A-n29A | - |
|  | CA_n5A-n30A | - |
|  | CA_n5A-n48A | - |
|  | CA_n5A-n66A | - |
|  | CA_n5A-n77A | - |
|  | CA_n5A-n78A | - |
|  | CA_n5A-n79A | - |
|  | CA_n8A-n77A | - |
|  | CA_n8A-n78A | - |
|  | CA_n14A-n30A | - |
|  | CA_n14A-n66A | - |
|  | CA_n14A-n77A | - |
|  | CA_n20A-n78A | - |
|  | CA_n24A-n41A | - |
|  | CA_n24A-n48A | - |
|  | CA_n24A-n77A | - |
|  | CA_n25A-n77A | - |
|  | CA_n25A-n78A | - |
|  | CA_n26A-n78A | - |
|  | CA_n28A-n40A | - |
|  | CA_n28A-n41A | - |
|  | CA_n28A-n71A | - |
|  | CA_n28A-n77A | - |
|  | CA_n28A-n78A | - |
|  | CA_n28A-n79A | - |
|  | CA_n30A-n66A | - |
|  | CA_n30A-n77A | - |
|  | CA_n39A-n41A | - |
|  | CA_n40A-n71A | - |
|  | CA_n40A-n77A | - |
|  | CA_n41A-n66A (Note 1) | - |
|  | CA_n41A-n71A | - |
|  | CA_n41A-n77A | - |
|  | CA_n41A-n79A | - |
|  | CA_n48A-n77A | - |
|  | CA_n66A-n70A | - |
|  | CA_n66A-n71A | - |
|  | CA_n66A-n77A | - |
|  | CA_n66A-n78A | - |
|  | CA_n70A-n71A | - |
|  | CA_n71A-n77A | - |
|  | CA_n71A-n78A | - |
|  | CA_n78A-n79A | - |
| SDL configuration | CA_n29A-n66A | - |
|  | CA_n29A-n70A | - |
|  | CA_n29A-n71A (NOTE 1) | - |
| Note 1: Configuration has sensitivity exceptions according to notes in table 7.3A.1.4.1-2. Other Rx cases shall be tested with reference sensitivity levels defined in Clause 7.3A.1_1 Note 2: CA_n41A-66A testpoint 1, reference sensitivity requirement is tested in Clause 7.3A.1_1 due to cross band isolation exception specified in Table 7.3A.0.6-3. Other Rx cases using configuration for testpoint 1 shall be tested with reference sensitivity levels defined in Clause 7.3A.1_1.5. |  |  |

-------- Skipped unchanged clauses --------

7.3A.2.5	Test requirement

For 3DL carrier aggregation, test parameters are specified in table 7.3A.2.4.1-1. For the CA configurations listed in table 7.3A.2.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference power level specified in table 7.3.2.5-1a and table 7.3.2.5-1bfor each non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2a and 7.3.2.5-2b for each non-SDL carrier for 4 Rx antenna port, in table 7.3.2.5-2e and table 7.3.2.5-2f for non-SDL carrier for 8 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous CA with one uplink carrier and two or more downlink sub-blocks, the test requirement for SCC(s) shall be increased by ΔRIBNC given in Table 7.3A.0.2.2-1. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.5-1: Reference sensitivity requirement for 3DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
| Intra-band non-contiguous 3DL CA | CA_n48(3A) | - |
|  | CA_n66(3A) | - |
| Inter-band 3DL CA | CA_n1A-n78C |  |
|  | CA_n1A-n78(2A) | - |
|  | CA_n1A-n8A-n78A |  |
|  | CA_n1A-n28A-n78A | - |
|  | CA_n1A-n78A-n79A | - |
|  | CA_n1A-n3A-n28A | - |
|  | CA_n1A-n3A-n77A |  |
|  | CA_n1A-n3A-n78A | - |
|  | CA_n1A-n5A-n78A |  |
|  | CA_n1A-n28A-n77A |  |
|  | CA_n1A-n28A-n78A | - |
|  | CA_n1A-n28A-n79A | - |
|  | CA_n1A-n41A-n77A |  |
|  | CA_n2A-n5A-n48A |  |
|  | CA_n2A-n5A-n66A |  |
|  | CA_n2A-n5A-n77A |  |
|  | CA_n2A-n14A-n30A | - |
|  | CA_n2A-n14A-n66A | - |
|  | CA_n2A-n14A-n77A | - |
|  | CA_n2A-n30A-n77A | - |
|  | CA_n2A-n48A-n66A |  |
|  | CA_n2A-n48A-n77A |  |
|  | CA_n2A-n66A-n77A |  |
|  | CA_n2(2A)-n5A | - |
|  | CA_n2(2A)-n14A | - |
|  | CA_n2(2A)-n30A | - |
|  | CA_n2A-n48(2A) |  |
|  | CA_n2A-n48B |  |
|  | CA_n2(2A)-n66A | - |
|  | CA_n2A-n66(2A) | - |
|  | CA_n2A-n77C |  |
|  | CA_n2A-n77(2A) | - |
|  | CA_n3A-n5A-n78A |  |
|  | CA_n3A-n8A-n78A |  |
|  | CA_n3A-n28A-n40A |  |
|  | CA_n3A-n28A-n41A |  |
|  | CA_n3A-n28A-n77A |  |
|  | CA_n3A-n28A-n78A | - |
|  | CA_n3A-n40A-n77A |  |
|  | CA_n3A-n41A-n77A |  |
|  | CA_n3A-n77(2A) |  |
|  | CA_n5A-n25A-n77A | - |
|  | CA_n5A-n30A-n77A |  |
|  | CA_n5A-n48A-n66A |  |
|  | CA_n5A-n48A-n77A |  |
|  | CA_n5A-n66A-n77A |  |
|  | CA_n5A-n48(2A) |  |
|  | CA_n5A-n48B |  |
|  | CA_n5A-n66(2A) | - |
|  | CA_n5A-n77C |  |
|  | CA_n5A-n77(2A) | - |
|  | CA_n5B-n77A |  |
|  | CA_n8A-n77(2A) |  |
|  | CA_n14A-n30A-n66A |  |
|  | CA_n14A-n66(2A) | - |
|  | CA_n14A-n66A-n77A | - |
|  | CA_n14A-n77(2A) | - |
|  | CA_n25A-n66A-n77A | - |
|  | CA_n25A-n66A-n78A | - |
|  | CA_n26A-n66-n70A |  |
|  | CA_n26A-n66(2A) |  |
|  | CA_n28A-n40A-n71A | - |
|  | CA_n28A-n40A-n77A |  |
|  | CA_n28A-n41A-n77A |  |
|  | CA_n28A-n41C |  |
|  | CA_n28A-n41A-n79A | - |
|  | CA_n28A-n71A-n77A | - |
|  | CA_n28A-n77(2A) |  |
|  | CA_n28A-n78A-n79A |  |
|  | CA_n3A-n78A-n79A | - |
|  | CA_n30A-n66(2A) | - |
|  | CA_n30A-n77(2A) | - |
|  | CA_n40A-n71A-n77A | - |
|  | CA_n41A-n66A-n71A |  |
|  | CA_n41A-n79C |  |
|  | CA_n41C-n79A |  |
|  | CA_n48A-n66(2A) |  |
|  | CA_n48A-n71(2A) |  |
|  | CA_n48A-n77C |  |
|  | CA_n48B-n66A |  |
|  | CA_n48B-n70A |  |
|  | CA_n48B-n71A |  |
|  | CA_n48B-n77A |  |
|  | CA_n48(2A)-n66A |  |
|  | CA_n48(2A)-n70A |  |
|  | CA_n48(2A)-n71A |  |
|  | CA_n48(2A)-n77A |  |
|  | CA_n48A-n66A-n70A |  |
|  | CA_n48A-n66A-n71A |  |
|  | CA_n48A-n66A-n77A |  |
|  | CA_n48A-n70A-n71A |  |
|  | CA_n66A-n70A-n71A | - |
|  | CA_n66A-n71A-n77A | - |
|  | CA_n66A-n71A-n78A | - |
|  | CA_n66A-n71(2A) |  |
|  | CA_n66(2A)-n70A | - |
|  | CA_n66(2A)-n71A | - |
|  | CA_n66B-n70A | - |
|  | CA_n66B-n71A | - |
|  | CA_n66A-n77C |  |
|  | CA_n66A-n77(2A) | - |
|  | CA_n66(2A)-n77A | - |
|  | CA_n70A-n71A(2A) |  |
|  | CA_n3A-n28A-n77(2A) |  |
| SDL configuration | CA_n29A-n66A-n70A | - |
|  | CA_n29A-n66B | - |
|  | CA_n29A-n66(2A) | - |

-------- Skipped unchanged clauses --------

### 7.3A.2_1	Reference sensitivity power level for 3DL CA exceptions

Editor’s Note: Test configuration table and test requirements in clauses 7.3A.2_1.4.1 and 7.3A.2_1.5 alignment with TS 38.101-1 (V18.9.0) is FFS.

7.3A.2_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.2_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support inter-band NR 3DL CA with UL CA support.

7.3A.2_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.2_1.4	Test description

7.3A.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2_1.4.1-1: Test Configuration Table for inter-band 3DL CA exceptions

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “NRB_PCC”and “NRB_SCC” columns |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL allocation (Note 2) |  |  |
|  | CA configuration |  |  |  |  |  | PCC | SCC1 | SCC2 | CC Mod | PCC & SCC RB allocation |  |  | CC Mod | PCC, SCC1 or SCC2 RB allocation |  |
|  | PCC |  | SCC1 |  | SCC2 |  |  |  |  |  | PCC | SCC1 | SCC2 |  |  |  |
|  | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DL_n1A-n3A-n77A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n77 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1775/ DL 1870 | n77 | 3915 | n1 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n78 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n1 | UL 1950/ DL 2140 | n3 | UL 1770/ DL 1865 | n78 | 3360 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n78 | 3780 | n3 | UL 1770/ DL 1865 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n5A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/DL 874 | n78 | 3780 | n1 | DL 2122 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL1975/DL2165 | n78 | 3405 | n5 | DL 885 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n1A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL1950/DL2140 | n5 | UL830/DL875 | n78 | DL 3610 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n8A-n78A_UL_n1A-n8A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1945/ DL 2135 | n8 | UL 900/ DL 945 | n78 | DL 3745 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n8A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1940/ DL 2130 | n78 | 3380 | n8 | DL 940 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n77 | 3630 | n1 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n41A-n77A_UL_n1A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1970/ DL 2160 | n41 | 2650 | n77 | 3330 | 5MHz | 10MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2640 | n77 | 3710 | n1 | DL 2140 | 10MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n1A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n28 | UL 733/ DL 788 | n78 | 3416 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1970/ DL 2160 | n78 | 3352 | n28 | DL 794 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n28A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n78 | 3630 | n1 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n79A_UL_n1A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n28 | UL 730/ DL 785 | n79 | 4630 | 5MHz | 5MHz | 40MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n79A_UL_n1A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1930/ DL 2120 | n79 | 4648 | n28 | DL 788 | 5MHz | 40MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 745.5/ DL 800.5 | n79 | 4420 | n1 | DL 2167.5 | 5MHz | 40MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n78A-n79A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n78 | 3410 | n79 | 4870 | 5MHz | 10MHz | 40MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n78A-n79A_UL_n1A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n79 | 4670 | n78 | 3490 | 5MHz | 40MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n78A-n79A_UL_n78A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3400 | n79 | 4660 | n1 | DL 2140 | 10MHz | 40MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n66A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1990/ DL 1980 | n5 | UL 830/ DL 875 | n66 | DL 2140 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n5A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 839/ DL 884 | n48 | 3640 | n2 | DL 1962 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n5 | UL 844/ DL 1889 | n48 | DL 3593 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1907.5/ DL 1987.5 | n77 | 3305 | n5 | DL 887.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 846.5/ DL 891.5 | n77 | 3680 | n2 | DL 1987 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n5 | UL 830/ DL 875 | n77 | DL 3540 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n14A-n66A_UL_n2A-n14A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1874/ DL 1954 | n14 | UL 793/ DL 763 | n66 | DL 2162 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n14A-n66A_UL_n14A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n14 | UL 793/ DL 763 | n66 | UL 1770/ DL 2170 | n2 | DL 1954 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n14A-n77A_UL_n2A-n14A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1880/ DL 1960 | n14 | UL 793/ DL 763 | n77 | DL 3466 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n14A-n77A_UL_n14A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n14 | UL 793/ DL 763 | n77 | 3540 | n2 | DL 1954 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n30A-n77A_UL_n2A-n30A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1870/ DL 1950 | n30 | UL 2310/ DL 2355 | n77 | DL 4180 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n30A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n77 | 3361 | n30 | DL 2354 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n2 | UL 1860/ DL 1940 | n77 | 3967 | n30 | DL 2354 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n30A-n77A_UL_n30A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n30 | UL 2312/ DL 2357 | n77 | 3305 | n2 | DL 1986 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n66 | UL 1770/ DL 2190 | n48 | DL 3625 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n48 | 3560 | n66 | DL 2155 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n48A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1735/ DL 2135 | n48 | 3695 | n2 | DL 1960 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3620 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3900 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n77 | 3970 | n66 | DL 2115 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n77 | 3500 | n66 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n2 | UL 1885/ DL 1965 | n77 | 3915 | n66 | DL 2175 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1760/ DL 2160 | n77 | 3720 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n66 | UL 1760/ DL 2160 | n77 | 3350 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n66 | UL 1760/ DL 2160 | n77 | 3620 | n2 | DL 1960 | 5Mhz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n5A-n78A_UL_n3A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1730/ DL1825 | n5 | UL 839/ DL 884 | n78 | DL 3408 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1730/ DL1825 | n5 | UL 839/ DL 884 | n78 | DL 3512 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n5A-n78A_UL_n5A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 839/ DL 884 | n78 | 3540 | n3 | DL 1862 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n8A-n78A_UL_n3A-n8A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1730/ DL 1825 | n8 | UL 910/ DL 955 | n78 | DL 3550 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1730/ DL 1825 | n8 | UL 910/ DL 955 | n78 | DL 3370 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n8A-n78A_UL_n8A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | UL 910/ DL 955 | n78 | 3640 | n3 | DL 1820 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2518 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 25 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2687 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n40A-n77A_UL_n3A-n40A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1730/ DL 1825 | n40 | 2320 | n77 | 4050 | 5MHz | FFS | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n40A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n77 | 4030 | n40 | 2310 | 5MHz | 10MHz | FFS | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n40A-n77A_UL_n40A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2310 | n77 | 4130 | n3 | 1820 | FFS | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n41 | 2510 | n28 | UL 735/ DL 790 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 710.5/ DL 765.5 | n41 | 2543 | n3 | UL 1737.5/ DL 1832.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n28 | UL 733/ DL 788 | n77 | 4173 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n77 | 3320 | n3 | UL 1755/ DL 1850 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1712.5/ DL 1807.5 | n77 | 4195 | n28 | UL 715/ DL 770 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n78A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1755/ DL 1850 | n28 | UL 735/ DL 790 | n78 | 3320 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1750/ DL 1845 | n28 | UL 743/ DL 798 | n78 | 3764 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n3A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n41 | 2580 | n77 | 3440 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2620 | n77 | 3400 | n3 | DL 1840 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n77 | 3900 | n41 | 2640 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n25A-n77A_UL_n5A-n25A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 830/ DL 875 | n25 | UL 1880/ DL 1960 | n77 | DL 3540 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n25A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 846.5/ DL 891.5 | n77 | 3680 | n25 | DL 1987 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | CP-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n25A-n77A_UL_n25A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n25 | UL 1907/ DL 1987 | n77 | 3305 | n5 | DL 889 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | CP-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n30A-n77A_UL_n5A-n30A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 840/ DL 885 | n30 | UL 2310/ DL 2355 | n77 | DL 3780 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n30A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 835/ DL 880 | n77 | 4025 | n30 | DL 2355 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | CP-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n30A-n77A_UL_n30A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n30 | UL 2310/ DL 2355 | n77 | 3740 | n30 | DL 880 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | CP-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n48A-n66A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/ DL 874 | n66 | UL 1760/ DL 2160 | n48 | DL 3622 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n5 | UL 845/ DL 890 | n66 | UL 1775/ DL 2175 | n77 | DL 3465 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n5 | UL 826.5/ DL 871.5 | n66 | UL 1712.5/ DL 2112.5 | n77 | DL 4192 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n5 | UL 835/ DL 880 | n66 | UL 1735/ DL 2135 | n77 | DL 3535 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 826.5/ DL 871.5 | n77 | 3795 | n66 | DL 2142 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n14A-n66A-n77A_UL_n14A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n14 | UL 793 / DL 763 | n66 | UL 1755 / DL 2155 | n77 | DL 3341 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n14A-n66A-n77A_UL_n14A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n14 | UL 793 / DL 763 | n77 | 3741 | n66 | DL 2155 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n14A-n66A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1712.5 / DL 2112.5 | n77 | 4188 | n14 | DL 763 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n40A-n71A_UL_n28A-n40A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 743/ DL 798 | n40 | 2350 | n71 | DL 622 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n40A-n77A_UL_n28A-n40A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 708/ DL 763 | n40 | 2310 | n77 | 3726 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n40A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 708/ DL 763 | n77 | 3806 | n40 | 2390 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n40A-n77A_UL_n40A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2302.5 | n77 | 3795 | n28 | 800.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 743/ DL 798 | n41 | 2580 | n77 | 3323 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 738/ DL 793 | n77 | 3380 | n41 | 2642 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2642 | n77 | 3440 | n28 | DL 798 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 25 | n41 | 2567.5 | n77 | 3460 | n28 | DL 782.5 | 10MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n41A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2600 | n79 | 4600 | n28 | UL 725/ DL 780 | 10MHz | 40 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 720/ DL 780 | n41 | 2600 | n79 | 4600 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n79 | 4850 | n41 | 2645 | 5MHz | 40 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n71A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 713/ DL 768 | n77 | 3480 | n71 | DL 628 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n71A-n77A_UL_n71A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 688/ DL 642 | n77 | 3530 | n28 | DL 778 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n78A-n79A_UL_n28A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n78 | 3700 | n79 | 4440 | 5MHz | 10MHz | 40MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n78A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n79 | 4440 | n78 | 3700 | 5MHz | 40MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n78A-n79A_UL_n78A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3620 | n79 | 4420 | n28 | DL 800 | 10MHz | 40MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n40A-n71A-n77A_UL_n40A-n71A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2330 | n71 | UL 688/ DL 642 | n77 | 3706 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n40A-n71A-n77A_UL_n40A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2320 | n77 | 3580 | n71 | DL 630 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n40A-n71A-n77A_UL_n71A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 688/ DL 642 | n77 | UL 3730 | n40 | 2354 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n66A-n71A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n71 | UL 668/ DL 622 | n77 | 4108 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n77 | 4080 | n71 | DL 640 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n71A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 690/ DL 644 | n77 | 3530 | n66 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n78A_UL_n66A-n71A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n71 | UL 668/ DL 622 | n78 | 3724 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n78A_UL_n71A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 693/ DL 647 | n78 | 3546 | n66 | DL 2160 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference of 3DL/2UL according to Table 7.3A.0.5-2 for PC3. Note 3: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. Note 4: This step can be skipped for PC2 since no corresponding exceptions are defined for PC2 cases in Table 7.3A.0.5-2a. Note 5: This test ID is for UL PC2 only. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skipped unchanged clauses --------

7.3A.2_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 ans A.2.3 with parameters specified in Table 7.3A.2_1.5-1 for PC3 and Table 7.3A.2_1.5-1a for PC2.

Table 7.3A.2_1.5-1: Reference sensitivity requirement for PC3 inter-band 3DL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 7, Note 8, Note 9) |
| --- | --- | --- | --- | --- |
| DL_n1A-n3A-n77A_UL_n1A-n3A | 1 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +28.4 |
| DL_n1A-n3A-n77A_UL_n3A-n77A | 1 | n1 | 5 | REF_victim +31.0 |
|  |  | n3 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n1A-n3A-n78A_UL_n1A-n3A | 1 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +28.4 |
|  | 2 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +11.2 |
| DL_n1A-n3A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_victim +27.9 |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n5A-n78A_UL_n5A-n78A | 1 | n1 | 5 | REF_victim +18.1 |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n5A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_victim +3.1 |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n5A-n78A_UL_n1A-n5A | 1 | n1 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +15.7 |
| DL_n1A-n8A-n78A_UL_n1A-n8A | 1 | n1 | 5 | REF_aggressor |
|  |  | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +14.9 |
| DL_n1A-n8A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REF_aggressor |
|  |  | n8 | 5 | REF_victim +3.3 |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n28A-n78A_UL_n1A-n28A | 1 | n1 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +15.7 |
| DL_n1A-n28A-n78A_UL_n1A-n78A | 1 | n1 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_victim +4.2 |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n28A-n78A_UL_n28A-n78A | 1 | n1 | 5 | REF_victim +15.7 |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
| DL_n1A-n28A-n77A_UL_n28A-n77A | 1 | n1 | 5 | REF_victim +15.7 |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n1A-n28A-n79A_UL_n1A-n28A | 1 | n1 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +14.9 |
| DL_n1A-n28A-n79A_UL_n1A-n79A | 1 | n1 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_victim +15.2 |
|  |  | n79 | 40 | REF_aggressor |
| DL_n1A-n28A-n79A_UL_n28A-n79A | 1 | n1 | 5 | REF_victim +1.2 |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n79 | 40 | REF_aggressor |
| DL_n1A-n41A-n77A_UL_n1A-n41A | 1 | n1 | 5 | REF_aggressor |
|  |  | n41 | 10 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +19.6 |
| DL_n1A-n41A-n77A_UL_n41A-n77A | 1 | n1 | 5 | REF_victim +9.3 |
|  |  | n41 | 10 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n1A-n78A-n79A_UL_n1A-n78A | 1 | n1 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +15.9 |
| DL_n1A-n78A-n79A_UL_n1A-n79A | 1 | n1 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +4.6 |
|  |  | n79 | 40 | REF_aggressor |
| DL_n1A-n78A-n79A_UL_n78A-n79A | 1 | n1 | 5 | REF_victim +15.6 |
|  |  | n78 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_aggressor |
| DL_n2A-n5A-n48A_UL_n5A-n48A | 1 | n2 | 5 | REF_victim +15.6 |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n48 | 5 | REF_aggressor |
| DL_n2A-n5A-n48A_UL_n2A-n5A | 1 | n2 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n48 | 5 | REF_victim +16.6 |
| DL_n2A-n5A-n66A_UL_n2A-n5A | 1 | n2 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +7.2 |
| DL_n2A-n5A-n77A_UL_n2A-n77A | 1 | n2 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_victim +3.8 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n5A-n77A_UL_n5A-n77A | 1 | n2 | 5 | REF_victim +16.5 |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n5A-n77A_UL_n2A-n5A | 1 | n2 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.0 |
| DL_n2A-n14A-n66A_UL_n2A-n14A | 1 | n2 | 5 | REF_aggressor |
|  |  | n14 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +7.6 |
| DL_n2A-n14A-n66A_UL_n14A-n66A | 1 | n2 | 5 | REF_victim +7.2 |
|  |  | n14 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
| DL_n2A-n14A-n77A_UL_n2A-n14A | 1 | n2 | 5 | REF_aggressor |
|  |  | n14 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.0 |
| DL_n2A-n14A-n77A_UL_n14A-n77A | 1 | n2 | 5 | REF_victim +16.5 |
|  |  | n14 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n30A-n77A_UL_n2A-n30A | 1 | n2 | 5 | REF_aggressor |
|  |  | n30 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim + 29.4 |
| DL_n2A-n30A-n77A_UL_n2A-n77A | 1 | n2 | 5 | REF_aggressor |
|  |  | n30 | 5 | REF_victim + 10.6 |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n2 | 5 | REF_aggressor |
|  |  | n30 | 5 | REF_victim + 3.4 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n30A-n77A_UL_n30A-n77A | 1 | n2 | 5 | REF_victim + 8.6 |
|  |  | n30 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n48A-n66A_UL_n2A-n66A | 1 | n2 | 5 | REF_aggressor |
|  |  | n48 | 5 | REF_victim +32 |
|  |  | n66 | 5 | REF_aggressor |
| DL_n2A-n48A-n66A_UL_n2A-n48A | 1 | n2 | 5 | REF_aggressor |
|  |  | n48 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +12.1 |
| DL_n2A-n48A-n66A_UL_n48A-n66A | 1 | n2 | 5 | REF_victim +28.3 |
|  |  | n48 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
| DL_n2A-n66A-n77A_UL_n2A-n66A | 1 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +29.4 |
|  | 2 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +8.9 |
| DL_n2A-n66A-n77A_UL_n2A-n77A | 1 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +29.2 |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +10.4 |
|  |  | n77 | 10 | REF_aggressor |
|  | 3 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +4.0 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n2A-n66A-n77A_UL_n66A-n77A | 1 | n2 | 5 | REF_victim +32.1 |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n2 | 5 | REF_victim +9.1 |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
|  | 3 | n2 | 5 | REF_victim +2.1 |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n5A-n78A_UL_n3A-n5A | 1 | n3 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +16.1 |
|  | 2 | n3 | 5 | REF_aggressor |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +4.5 |
| DL_n3A-n5A-n78A_UL_n5A-n78A | 1 | n3 | 5 | REF_victim + 15.7 |
|  |  | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
| DL_n3A-n8A-n78A_UL_n3A-n8A | 1 | n3 | 5 | REF_aggressor |
|  |  | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +16.1 |
|  | 2 | n3 | 5 | REF_aggressor |
|  |  | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +4.5 |
| DL_n3A-n8A-n78A_UL_n8A-n78A | 1 | n3 | 5 | REF_victim +15.7 |
|  |  | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
| DL_n3A-n28A-n78A_UL_n3A-n28A | 1 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +17.3 |
|  | 2 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +4.5 |
| DL_n3A-n28A-n41A_UL_n3A-n28A | 1 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +27.4 |
|  | 2 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +15.9 |
| DL_n3A-n28A-n41A_UL_n3A-n41A | 1 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_victim +26.0 |
|  |  | n41 | 5 | REF_aggressor |
| DL_n3A-n28A-n41A_UL_n28A-n41A | 1 | n3 | 5 | REF_victim +26.0 |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n41 | 10 | REF_aggressor |
| DL_n3A-n28A-n77A_UL_n3A-n28A | 1 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim 15.9 |
| DL_n3A-n28A-n77A_UL_n28A-n77A | 1 | n3 | 5 | REF_victim +17.0 |
|  |  | n28 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n28A-n77A_UL_n3A-n77A | 1 | n3 | 5 | REF_aggressor |
|  |  | n28 | 5 | REF_victim +15.3 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n40A-n77A_UL_n3A-n40A | 1 | n3 | 5 | REF_aggressor |
|  |  | n40 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim + 19.0 |
| DL_n3A-n40A-n77A_UL_n3A-n77A | 1 | n3 | 5 | REF_aggressor |
|  |  | n40 | 5 | REF_victim + 29.4 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n40A-n77A_UL_n40A-n77A | 1 | n3 | 5 | REF_victim + 29.9 |
|  |  | n40 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n41A-n77A_UL_n3A-n41A | 1 | n3 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.8 |
| DL_n3A-n41A-n77A_UL_n41A-n77A | 1 | n3 | 5 | REF_victim +16.4 |
|  |  | n41 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n3A-n41A-n77A_UL_n3A-n77A | 1 | n3 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +5.3 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n5A-n25A-n77A_UL_n5A-n25A | 1 | n5 | 5 | REF_aggressor |
|  |  | n25 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim + 16.0 |
| DL_n5A-n25A-n77A_UL_n5A-n77A | 1 | n5 | 5 | REF_aggressor |
|  |  | n25 | 5 | REF_victim + 16.5 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n5A-n25A-n77A_UL_n25A-n77A | 1 | n5 | 5 | REF_victim + 3.8 |
|  |  | n25 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n5A-n30A-n77A_UL_n5A-n30A | 1 | n5 | 5 | REF_aggressor |
|  |  | n30 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim + 16.1 |
| DL_n5A-n30A-n77A_UL_n5A-n77A | 1 | n5 | 5 | REF_aggressor |
|  |  | n30 | 5 | REF_victim + 13.2 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n5A-n30A-n77A_UL_n30A-n77A | 1 | n5 | 5 | REF_victim + 15.2 |
|  |  | n30 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n5A-n48A-n66A_UL_n5A-n66A | 1 | n5 | 5 | REF_aggressor |
|  |  | n48 | 5 | REF_victim +3.6 |
|  |  | n66 | 5 | REF_aggressor |
| DL_n5A-n66A-n77A_UL_n5A-n66A | 1 | n5 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.1 |
|  | 2 | n5 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +8.2 |
|  | 3 | n5 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +3.3 |
| DL_n5A-n66A-n77A_UL_n5A-n77A | 1 | n5 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +13.2 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n14A-n66A-n77A_UL_n14A-n66A | 1 | n14 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.0 |
| DL_n14A-n66A-n77A_UL_n14A-n77A | 1 | n14 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +13.2 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n14A-n66A-n77A_UL_n66A-n77A | 1 | n14 | 5 | REF_victim +15.2 |
|  |  | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| L_n28A-n40A-n71A_UL_n28A-n40A | 1 | n28 | 5 | REF_aggressor |
|  |  | n40 | 5 | REF_aggressor |
|  |  | n71 | 5 | REF_victim +3.9 |
| DL_n28A-n40A-n77A_UL_n28A-n40A | 1 | n28 | 5 | REF_aggressor |
|  |  | n40 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +26.7 |
| DL_n28A-n40A-n77A_UL_n28A-n77A | 1 | n28 | 5 | REF_aggressor |
|  |  | n40 | 5 | REF_victim +12.3 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n28A-n40A-n77A_UL_n40A-n77A | 1 | n28 | 5 | REF_victim +11 |
|  |  | n40 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n28A-n41A-n77A_UL_n28A-n41A | 1 | n28 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +28.2 |
| DL_n28A-n41A-n77A_UL_n28A-n77A | 1 | n28 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +29.5 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n28A-n41A-n77A_UL_n41A-n77A | 1 | n28 | 5 | REF_victim +30.8 |
|  |  | n41 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n28 | 5 | REF_victim +3.0 |
|  |  | n41 | 10 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n28A-n41A-n79A_UL_n41A-n79A | 1 | n28 | 5 | REF_victim +13 |
|  |  | n41 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_aggressor |
| DL_n28A-n41A-n79A_UL_n28A-n41A | 1 | n28 | 5 | REF_aggressor |
|  |  | n41 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +10.1 |
| DL_n28A-n41A-n79A_UL_n28A-n79A | 1 | n28 | 5 | REF_aggressor |
|  |  | n41 | 10 | REF_victim +10.4 |
|  |  | n79 | 40 | REF_aggressor |
| DL_n28A-n71A-n77A_UL_n28A-n77A | 1 | n28 | 5 | REF_aggressor |
|  |  | n71 | 10 | REF_victim +4.7 |
|  |  | n77 | 40 | REF_aggressor |
| DL_n28A-n71A-n77A_UL_n71A-n77A | 1 | n28 | 5 | REF_victim +4.3 |
|  |  | n71 | 10 | REF_aggressor |
|  |  | n77 | 40 | REF_aggressor |
| DL_n40A-n71A-n77A_UL_n40A-n71A | 1 | n40 | 5 | REF_aggressor |
|  |  | n71 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +16.3 |
| DL_n40A-n71A-n77A_UL_n40A-n71A | 1 | n40 | 5 | REF_aggressor |
|  |  | n71 | 5 | REF_victim +18.7 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n40A-n71A-n77A_UL_n40A-n71A | 1 | n40 | 5 | REF_victim +15.5 |
|  |  | n71 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_aggressor |
| DL_n66A-n71A-n77A_UL_n66A-n71A | 1 | n66 | 5 | REF_aggressor |
|  |  | n71 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +15.9 |
| DL_n28A-n78A-n79A_UL_n28A-n78A | 1 | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +26.2 |
| DL_n28A-n78A-n79A_UL_n28A-n79A | 1 | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +26.9 |
|  |  | n79 | 40 | REF_aggressor |
| DL_n28A-n78A-n79A_UL_n78A-n79A | 1 | n28 | 5 | REF_victim +16.2 |
|  |  | n78 | 10 | REF_aggressor |
|  |  | n79 | 40 | REF_aggressor |
| DL_n66A-n71A-n77A_UL_n66A-n77A | 1 | n66 | 5 | REF_aggressor |
|  |  | n71 | 5 | REF_victim +15.3 |
|  |  | n77 | 10 | REF_aggressor |
| DL_n66A-n71A-n78A_UL_n66A-n71A | 1 | n66 | 5 | REF_aggressor |
|  |  | n71 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +9 |
| DL_n66A-n71A-n78A_UL_n71A-n78A | 1 | n66 | 5 | REF_victim +15.5 |
|  |  | n71 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_aggressor |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: Void. Note 4: Void Note 5: Void Note 6: Void Note 7: The symbol “REF_victim” in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to reference sensitivity values for 2 antenna ports. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports. Note 9: The symbol "REF_aggressor" in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to the corresponding reference sensitivity values. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports, Table 7.3.2.5-2a and Table 7.3.2.5-2b for reference sensitivity values for 4 antenna ports, and Table 7.3.2.5-2e and Table 7.3.2.5-2f for reference sensitivity values for 8 antenna ports. Note 10: The test requirement for 2Rx victim band is “REF_victim + MSD”, for 4Rx victim band is “REF_victim + ΔRIB,4R + MSD +¦ΔRIB,4R¦ = REF_victim + MSD”, and for 8Rx victim band is “REF_victim + ΔRIB,8R + MSD +¦ΔRIB,8R ¦ = REF_victim+ MSD”, where the “MSD” refers to the MSD requirements for 2Rx antenna ports. |  |  |  |  |

-------- End of changes --------