3GPP TSG-RAN WG5 Meeting #109	R5-255887

Dallas, United States, Nov 17th – 21st, 2025

Agenda item:	6.3.29.6

Source:	Qualcomm

Title:	Discussion on aligning NTN event (D1/D2) test cases with core spec

WID/SID:	NR_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

Rel-17 event reporting test case 8.1.3.1.26 was introduced for Event D1.

Later for Rel-18 event reporting test case 8.1.3.1.27 was introduced for Event D2

In the other measurement procedure (e.g. A3), UE derives cell measurement results based on SSB/CSI-RS measurements for the trigger quantity and each measurement quantity indicated in reportQuantityCell. However, trigger quantity is not defined for eventD1/D2.

During RAN2#131 following clarifications have been added to the UE behaviour, for event D1/D2, which have impact on respective RAN5 test cases,

8.1.3.1.26	NR NTN / Measurement configuration control and reporting / Event D1 / Measurement of Neighbour NR cell

8.1.3.1.27	NR NTN Enhancements / Measurement configuration control and reporting / Event D2

# Discussion

In RAN5 NTN event reporting test procedures are written with the previous core spec understanding where UE must perform measurement for neighbour cells. However, latest core spec[3] has following changes,

The UE shall: - whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows: . . . . 2> if the reportType for the associated reportConfig is periodical, eventTriggered or 2> if the reportType for the associated reportConfig is eventTriggered, and the eventId is not set to eventD1 or eventD2; or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the MCG VarMeasConfig and is indicated in the condExecutionCond or in the condExecutionCondPSCell associated to a condReconfigId in the MCG VarConditionalReconfig (for CHO, CPA, MN-initiated inter-SN CPC, or subsequent CPAC in NR-DC), and the condEventId is not set to condEventD1, condEventD2 or condEventT1; or . . . NOTE 0: For CHO configured with only condEventD1, condEventD2 or condEventT1 (without any RRM measurement event), it is up to UE implementation whether and how to detect the applicable cell.

It was clarified in RAN2 with [1][2] that For measId configured with only eventD1/D2 for neighbour cells, UE does not need to perform RRM measurement or report result for neighbour cells in the report of the measId.

Also we found that for event trigger test cases, the report amount is set to infinity which is not a real world deployment scenario.

Summarizing,

Observation 1: RAN5 event reporting test cases (8.1.3.1.26, 8.1.3.1.27) expects RSRP reporting in the measurement reports.

As per the latest core spec clarification (TS 38.331, [1][2]) doesn’t require the UE to perform measurement when the measID is only configured event D1/D2

Observation 2: RAN5 event reporting test cases (8.1.3.1.26, 8.1.3.1.27) are defined with ReportAmount set to infinity

Report quantity to be set to 1. UE will get triggered to send one MR at the entry and one MR at the exit of the condition.

# Proposals

To address these followings are proposed for both eventD1 (8.1.3.1.26) and eventD2 (8.1.3.1.27) test cases are as below.,

Proposal1: In the measurement report check only cell ID but not RSRP (or RSRQ or other measurements)

Proposal2: Aligning the report quantity to real world deployment to 1.

Associated Prose CR correcting TC 8.1.3.1.26/27[5] [6] have been raised during RAN5#109

# Reference

[1] R2-2506289, “Corrections on eventD1 and condEventD1/T1”, Rel-17 38.331

[2] R2-2506294, “Corrections on eventD1/D2 and condEventD1/D2/T1”,  Rel-18 38.331

[3] TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification”, Rel-18 Ver 18.7.0

[4] TS 38.523-1, “5GS; User Equipment (UE) conformance specification; Part 1: Protocol”, Rel-19 Ver 19.2.0

[5] R5-255889, “Corrections to NTN Test Cases 8.1.3.1.26”, RAN5#109

[6] R5-255884, “Corrections to NTN Test Cases 8.1.3.1.27”, RAN5#109