3GPP TSG-RAN5 Meeting # 109	R5-256174

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

Source:	Huawei, HiSilicon

Title:	6G testing discussion

Agenda item:	4.2.5

Document for:	Discussion

# Introduction

After the start 6G in RAN5, RAN5 will need to maintain 4G/5G (even 2G/3G) while developing 6G, which will be a massive undertaking. Currently, RAN5 has initiated study on modernization of specification format and procedures for 6G. We suggest that RAN5 may also begin other discussions 6G, for example efficiency improvements and test simplification.

# Discussion

## Enhance standardization/templatization of test procedure language in 6G test spec.

Enhancing the standardization/templatization of test procedure language improves machine readability. This enables direct mapping to TTCN code from the test step in the SIG test procedure description.

For Example: TS 38.508-1 clause 4.5.2.2-2 contains 6 tables (Table 4.5.2.2-1/2/3/4/5/6). If test procedure only refered to clause number(in Figure 1), it might be difficult for an automated system to refer to the exact table/test procedure.

| 4-21 | Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 12. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| --- | --- | --- | --- | --- | --- |

Figure1

| 5-22 | Steps 3-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 2. Note: The UE performs initial registration procedure with REGISTRATION ACCEPT message including CAG information list with PLMN ID 3 and CAG only indication and the RRC connection is released. | - | - | - | - |
| --- | --- | --- | --- | --- | --- |

Figure2

More strict rules or Template can be introduced to standarized the test procedure description, and template can also be updated when new form test procedure is added, but How to make sure all the delegate are using the template or following the rules? The work load for checking the test procedure description may even larger than mapping test procedure to TTCN code by human.

Proposal 1: Study the rules or templates that can be introduced to standardize the test procedure description and use them when the first 6G work item starts.

Proposal 2: Tools can be introduced for both checking test procedures (such as checking overlapping Tdoc) and automatically mapping TTCN code.

## Reduce the SIG and RRM test case overlap to simplify test in 6G test spec.

Normally, SIG test case and RRM test case are designed separately, but several test case test similar UE behaviour.

For example, both of them test cell reselection, handover, redirection, re-establishment, and measurement procedure, while RRM test case adding delay/measurement accuracy test requirement in these test procedure. If the RRM test case passed, part of SIG test case for this feature can be skipped.

Proposal 3: Study the possibility of skipping part of the SIG test case if an RRM test case with a similar test procedure has already passed.

Some RRM subtest cases are testing pure signalling procedure without delay/timing/power… requirement. SIG is a better place to test this test scenario.

For example, 38.533 TC 6.3.2.2.1 Test4 only check UE can re-transmit msg3.

Proposal 4: RAN5 can make its own decision to test the Minimum conformance requirements listed by RAN4 in SIG.

# Conclusion

Proposal 1: Study the rules or templates that can be introduced to standardize the test procedure description and use them when the first 6G work item starts.

Proposal 2: Tools can be introduced for both checking test procedures (such as checking overlapping Tdoc) and automatically mapping TTCN code.

Proposal 3: Study the possibility of skipping part of the SIG test case if an RRM test case with a similar test procedure has already passed.

Proposal 4: RAN5 can make its own decision to test the Minimum conformance requirements listed by RAN4 in SIG.