3GPP TSG-RAN5 Meeting #109	R5-256533

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

Agenda item:	5.3.2.11

Source: 	Apple Inc

Title:       	Inputs to the Test Frequency Calculation Tool

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

The purpose of this paper is to provide some inputs based on validation of the test frequency tool that exists on 3GPP Forge and based on [1] and [2].  An RF Action point #104.21 was created for this effort. The recommendation from the coordinator for this effort was to begin validation with CA_n41(2A). Some outcomes and observations from our effort are shared in this contribution.

# 2	Discussion

## 2.1 Tool Validation

## The 3GPP frequency calculation tool was executed for Band n41(2A) - CA Intra Band Non-Contiguous with two component carriers using 30 kHz subcarrier spacing. The tool generated test frequencies for all valid bandwidth combinations in CA Intra Band Non-Contiguous (NONCONTUL) mode with uplink carrier aggregation enabled.

The tool successfully displayed these associated files for n41(2A):

CA_nonContUL_n41_30.csv - Test frequency calculations in CSV format for all n41(2A) combinations

CA_nonContUL_n41_30.json - Structured test configuration in JSON format with carrier details.

The CSV file contained 24 bandwidth combinations for Band n41(2A) ranging from 40+40 MHz to 100+100 MHz aggregations. Each combination included:

- Component carrier bandwidths (CBWs) conforming to TS 38.101-1 Table 5.3.5-1

- Bandwidth Combination Set (BCS) indicator - all combinations showed BCS 0 for n41(2A)

- MaxFsBW (Maximum Full Subband Bandwidth) values per TS 38.508-1 Table C.2.4.3.1A-1

- MaxWgap (Maximum Gap) between the two carriers calculated as: (MaxFsBW - BW1 - BW2) / 1

Observation 1 — In general, thanks to the instructions provided by the developer, the tool was intuitive to install, setup and execute with basic understanding of the script interface.

One observation made during the validation, for additional investigation, is that the getFreq2() function, which calculates the second carrier's frequency based on the gap (Wgap) from the first carrier, could produce frequencies that violate band edges due to cumulative rounding errors from ARFCN/channel raster alignment.

On a review of the above function, it seems to already have some band edge constraints, but no safety margin to account for post-calculation rounding when frequencies are converted to ARFCN values and back.

A possible fix might be to add a safety margin equal to one raster step (deltaFraster) to the band edge constraint calculation to prevent rounding from pushing carrier edges beyond band limits.

Current implementation in getFreq2() calculates the maximum allowable center frequency for the second carrier by:

#1  Computing the ideal spacing based on Wgap

#2  Applying a band edge constraint: edge_limit = FDL - BW2/2

The enhancement modifies step 2 to:

edge_limit = FDL - BW2/2 - deltaFraster

Proposal 1: Investigate modifying the band edge constraint calculation in getFreq2() from edge_limit = FDL - BW2/2 to edge_limit = FDL - BW2/2 - deltaFraster, reserving one raster step (e.g., 0.03 MHz for 30 kHz SCS) as a buffer. This prevents subsequent ARFCN conversion and channel raster alignment from causing the carrier's upper edge to exceed the band limit

# Summary

Observation 1 — In general, thanks to the instructions provided by the developer, the tool was intuitive to install, setup and execute with basic understanding of the script interface.

Proposal 1: Investigate modifying the band edge constraint calculation in getFreq2() from edge limit = FDL - BW2/2 to edge limit = FDL - BW2/2 - deltaFraster, reserving one raster step (e.g., 0.03 MHz for 30 kHz SCS) as a buffer. This prevents subsequent ARFCN conversion and channel raster alignment from causing the carrier's upper edge to exceed the band limit

# 3 References

RF AP#104.21 - Implement a python based tool for calculating test frequencies supporting at least 1 CA configuration for Intraband non-contiguous CA, including UL CA(CA_n77(2A), CA_n79C)

R5-254695 “Test Frequency Calculation Tool, updates and way forward", Ericsson, RAN5#108, Bengaluru India, Aug 25th to Aug 29th, 2025