3GPP TSG-RAN5 Meeting #109	R5-256605

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5495 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.3.1.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the current Prose, in table 22.3.1.13.3.2-1, at Step 19 a differential Koffset set to 5ms. This differential Koffset value is reasonable for UE in GSO mode, as in GSO, the Koffset value is 550ms. In NGSO mode, the Koffset value is 5ms, and having the differential Koffset value as 5ms, would result in NTN koffset as “zero”. This is unreasonable and causes TA problem for UE to sync due to no Koffset being applied in NTN mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to set differential Koffset as 1ms for NGSO mode at step 19; and set to 5ms for GSO. Updated Steps 19 and 21 accordingly |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case in NGSO mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is final version of R5-255859 |  |  |  |  |  |  |  |  |

#### 22.3.1.13	NB-IoT / NTN / UE specific TA report / UE specific Koffset

##### 22.3.1.13.1	Test Purpose (TP)

(1)

with { UE in RRC_IDLE state and ta-Report is configured with value enabled in SystemInformationBlockType2-NB }

ensure that {
  when { during RRC Connection establishment procedure }

then { UE performs the Timing Advance reporting procedure }

}

(2)

with { UE in RRC_IDLE state and ta-Report is not configured in SystemInformationBlockType2-NB }

ensure that {

when { RRCConnectionSetup-NB with offsetThresholdTA is received }

then { UE performs the Timing Advance reporting procedure }

}

(3)

with { UE in RRC_CONNECTED state and k-Offset is broadcasted in SystemInformationBlockType31-NB }

ensure that {
  when { Differential Koffset MAC CE is received }

then { UE applies received Differential Koffset and performs transmission on NPUSCH  }

}

##### 22.3.1.13.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clauses 5.3.3.2, TS 36.321 clauses 5.4.9, 5.26, 6.1.3.20 and 6.3.1.21. Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.3.3.2]

For NB-IoT, upon initiation of the procedure, the UE shall:

…

1>	if UE supports timing advance reporting and ta-Report is included in SystemInformationBlockType2-NB:

2>	instruct the associated MAC entity to trigger Timing Advance reporting;

[TS 36.321, clause 5.4.9]

The UE may be configured to report information about UE specific timing advance during a Random Access procedure and in RRC_CONNECTED Mode.

The Timing Advance reporting procedure is used in a non-terrestrial network to provide the eNB with an estimate of the UE's Timing Advance, see TTA in TS 36.211 [7] clause 8.1.

Timing Advance reporting shall be triggered if any of the following events occur:

-	if triggered by upper layers;

-	upon configuration or reconfiguration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell;

-	if the variation between current information about Timing Advance and the last reported information about Timing Advance is equal to or larger than offsetThresholdTA, if configured.

If the Timing Advance reporting procedure determines that at least one Timing Advance Report has been triggered and not cancelled:

-	if the MAC entity has UL resources allocated for new transmission for this TTI, and;

-	if the allocated UL resources can accommodate the Timing Advance Report MAC control element plus its subheader, as a result of logical channel prioritization:

-	instruct the Multiplexing and Assembly procedure to generate the Timing Advance report MAC control element as defined in clause 6.1.3.20.

A MAC PDU shall contain at most one Timing Advance Report MAC CE, even when multiple events have triggered a Timing Advance report.

All triggered Timing Advance reports shall be cancelled when a Timing Advance Report MAC CE is included in a MAC PDU for transmission.

[TS 36.321, clause 5.26]

The network may provide and update the Differential Koffset of a Serving Cell in a non-terrestrial network by sending the Differential Koffset MAC CE described in clause 6.1.3.21.

The MAC entity shall:

-	if the MAC entity receives a Differential Koffset MAC CE on a Serving Cell:

-	indicate to lower layers the information regarding the Differential Koffset MAC CE.

[TS 36.321, clause 6.1.3.20]

The Timing Advance MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2.

It has a fixed size and consists of a single field defined as follows (Figure 6.1.3.20-1):

-	R: Reserved bit, set to 0;

-	Timing Advance: The Timing Advance field indicates the least integer number of subframes greater than or equal to the Timing Advance value (see TS 36.211 [7] clause 8.1). The length of the field is 14 bits.

Figure 6.1.3.20-1: Timing Advance MAC CE

[TS 36.321, clause 6.1.3.21]

The Differential Koffset MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-1.

It has a fixed size and consists of a single field defined as follows (Figure 6.1.3.21-1):

-	R: Reserved bit, set to 0;

-	Differential Koffset: This field contains the differential Koffset. The length of the field is 6 bits.

Figure 6.1.3.21-1: Differential Koffset MAC CE

##### 22.3.1.13.3	Test description

22.3.1.13.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 is configured according to Table 8.1.4.2-1 and Table 8.1.4.2-2 in TS 36.508 [18].

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble

-	The UE is in state 1-NB according to TS 36.508 [18] clause 8.1.5.1.

22.3.1.13.3.2	Test procedure sequence

Table 22.3.1.13.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on UE. | - | - | - | - |
| 2 | The UE transmits preamble on NPRACH. | --> | NPRACH Preamble | - | - |
| 3 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 2. | <-- | Random Access Response | - | - |
| 4 | The UE transmit an RRCConnectionRequest-NB message. | --> | MAC PDU (RRCConnectionRequest-NB) | - | - |
| 4A | The SS transmits an RRCConnectionSetup-NB message. | <-- | MAC PDU (RRCConnectionSetup-NB) | - | - |
| 4B | Check: Does the UE transmit an RRCConnectionSetupComplete-NB message with one Timing Advance Report MAC CE? | --> | MAC PDU (Timing Advance Report MAC CE, RRCConnectionSetupComplete-NB) | 1 | P |
| 5-6 | Void |  |  |  |  |
| 7-16 | Steps 5 to 14b1 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. | - | - | - | - |
| 16A | If possible switch off is performed or the USIM is removed, otherwise the power is removed. | - | - | - | - |
| 16B-16C | Void |  |  |  |  |
| - | EXCEPTION: Step 16C1a1 to 16C1b2 describes behaviour that depends on the UE capability. | - | - | - | - |
| 16C1a1 | If pc_SwitchOnOff or pc_USIM_Removal, then the UE transmits a DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 16C1a2 | The SS transmits an RRCConnectionRelease-NB message to release RRC connection. | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 16C1b1 | Else power off the UE | - | - | - | - |
| 16C1b2 | SS locally releases the RRC connection | - | - | - | - |
| 16D | SS broadcasts SystemInformationBlockType2-NB with ta-Report-r17 not present. | <-- | RRC: SystemInformationBlockType2-NB | - | - |
| 16E | Power on UE. | - | - | - | - |
| 16F | The UE transmits preamble on NPRACH. | --> | NPRACH Preamble | - | - |
| 16G | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 16F. | <-- | Random Access Response | - | - |
| 16H | The UE transmits an RRCConnectionRequest-NB message. | --> | MAC PDU (RRCConnectionRequest-NB) | - | - |
| 17 | The SS transmits an RRCConnectionSetup-NB message to configure the offsetThresholdTA. | <-- | MAC PDU(RRCConnectionSetup-NB) | - | - |
| 18 | Check: Does the UE transmit an RRCConnectionSetupComplete-NB message with one Timing Advance Report MAC CE? | --> | MAC PDU (Timing Advance Report MAC CE, RRCConnectionSetupComplete-NB) | 2 | P |
| 18A-18J | Steps 5 to 14b1 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1. | - | - | - | - |
| 19 | The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 5 ms (GSO scenario) or 1ms (NGSO scenario). | <-- | MAC PDU (Differential Koffset MAC CE) | - | - |
| 20 | The SS sends UECapabilityEnquiry-NB message to the UE. | <-- | UECapabilityEnquiry-NB | - | - |
| 21 | Check: Does UE send a UECapabilityInformation-NB message after applying differential Koffset of 5ms (GSO scenario) or 1ms (NGSO scenario) received at step 19? Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. | --> | UECapabilityInformation-NB | 3 | P |

22.3.1.13.3.3	Specific message contents

Table 22.3.1.13.3.3-1: SystemInformationBlockType2-NB (preamble, Table 22.3.1.13.3.2-1)

| Derivation Path: TS 36.508 [18], Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon-r13 SEQUENCE { |  |  |  |
| ntn-ConfigCommon-r17 SEQUENCE { |  |  |  |
| ta-Report-r17 | enabled |  | Preamble |
|  | Not present |  | Step 16D |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.13.3.3-2: RRCConnectionSetup-NB (step 17, Table 22.3.1.13.3.2-1)

| Derivation Path: TS 36.508 [18], Table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| mac-MainConfig-r13 CHOICE { |  |  |  |
| explicitValue-r13 SEQUENCE { |  |  |  |
| offsetThresholdTA-r17 CHOICE { |  |  |  |
| setup | ms3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.3.1.13.3.3-3: RRCConnectionRequest-NB (steps 4 and 16H, Table 22.3.1.13.3.2-1)

| Derivation Path: TS 36.508 [18], Table 8.1.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r13 SEQUENCE { |  |  |  |
| ue-Identity-r13 CHOICE { |  |  |  |
| randomValue | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |