3GPP TSG-RAN5 Meeting #109	R5-256688

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5189 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTM test case 7.1.3.5.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New LTM test case 7.1.3.5.13 need to be added according to work plan of the WI NR_MC_enh-UEConTest. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new test case 7.1.3.5.13. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MC_enh-UEConTest WP is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.13(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.523-2 CR0639 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-255898 with two revisions. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-255898 -> R5-256688 |  |  |  |  |  |  |  |  |

==============First change==============

7.1.3.5.13	LTM cell switch / Master Key / PDCP persists

7.1.3.5.13.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with candidate configuration for LTM }

ensure that {

when { UE receives LTM Cell Switch Command MAC CE }

then { UE triggers RACH procedure on LTM Cell }

}

(2)

with { UE in RRC_Connected state with candidate configuration for LTM }

ensure that {

when { UE receives RRCReconfiguration with new candidate configuration for LTM }

then { UE takes the new configuration into account at the next LTM cell switch }

}

7.1.3.5.13.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.3.5.18.6 and 5.3.1.2, TS 38.323 clauses 5.2.2.1, 5.2.2.2, 5.2.2.3, 7.1 and 7.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.5.18.6]

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

1>	if the LTM cell switch is triggered on the MCG:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 00:	For all radio bearers and RLC bearers included in the LTM candidate configuration to be applied at an LTM cell switch execution (i.e., as derived from the LTM reference configuration and the LTM candidate configuration), even if those radio bearers and RLC bearers were configured before the LTM cell switch execution, the network includes fields as specified for the initial setup of radio bearers and RLC bearers and sets the values previously stored by the UE for the fields that cannot be modified according to presence conditions or field descriptions.

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group for which the LTM cell switch procedure is triggered, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group for which the LTM cell switch procedure is triggered;

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

NOTE 0:	The UE is not expected to perform UE-based TA measurements for an SpCell.

1>	if ltm-ConfigComplete is not included within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3:

2>	consider ltm-ReferenceConfiguration in ltm-Config, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations to be released by the actions above in this procedure;

2>	if measConfig is included within ltm-ReferenceConfiguration in ltm-Config;

3>	perform the measurement configuration procedure as specified in clause 5.5.2 by considering the measConfig within ltm-ReferenceConfiguration in ltm-Config as the received measConfig:

NOTE 1:	When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3, unless specified otherwise in this clause.

1>	if the LTM cell switch is triggered by an indication from lower layers:

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3;

1>	else (LTM cell switch triggered upon cell selection performed while timer T311 was running):

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3;

1>	if the LTM cell switch is triggered on the MCG:

2>	release the radio bearer(s) using the master key and the MCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release the radio bearer(s) using the secondary key and the SCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

NOTE 2:	When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.

[TS 38.323, clause 5.2.2.1]

In this clause, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU and consider it as not received;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers after performing header decompression using EHC.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.2]

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.3]

When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the receiving PDCP entity shall:

-	update RX_REORD to RX_NEXT;

-	stop and restart t-Reordering.

[TS 38.331, clause 5.3.1.2]

For a UE provided with an sk-counter, keyToUse indicates whether the UE uses the master key (KgNB) or the secondary key (S-KeNB or S-KgNB) for a particular DRB. The secondary key is derived from the master key and sk-Counter, as defined in TS 33.501[11]. Whenever there is a need to refresh the secondary key, e.g. upon change of MN with KgNB change or to avoid COUNT reuse, the security key update is used (see 5.3.5.7). When the UE is in NR-DC, the network may provide a UE configured with an SCG with an sk-Counter even when no DRB is setup using the secondary key (S-KgNB) in order to allow the configuration of SRB3. The network can also provide the UE with an sk-Counter, even if no SCG is configured, when using SN terminated MCG bearers.

[TS 38.323, clause 7.1]

This clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this clause are normative.

All state variables are non-negative integers, and take values from 0 to [232 – 1].

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-SizeUL] – 1] or 0 to [2[pdcp-SN-SizeDL] – 1] or 0 to [2[sl-PDCP-SN-Size] – 1].

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0, except for SRBs configured with state variables continuation. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, for multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

NOTE 1:	For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, it is up to UE implementation to select the HFN part for RX_NEXT such that initial value of RX_DELIV should be a positive value.

NOTE 2:	For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the HFN part of RX_NEXT is set by UE implementation.

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, the initial value of the SN part of RX_DELIV is (x – 0.5 × 2[sl-PDCP-SN-Size–1]) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the SN part of RX_DELIV is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs, the initial value of RX_DELIV is set, if provided, by initialRX-DELIV in TS 38.331 [3]. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

NOTE 3:	For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the HFN part of RX_DELIV is set by UE implementation.

c)	RX_REORD

This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

[TS 38.323, clause 7.3]

The transmitting PDCP entity shall maintain the following timers:

a) discardTimer

This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331 [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer as specified in clause 5.2.1.

b) discardTimerForLowImportance

This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331 [3]. In the transmitter, a new timer is started upon reception of an SDU belonging to a low importance PDU Set from upper layer as specified in clause 5.2.1.

The receiving PDCP entity shall maintain the following timers:

c) t-Reordering

The duration of the timer is configured by upper layers TS 38.331 [3], except for the case of NR sidelink communication or sidelink SRB4. For NR sidelink communication or sidelink SRB4, the t-Reordering timer is determined by the UE implementation. This timer is used to detect loss of PDCP Data PDUs as specified in clause 5.2.2. If t-Reordering is running, t-Reordering shall not be started additionally, i.e. only one t-Reordering per receiving PDCP entity is running at a given time.

7.1.3.5.13.3	Test description

7.1.3.5.13.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with the exception except the following:

-	3 NR cells (NR Cell 1, NR Cell 2 and NR Cell 10) are configured. NR Cell 1 is the PCell, NR Cell 10 is the PSCell, neighbour cell NR Cell 2 is on the same frequency as the PCell.

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC), Bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4. The UE uses the master key for MCG DRB.

-	AM DRB is configured according to Table 7.1.3.5.13.3.1-1.

Table 7.1.3.5.13.3.1-1: PDCP parameters

| Parameter | Value |
| --- | --- |
| t-Reordering | ms750 |
| statusReportRequired | Not present |

7.1.3.5.13.3.2	Test procedure sequence

Tables 7.1.3.5.13.3.2-1 and 7.1.3.5.13.3.2-2 illustrate the downlink power levels to be applied for NR Cell 1, NR Cell 2 and NR Cell 10 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 7.1.3.5.13.3.2-3.

Table 7.1.3.5.13.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -88 | -94 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -88 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.3.5.13.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.3.5.13.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 including MeasConfig to setup NR measurement and reporting for intra-frequency event A3. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message containing RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS sends the PDCP SDU #1 D/C field = 1 (PDCP Data PDU) and PDCP SN = 1 on NR Cell 1. | <-- | (PDCP SDU #1) | - | - |
| 4 | The SS sends the PDCP SDU #2 D/C field = 1 (PDCP Data PDU) and PDCP SN =2 on NR Cell 1. | <-- | (PDCP SDU #2) | - | - |
| 5 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.3.5.13.3.2-1/2. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message embedded in NR RRC message ULInformationTransferMRDC to report event A3 with the measured RSRP for NR Cell 2 on NR Cell 1. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCReconfiguration message on NR Cell 1 containing LTM-Candidate-r18 for NR Cell 2. | <-- | RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS transmits an RRCReconfiguration message on NR Cell 1 with t-Reordering set to ms160 for NR Cell 2. | <-- | RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 11 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 2 on NR Cell 1. | <-- | MAC PDU (Candidate Cell TCI state activation/deactivation MAC CE) | - | - |
| 12 | The SS transmits DCI 1_0 PDCCH order to trigger the UE PRACH transmission on NR Cell 1. | <-- | (PDCCH Order) | - | - |
| 13 | Check: Does UE transmits the PRACH preamble on NR Cell 2? | --> | (PRACH Preamble) | 1 | P |
| 14 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to 0 and Target Configuration ID value set to 0 to the UE on NR Cell 1. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message on NR Cell 2. | --> | RRCReconfigurationComplete | - | - |
| 16 | Wait 160ms from step 3 until t-Reordering expires. | - | - | - | - |
| 17 | The SS sends the PDCP SDU #0 D/C field = 1 (PDCP Data PDU) and PDCP SN = 0 on NR Cell 2. | <-- | (PDCP SDU #0) | - | - |
| - | EXCEPTION: Steps 18-20 below can occur in any order in the next 1 second. | - | - | - | - |
| 18 | Check: Does the UE transmit the PDCP SDU #0 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 0 on NR Cell 2? | --> | (PDCP SDU #0) | 2 | F |
| 19 | Check: Does the UE transmit the PDCP SDU #1 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 1 on NR Cell 2? | --> | (PDCP SDU #1) | 2 | F |
| 20 | Check: Does the UE transmit the PDCP SDU #2 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 2 on NR Cell 2? | --> | (PDCP SDU #2) | 2 | F |

7.1.3.5.13.3.3	Specific message contents

Table 7.1.3.5.13.3.3-1: RRCReconfiguration (step 1, Table 7.1.3.5.13.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with conditions NR_MEAS

Table 7.1.3.5.13.3.3-2: MeasConfig (Table 7.1.3.5.13.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.1.3.5.13.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 7.1.3.5.13.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.13.3.3-3: MeasObjectNR (Table 7.1.3.5.13.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.13.3.3-4: ReportConfigNR-EventA3 (Table 7.1.3.5.13.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.13.3.3-5: MeasurementReport (step 7, Table 7.1.3.5.13.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.13.3.3-6: RRCReconfiguration (step 8, Table 7.1.3.5.13.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with conditions LTM

Table 7.1.3.5.13.3.3-7: LTM-Config-r18(Table 7.1.3.5.13.3.3-6)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | Table 7.1.3.5.13.3.3-8 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfig-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.13.3.3-8: LTM-Candidate-r18 (Table 7.1.3.5.13.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 1 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 2 |  |  |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig specified in TS 38.508-1 [4] Table 4.6.3-19 with condition NR-DC_SCG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| } |  |  |  |

==============End of change==============