3GPP TSG-RAN WG5 Meeting #109	R5-256702

DALLAS, USA, 17th – 21st Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3609 | rev | 1 | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default message contents for SCI format 1-B and SCI format 2-D |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SCI format 1-B is used for the scheduling of SL PRS for a dedicated SL PRS resource pool. SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS for a shared SL PRS resource pool. The message contents of these two SCI are missing now. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added SCI format 1-B default message contents. - Added SCI format 2-D default message contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

4.3.6.2	Sidelink physical layer parameters

4.3.6.2.1	Physical layer parameters for scheduling of PSSCH on PSCCH

4.3.6.2.1.1	Physical layer parameters for SCI format 1-A

SCI format 1-A is used to schedule PSSCH transmission.

Default physical layer parameters for SCI format 1-A are specified in Table 4.3.6.2.1.1-1.

Table 4.3.6.2.1.1-1: Physical layer parameters for SCI format 1-A

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Priority | Dependent on test parameters | - |  |
| Frequency resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | only one PSCCH/PSSCH occasion is scheduled per grant | "00000"B |  |
| Resource reservation period | Not present | - |  |
| DMRS pattern | Indicating the first entry of sl-PSSCH-DMRS-TimePatternList-r16 to be used | "0"B |  |
| 2nd-stage SCI format | SCI format 2-A | “00”B |  |
|  | SCI format 2-B | “01”B | DISTANCE-BASED HARQ |
|  | SCI format 2-D | “11”B | SLPP |
| Beta_offset indicator | Indicating the first entry of sl-BetaOffsets2ndSCI to be used | "00"B |  |
| Number of DMRS port | Single port p = 1000 is used | "0"B |  |
|  | Two ports p = 1000 and 1001 are used | "1"B | SL MIMO |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| Additional MCS table indicator | Not present | - |  |
| PSFCH overhead indication | "1"B if mod (i, nPSFCH) = 0 "0"B if mod (i, nPSFCH ) ≠ 0 where i is the logical slot index belong to resource pool per 1024 frame as specified in 38.214 [21] clause 8. | - | nPSFCH = 2 OR nPSFCH = 4 (Note 1) |
|  | Not present |  |  |
| Reserved | Number of reserved bits is given by sl-NumReservedBits-r16 | "00" |  |
| Note 1: nPSFCH is the PSFCH period (in slots) provided by sl-PSFCH-Period-r16. nPSFCH = 0 if sl-PSFCH-Period-r16 is not configured. |  |  |  |

| Condition | Explanation |
| --- | --- |
| DISTANCE-BASED HARQ | For distance-based HARQ test |
| SL MIMO | For SL MIMO test |
| SLPP | For SLPP test cases |

4.3.6.2.1.2	Physical layer parameters for SCI format 1-B

SCI format 1-B is used for the scheduling of SL PRS for a dedicated SL PRS resource pool.

Default physical layer parameters for SCI format 1-B are specified in Table 4.3.6.2.1.2-1.

Table 4.3.6.2.1.2-1: Physical layer parameters for SCI format 1-B

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Priority | Dependent on test parameters | - |  |
| Source ID | Dependent on test parameters | - |  |
| Destination ID | Dependent on test parameters | - |  |
| Cast type indicator | Unicast | "10"B |  |
| Resource reservation period | Not present (0 bit, there is one entry in the higher layer parameter sl-PRS-ResourceReservePeriodList) | - |  |
| Time resource assignment | only one PSCCH/PSSCH occasion is scheduled per grant | "00000"B |  |
| Resource ID indication | Not present (0 bit, there is one entry in the higher layer parameter sl-PRS-ResourcesDedicatedSL-PRS-RP) | - |  |
| SL PRS request | Not present (0 bit, sl-SCI-basedSL-PRS-TxTriggerSCI1-B is not configured) | - |  |
| Reserved | Not present | - |  |

4.3.6.2.2	Physical layer parameters for scheduling on PSSCH

4.3.6.2.2.1	Physical layer parameters for SCI format 2-A

SCI format 2-A is used to provide HARQ information, to provide Souce/Destination L1 ID or to trigger CSI reporting.

Default physical layer parameters for SCI format 2-A are specified in Table 4.3.6.2.2.1-1.

Table 4.3.6.2.2.1-1: Physical layer parameters for SCI format 2-A

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| HARQ process number | Dependent on test parameters |  |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| Source ID | Dependent on test parameters | - |  |
| Destination ID | Dependent on test parameters | - |  |
| HARQ feedback enabled/disabled indicator | HARQ feedback is enabled | “1”B |  |
| Cast type indicator | broadcast | "00"B | BROADCAST |
|  | groupcast with ACK/NACK HARQ | "01"B | GROUPCAST_ACK_NACK |
|  | unicast | "10"B | UNICAST |
|  | groupcast with NACK-only HARQ | "11"B | GROUPCAST_NACK_ONLY |
| CSI request | Not ask for SL CSI report | "0"B |  |
|  | Ask for SL CSI report | "1"B | SL_CSI |

| Condition | Explanation |
| --- | --- |
| BROADCAST | For broadcast SL communication test |
| GROUPCAST_ACK_NACK | For groupcast SL communication with ACK/NACK HARQ feedback test |
| GROUPCAST_NACK_ONLY | For groupcast SL communication with NACK-only HARQ feedback test |
| UNICAST | For unicast SL communication test |
| SL_CSI | For SL CSI reporting test |

4.3.6.2.2.2	Physical layer parameters for SCI format 2-B

SCI format 2-B is used to provide HARQ information, Souce/Destination L1 ID or distance-based HARQ related parameters.

Default physical layer parameters for SCI format 2-B are specified in Table 4.3.6.2.2.2-1.

Table 4.3.6.2.2.2-1: Physical layer parameters for SCI format 2-B

| Parameter | Value | Value in binary |
| --- | --- | --- |
| HARQ process number | Dependent on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 | - |
| Redundancy version | Dependent on test parameters | - |
| Source ID | Dependent on test parameters | - |
| Destination ID | Dependent on test parameters | - |
| HARQ feedback enabled/disabled indicator | HARQ feedback enabled | “1”B |
| Zone ID | Dependent on test parameters | - |
| Communication range requirement | Indicating the first entry of sl-ZoneConfigMCR-List to be used | "0000"B |

4.3.6.2.2.3	Physical layer parameters for SCI format 2-D

SCI format 2-D is used for the decoding of PSSCH and the scheduling of SL PRS for a shared SL PRS resource pool.

Default physical layer parameters for SCI format 2-D are specified in Table 4.3.6.2.2.3-1.

Table 4.3.6.2.2.3-1: Physical layer parameters for SCI format 2-D

| Parameter | Value | Value in binary |
| --- | --- | --- |
| SL PRS resource ID | Not present (0 bit, there is one entry in the higher layer parameter sl-PRS-ResourcesSharedSL-PRS-RP) | - |
| SL PRS request | Not present (0 bits, sl-SCI-basedSL-PRS-TxTriggerSCI2-D is not configured) | - |
| Embedded SCI format | Dependent on test parameters | - |
| Embedded SCI format payload | Dependent on test parameters | - |