3GPP TSG WG RAN5 Meeting #109	R5-256719

# Dallas, Texas, USA 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-2 | CR | 0011 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to test case 6.6.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet, Ericsson, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - References to some Tables have incorrect table numbers in the test case ‘6.6.4 On-network / Emergency Alert / Emergency Alert Area’. - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected Tables numbers in the references to some Tables: - Corrected Table number (Table 5.5.3.2.1-2 changed to Table 5.5.3.2.1-1) in first row of 'Table 6.6.4.3.3-4: MCPTT-Info in SIP MESSAGE (Table 6.6.4.3.3-3)' in the Clause 6.6.4.3.3. - Corrected Table number (Table 6.6.1.3.3-5 changed to Table 6.6.4.3.3-7) in 5th row 2nd column of 'Table 6.6.4.3.3-6: SIP MESSAGE from the SS (step 3, Table 6.6.4.3.2-1; step 4, TS 37.579-1 [2] Table 5.3.30.3-1)' in the Clause 6.6.4.3.3. - Corrected Table number (Table 6.6.4.3.3-2 changed to Table 6.6.4.3.3-9) in 5th row 2nd column of 'Table 6.6.4.3.3-8: SIP MESSAGE from the SS (step 4, Table 6.6.4.3.2-1; step 2, TS 37.579-1 [2] Table 5.3.33.3-1)' in the Clause 6.6.4.3.3. - Made minor editorial changes in the 'Table 6.6.4.3.2-1: Main Behaviour' of the clause 6.6.4.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect Table numbers in the references to some Tables in the test case ‘6.6.4 On-network / Emergency Alert / Emergency Alert Area will remain incorrect’. Editorial errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications. |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final version of R5-255766 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

### 6.6.4	On-network / Emergency Alert / Emergency alert area

…

6.6.4.3.2	Test procedure sequence

Table 6.6.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to receive notification of entry into an emergency alert area? | - | - | 1 | P |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication that an emergency alert area has been entered. | - | - | - | - |
| 2a1 | IF pc_MCX_DisplayInfoEmergencyAlertArea THEN Check: Does the UE (MCPTT client) notify the user that an emergency alert area has been entered? (NOTE 1) | - | - | 1 | P |
| 2A | Make the UE (MCPTT client) send an emergency alert with location information. (NOTE 1) | - | - | - | - |
| 3 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX SIP MESSAGE Request - Accept CO' as described in TS 37.579-1 [2] Table 5.3.30.3-1 to request an emergency alert providing location information? |  |  | 2 | P |
| 4 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to receive notification of exit from an emergency alert area? | - | - | 3 | P |
| - | EXCEPTION: Step 5a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication that an emergency alert area has been exited. |  |  |  |  |
| 5a1 | IF pc_MCX_DisplayInfoEmergencyAlertArea THEN Check: Does the UE (MCPTT client) notify the user that a group geographic area has been exited? (NOTE 1) |  |  | 3 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.6.4.3.3	Specific message contents

Table 6.6.4.3.3-1: SIP MESSAGE from the SS (step 1, Table 6.6.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO, ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.6.4.3.3-2 |  |  |  |

Table 6.6.4.3.3-2: MCPTT-Info in SIP MESSAGE (Table 6.6.4.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| associated-group-id | px_MCPTT_Group_A_ID |  |  |  |
| anyExt |  |  |  |  |
| emergency-alert-area-ind | true |  |  |  |

Table 6.6.4.3.3-3: SIP MESSAGE from the UE (step 3, Table 6.6.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.1-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.6.4.3.3-4 |  |  |  |
| MIME body part |  | Location-info |  |  |
| MIME-part-body | Location-info as described in Table 6.6.4.3.3-5 |  |  |  |

Table 6.6.4.3.3-4: MCPTT-Info in SIP MESSAGE (Table 6.6.4.3.3-3)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | Encrypted <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_A_ID | Encryption according to NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-1 |  |  |
| alert-ind | Encrypted <alert-ind> with mcpttBoolean set to true | Encryption according to NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-1 |  |  |
| mcptt-client-id | encrypted <mcptt-client-id> with mcpttString set to the UE’s UUID URN | Encryption according to NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-1 |  |  |

Table 6.6.4.3.3-5: Location-Info in SIP MESSAGE (Table 6.6.4.3.3-3)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.4.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| CurrentLocation |  |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted <longitude> with any content | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.4.1-1 |  |  |
| latitude | Encrypted <latitude> with any content | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.4.1-1 |  |  |

Table 6.6.4.3.3-6: SIP MESSAGE from the SS (step 3, Table 6.6.4.3.2-1;
step 4, TS 37.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.6.4.3.3-7 |  |  |  |

Table 6.6.4.3.3-7: MCPTT-Info in SIP MESSAGE (Table 6.6.4.3.3-6)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |
| alert-ind | Encrypted <alert-ind> with mcpttBoolean set to true | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-1 |  |  |
| mcptt-client-id | Encrypted <mcptt-client-id> with mcpttString set to the client-id as received from the UE | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-1 |  |  |
| alert-ind-rcvd | true |  |  |  |

Table 6.6.4.3.3-8: SIP MESSAGE from the SS (step 4, Table 6.6.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO, ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.6.4.3.3-9 |  |  |  |

Table 6.6.4.3.3-9: MCPTT-Info in SIP MESSAGE (Table 6.6.4.3.3-8)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| associated-group-id | px_MCPTT_Group_A_ID |  |  |  |
| anyExt |  |  |  |  |
| emergency-alert-area-ind | false |  |  |  |