3GPP TSG-RAN5 Meeting #109	R5-256732

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1641 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RTT test case 17.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. Note 1 in main behavior table is misaligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Aligned Note 1. 2. Updated referencedTS 36.508 clause, from 4.5A.mm to 4.5A.34. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-256732 is the final version of R5-256248. |  |  |  |  |  |  |  |  |

==============First change==============

## 17.20	MT Speech / add RTT and remove RTT / EVS / Default configuration

### 17.20.1	Definition

Test to verify that the UE correctly adds and removes media text (RTT) to a mobile terminated speech session when using IMS Multimedia Telephony. This process is described in 3GPP TS 24.229 [10], clause 5.1.2A.2, TS 24.173 [65] and TS 26.114 [66].

### 17.20.2	Conformance requirement

[TS 24.229, clause 5.1.2A.2]

After the dialog is established the UE may change the dialog capabilities (e.g. add a media or request a supplementary service) if defined for the IMS communication service as identified by the ICSI value using the same dialog. Otherwise, the UE shall initiate a new initial request to the other user.

[TS 26.114, clause 6.2.5]

The SDP shall include bandwidth information for each media stream and also for the session in total. The bandwidth information for each media stream and for the session is defined by the Application Specific (AS) bandwidth modifier as defined in RFC 4566.

[TS 26.114, clause 6.3]:

During session renegotiation for adding or removing media components, the SDP offerer should continue to use the same media (m=) line(s) from the previously negotiated SDP for the media components that are not being added or removed.

[TS 26.114, clause 7.3.1]

The bandwidth for RTCP traffic shall be described using the "RS" and "RR" SDP bandwidth modifiers at media level, as specified by RFC 3556.

### 17.20.3	Test purpose

1) To verify that media RTT can be added and removed when an MT MTSI speech call is established.

2) To verify that within SIP signalling the UE performs the correct exchange of SIP header and parameter contents.

3) To verify that within SIP signalling the UE performs the correct exchange of SDP contents.

4) To verify that the UE is able to release the call.

### 17.20.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF, registered to IMS services and established an MT MTSI speech call, by executing the generic test procedure in Annex C.53 steps 1 to 13.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-9) UE executes the procedures described in TS 36.508 [94] table 4.5A.34.3-1, steps 1 to 13. In detail the following steps are done in IMS:

1) SS sends a re-INVITE request to the UE.

2) SS may receive 100 Trying from the UE.

3) SS receives 183 Session Progress from the UE.

4) SS sends PRACK to the UE to acknowledge the 183 Session Progress.

5) SS receives 200 OK for PRACK from the UE.

6) SS sends UPDATE to the UE, with SDP indicating that precondition is met on the server side.

7) SS receives 200 OK for UPDATE from the UE.

8) The UE accepts the session invite.

9) SS expects and receives 200 OK for re-INVITE from the UE.

10) SS sends ACK to the UE.

11) SS sends a re-INVITE to the UE with a SDP offer indicating that the RTT component is removed from the call.

12) SS may receive 100 Trying from the UE

13) SS expects and receives 200 OK for re-INVITE from the UE.

14) SS deactivates the EPS bearer corresponding to the RTT stream and releases the associated radio resources by applying the procedure described in TS 36.508 [94] clause 4.5A.15

15) SS sends ACK to the UE.

16-18) MT Call release according to procedure C.33

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | SS sends re-INVITE with second SDP offer to add RTT. |
| 2 |  |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response. |
| 3 |  |  | 183 Session Progress | The UE responds to re-INVITE by sending 183 response reliably with the SDP answer |
| 4 |  |  | PRACK | SS acknowledges the receipt of 183 response from the UE |
| 5 |  |  | 200 OK | The UE acknowledges the PRACK with 200 OK. |
| 6 |  |  | UPDATE | SS sends an UPDATE with SDP offer indicating SS reserved resources. |
| 7 |  |  | 200 OK | The UE acknowledges the UPDATE with 200 OK and includes SDP answer to acknowledge its current precondition status. |
| 8 |  |  |  | Make UE accept the speech and RTT offer (Note 1). |
| 9 |  |  | 200 OK | The UE responds to the re-INVITE with a 200 OK final response. |
| 10 |  |  | ACK | The SS acknowledges the receipt of 200 OK for the re-INVITE. |
| 11 |  |  | INVITE | SS sends a re-INVITE with a SDP offer indicating that the RTT component is removed from the call |
| 12 |  |  | 100 Trying | (Optional) The UE responds with a 100 Trying provisional response. |
| 13 |  |  | 200 OK | The UE responds to the re-INVITE with a 200 OK final response. |
| 14 |  |  | SS deactivates the EPS bearer for RTT |  |
| 15 |  |  | ACK | The SS acknowledges the receipt of 200 OK for the re-INVITE. |
| 16-18 |  |  | Steps defined in annex C.33 | MT Call release |
| NOTE 1: This can be done with AT command AT+CCMMD if supported by the UE or with an appropriate MMI command otherwise. |  |  |  |  |

NOTE:	The default messages contents in annex A are used with condition “IMS security“or “GIBA” when applicable

Specific Message Content

INVITE (Step 1)

Use the default message “INVITE for MT Call” in annex A.2.9 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: v=0 "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one s=- c=IN (addrtype) (connection-address for SS) b=AS:68 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP 96 b=AS: 65 b=RS: 0 b=RR: 2000 Attributes for media: a=rtpmap:96 EVS/16000/1 a=fmtp:96 br=5.9-24.4; bw=nb-swb; max-red=220 a=ptime:20 a=maxptime:240 Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv Media description: m=text (transport port) RTP/AVP 99 101 b=AS: 3 b=RS: 0 b=RR: 500 Attributes for media: a=rtpmap: 99 t140/1000 a=rtpmap: 101 red/1000 a=fmtp: 101 99/99/99 Attributes for preconditions: a=curr:qos local none a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv |

183 Session Progress (step 3)

Use the default message "183 Session Progress" in annex A.2.3 with condition A15 (183 sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 3] s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for media: a =rtpmap: (payload type) EVS/16000 [Note 2] a=fmtp:(format) Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv Media description: m=text (transport port) RTP/AVP (fmt) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for media: a=rtpmap: (payload type) t140/1000 a=rtpmap: (payload type) red/1000 a=fmtp: (format) Attributes for preconditions: a=curr:qos local none or a=curr:qos local sendrecv a=curr:qos remote none a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv a=conf:qos remote sendrecv Note 1: If there is no c= line in the session description there shall be a c= line in each media description. Note 2: The AMR channel number shall be “/1” or omitted. Note 3: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. |

PRACK (step 4)

Use the default message "PRACK" in annex A.2.4. No content body is included in this PRACK message.

200 OK (step 5)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1.

UPDATE (step 6)

Use the default message "UPDATE" in annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported option-tag | precondition |
| Message-body | The following SDP types and values. Session description: v=0 "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one s=- c=IN (addrtype) (connection-address for SS) b=AS:68 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP 96 b=AS: 65 b=RS: 0 b=RR: 2000 Attributes for media: a=rtpmap:96 EVS/16000/1 a=fmtp:96 br=5.9-24.4; bw=nb-swb; max-red=220 a=ptime:20 a=maxptime:240 Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv Media description: m=text (transport port) RTP/AVP 99 101 b=AS: 3 b=RS: 0 b=RR: 500 Attributes for media: a=rtpmap: 99 t140/1000 a=rtpmap: 101 red/1000 a=fmtp: 101 99/99/99 Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote none or curr:qos remote sendrecv [Note 1] a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv Note 1: Use the value (none/sendrecv) received from 183 Session Progress and attribute a=curr:qos local. |

200 OK for UPDATE (step 7)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | The following SDP types and values shall be present. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 3] s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for media: a=rtpmap: (payload type) EVS/16000 [Note 2] a=fmtp: (format) Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv Media description: m=text (transport port) RTP/AVP (fmt) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) b=RR: (bandwidth-value) Attributes for media: a=rtpmap: (payload type) t140/1000 a=rtpmap: (payload type) red/1000 a=fmtp: (format) Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos mandatory remote sendrecv Note 1: If there is no c= line in the session description there shall be a c= line in each media description. Note 2: The AMR channel number shall be “/1” or omitted. Note 3: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one |

200 OK for INVITE (Step 9)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog).

ACK (Step 10)

Use the default message “ACK” in annex A.2.7 with condition A5.

INVITE (step 11)

Use the default message “INVITE for MT Call” in annex A.2.9 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: v=0 "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one s=- c=IN (addrtype) (connection-address for SS) b=AS:65 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP 96 b=AS: 65 b=RS: 0 b=RR: 2000 Attributes for media: a=rtpmap:96 EVS/16000/1 a=fmtp:96 br=5.9-24.4; bw=nb-swb; max-red=220 a=ptime:20 a=maxptime:240 Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv Media description: m=text 0 RTP/AVP 99 101 b=AS: 3 b=RS: 0 b=RR: 500 Attributes for media: a=rtpmap: 99 t140/1000 a=rtpmap: 101 red/1000 a=fmtp: 101 99/99/99 Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv |

100 Trying (step 12)

Use the default message "100 Trying for INVITE" in annex A.2.2.

200 OK for INVITE (step 13)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | SDP body not checked. |

ACK (step 15)

Use the default message "ACK" in annex A.2.7 with condition A5.

BYE (step 16)

Use the default message "BYE" in annex A.2.8.

200 OK (step 17)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1.

### 17.20.5	Test requirements

The UE shall send requests and responses as described in clause 17.20.4

==============End of change==============