3GPP TSG-RAN5 Meeting #109	R5-256829

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3549 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to TC 6.2I.2a for PC2 eRedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_RedCap_UE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC 6.2I.2a needs to be updated based on the WP of NR power class 2 RedCap (Reduced Capability) UE in FR1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC 6.2I.2a was updated to include PC2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of test coverage for NR power class 2 eRedCap UE in FR1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2I.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-255818 with one revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-255818 -> R5-256829 |  |  |  |  |  |  |  |  |

==============First change==============

6.2I.2a	UE maximum output power for eRedCap

6.2I.2a.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2I.2a.2	Test applicability

This test case applies to all types of NR eRedCap UE release 18 and forward.

6.2I.2a.3	Minimum conformance requirements

For eRedcap UE, the requirements for power class 3 specified in clause 6.2.1 apply.

For eRedcap UE supporting PC2, the requirements specified for PC2 in clause 6.2.1 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2I.2.

6.2I.2a.4	Test description

6.2I.2a.4.1	Initial conditions

For eRedcap UE supporting IE eRedCapNotReducedBB-BW-r18, same initial conditions as in 6.2.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for eRedCap.

For eRedcap UE not supporting eRedCapNotReducedBB-BW-r18, same initial conditions as in 6.2.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for eRedCap.

-	The uplink RB allocation in test configuration table refers to Table 6.1I-1.

6.2I.2a.4.2	Test procedure

Same test procedure as steps 1~3 of clause 6.2.1.4.2 for UEs supporting Power Class 3.

Same test procedure as steps 4~5 of clause 6.2.1.4.2 for UEs supporting Power Class 2.

6.2I.2a.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2I.2a.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2I.2a.4.3-2: P-Max (Step 4 for PC2 UE)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 | PC2 UE will fallback to PC3 UE with P-Max=23 |  |

6.2I.2a.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2I.2a.5-1 for Power Class 3 and Power Class 2.

Table 6.2I.2a.5-1: Maximum Output Power test requirement for Power Class 3 and Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2+TT/-3-TT | 23 | ±(2+TT) |
| n2 |  |  | 26 | +2+TT/-33-TT | 23 | ±(23+TT) |
| n3 |  |  | 26 | +2+TT/-33-TT | 23 | ±(23+TT) |
| n5 |  |  | 26 | +2+TT/-33-TT | 23 | ±(2+TT) |
| n7 |  |  | 26 | +2+TT/-33-TT | 23 | ±(23+TT) |
| n8 |  |  | 26 | +2+TT/-33-TT | 23 | ±(23+TT) |
| n12 |  |  |  |  | 23 | ±(23+TT) |
| n13 |  |  | 26 | +2+TT/-3-TT | 23 | ±(2+TT) |
| n14 |  |  | 26 | +2+TT/-3-TT | 23 | ±(2+TT) |
| n20 |  |  |  |  | 23 | ±(23+TT) |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  | 26 | +2+TT/-33-TT | 23 | ±(23+TT) |
| n26 |  |  | 26 | +2+TT/-33-TT | 23 | ±(23+TT) |
| n28 |  |  | 26 | +2+TT/-33-TT | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±(2+TT) |
| n34 |  |  | 26 | +2+TT/-3-TT | 23 | ±(2+TT) |
| n38 |  |  |  |  | 23 | ±(2+TT) |
| n39 |  |  | 26 | +2+TT/-3-TT | 23 | ±(2+TT) |
| n40 |  |  | 26 | +2+TT/-3-TT | 23 | ±(2+TT) |
| n41 |  |  | 26 | +2+TT/-33-TT | 23 | ±(23+TT) |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±(2+TT) |
| n51 |  |  |  |  | 23 | ±(2+TT) |
| n53 |  |  |  |  | 23 | ±(2+TT) |
| n54 |  |  |  |  | 23 | ±(2+TT) |
| n65 |  |  |  |  | 23 | ±(2+TT) |
| n66 |  |  | 26 | +2+TT/-3-TT | 23 | ±(2+TT) |
| n70 |  |  | 26 | +2+TT/-3-TT | 23 | ±(2+TT) |
| n71 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±(2+TT) |
| n77 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| n78 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| n79 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| n85 |  |  | 26 | +2+TT/-33-TT | 23 | ±(23+TT) |
| n91 |  |  |  |  | 23 | ±(23, 5+TT) |
| n92 |  |  |  |  | 23 | ±(23, 5+TT) |
| n93 |  |  |  |  | 23 | ±(23, 5+TT) |
| n94 |  |  |  |  | 23 | ±(23, 5+TT) |
| n100 |  |  |  |  | 23 | ±(2+TT) |
| n101 |  |  |  |  | 23 | ±(2+TT) |
| n104 |  |  | 26 | +2+TT/-3-TT | 23 | ±(2+TT) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2I.1.5-2. NOTE 5: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |

Table 6.2I.2a.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

==============End of change==============