3GPP TSG-RAN WG5 Meeting #109	R5-256883

Dallas, USA, 17th -21st , Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4223 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of 6.5.12.2 message contents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The message contents of 6.5.12.2 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added message contents - Removed editor’s note. - Fixed wrong table numbers and clause numbers |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WI cannot complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<start of change>

6.5.12.2	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC

6.5.12.2.1	Test purpose

The purpose of this test is to verify that the subsequent conditional NR PSCell addition under NR-DC is within the requirements stated in clause 8.9C.2 of 38.133.

6.5.12.2.2	Test applicability

This test applies to all types of NR UE release 18 onwards.

6.5.12.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.12.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.12.2

6.5.12.2.4	Test description

6.5.12.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.12.2.4.1-1.

Table 6.5.12.2.4.1-1: Supported test configurations for Inter-frequency Subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: FDD |
| 2 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode. PSCell: FDD |
| 3 | PCell: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. PSCell: FDD |
| 4 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: TDD |
| 5 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode. PSCell: TDD |
| 6 | PCell: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. PSCell: TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.12.2.4.1-2.

Table 6.5.12.2.4.1-2: Initial conditions for DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.12.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2a | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. - Without LTE link - Without Faders |  |  |

1. The general test parameter settings are set up according to Table 6.5.12.2.4.1-3.

2. Message contents are defined in clause 6.5.12.2.4.3.

3. The test scenario comprises of three NR Cells. (Cell 1, Cell 2 and Cell 3). Cell 1, Cell 2 and Cell 3 are configured according to Annex C.1.2 and C.1.3.

Table 6.5.12.2.4.1-3: General Test Parameters for Subsequent Conditional PSCell Addition and Release

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for NR Cells for this test. |
| Initial | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell 2 | Neighbour cell on RF channel number 2. |
|  |  |  | Cell 3 | Neighbour cell on RF channel number 3. |
| Final | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Condition | Active PSCell |  | Cell 3 | PSCell on RF channel number 3 |
|  | Neighbour Cell |  | Cell 2 | Neighbour cell on RF channel number 2 |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -118 | A4 event is used to trigger conditional PSCell addition of Cell 2. |
|  | Time to Trigger | s | 0 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured during T1, T2, T3 and T4. |
| PRACH configuration on cell2 |  |  | FR1 PRACH configuration 1 | Captured in clause A.3.8.2.1 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell 2. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell 3. |
| T1 |  | s | 1 | During this time the PCell is known, PSCell 1 and PSCell 2 are unknown. |
| T2 |  | s | 7 | During this time PSCell 1 meets the PSCell addition condition and UE adds this PSCell. |
| T3 |  | s | 1 | During this time the UE releases the PSCell 1. |
| T4 |  | s | 7 | During this time PSCell 2 meets the addition condition and UE adds this PSCell. |

6.5.12.2.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS transmits a SN RRCReconfiguration message to the UE to configure Cell 2 and Cell 3 as SCells according to Table 6.5.12.2.4.5-1.

3.	The UE shall transmit an RRCReconfigurationComplete message.

4.	The SS activates Cell 2 and Cell 3

5.	Set the parameters according to T1 in Table 6.5.12.2.4.4.1-3 and Table 6.5.12.2.4.5-1. Propagation conditions are set according to Annex C clauses C.2.2. T1 starts.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.5.12.2.5-1. T2 starts.

7. When T2 expires, the SS shall switch the power settings at T2 to power setting at T3 as per Tables 6.5.12.2.5-1 and 6.5.12.2.5-2. T4 starts.

8.	If

a)	The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_DU + 2 ms = 1040+10+62 ms=1112 ms from the start of T2.

b)	The UE shall start to transmit the PRACH to Cell 3 less than TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_DU + 2 ms = 0+1040+10+62 ms=1112 ms from the start of T4.

and

c)	no longer than X consecutive ACK/NACK DTXs are observed by the SS from the start of T2 to the instant the UE transmits the first PRACH preamble, where

-	X = Tinterrupt +k1 for test configuration 1 in Table 6.5.12.2.4.1-1

-	X = Tinterrupt for test configuration 2 in Table 6.5.12.2.4.1-1

-	X = 2∙Tinterrupt for test configuration 3 in Table 6.5.12.2.4.1-1.

9.	When T4 expires the UE shall be capable to transmit PRACH preamble towards the Cell3.

10.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

11.	Repeat steps 2-10 until the confidence is achieved.

6.5.12.2.4.3	Message contents

Table 6.5.12.2.4.3-1: RRCReconfiguration (Step 4)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS and CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig | Table 6.5.12.2.4.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| conditionalReconfiguration-r16 |  | ConditionalReconfiguration | Table 6.5.12.2.4.3-6 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.12.2.4.3-2: MeasConfig (Table 6.5.12.2.4.3-1)

| Derivation Path: Table H.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod { | 2 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 | Table 6.5.12.2.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f2 | Table 6.5.12.2.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entry |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR for PSCell | Table 6.5.12.2.4.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig specified in Table H.3.1-6 with condition gapUE and Pattern #0 |  |  |
| quantityConfig | QuantityConfig specified in Table H.3.1-5 |  |  |
| } |  |  |  |

Table 6.5.12.2.4.3-3: MeasObjectNR-f1 (Table 6.5.12.2.4.3-2)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for Cell 1 |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1[14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |

Table 6.5.12.2.4.3-4: MeasObjectNR-f2 (Table 6.5.12.2.4.3-2)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for Cell 2 |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1[14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |

Table 6.5.12.2.4.3-5: ReportConfigNR for PSCell (Table 6.5.12.2.4.3-2)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-142 with condition CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig SEQUENCE { |  |  | CPA |
| condEventId CHOICE { |  |  |  |
| condEventA4 SEQUENCE { |  |  |  |
| A4-Offset CHOICE { |  |  |  |
| rsrp | -118 | actual value = -118*0.5 = -59dB |  |
| } |  |  |  |
| hysteresis | 0 | actual value = 0*0.5 = 0dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.12.2.4.3-6: ConditionalReconfiguration (Table 6.5.12.2.4.3-1)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| condReconfigToAddModList-r16 SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 2 entry |  |  |
| CondReconfigToAddMod-r16 [1] SEQUENCE { |  | entry 2 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16 SEQUENCE (SIZE (1..2)) OF MeasId { | 1 entry |  |  |
| MeasId[1] | 1 | The MeasId configured in Table 6.5.12.2.4.3-2 |  |
| } |  |  |  |
| condExecutionCondPSCell-r18 SEQUENCE (SIZE (1..2)) OF MeasId { | 1 entry |  |  |
| MeasId[2] | 1 | The MeasId configured in Table 6.5.12.2.4.3-2 |  |
| } |  |  |  |
| condRRCReconfig-r16 | OCTET STRING |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.5.12.2.5	Test requirements

Table 6.5.12.2.5-1: Cell Specific Parameters for Subsequent Conditional PSCell Addition and Release

| Parameter | Unit | Config | Cell 2 |  |  |  |  | Cell 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 | T3 | T4 | T1 | T2 |  |  | T3 |  | T4 |
| NR RF Channel Number |  | 1,2,3,4,5,6 | 2 |  |  |  |  | 3 |  |  |  |  |  |  |
| TDD |  | 1,4 | Not Applicable |  |  |  |  | Not Applicable |  |  |  |  |  |  |
| configuration |  | 2,5 | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | 3,6 | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel | MHz | 1,4 | 10: NPRB,c = 52 |  |  |  |  | 10: NPRB,c = 52 |  |  |  |  |  |  |
|  |  | 2,5 | 10: NPRB,c = 52 |  |  |  |  | 10: NPRB,c = 52 |  |  |  |  |  |  |
|  |  | 3,6 | 40: NPRB,c = 106 |  |  |  |  | 40: NPRB,c = 106 |  |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |  |
| PDSCH Reference |  | 1,4 | SR.1.1 FDD |  |  |  |  | SR.1.1 FDD |  |  |  |  |  |  |
| measurement |  | 2,5 | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  |  |  |  |
| channel |  | 3,6 | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Reference |  | 1,4 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |  |  |  |  |
| Channel |  | 2,5 | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | 3,6 | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  |  |  |  |
| Dedicated CORESET Reference |  | 1,4 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |  |  |  |  |
| Channel |  | 2,5 | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |  |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  | OP.1 |  |  |  |  |  |  |
| SSB configuration |  | 1,2,4,5 | SSB.1 FR1 |  |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | 3,6 | SSB.2 FR1 |  |  |  |  | SSB.2 FR1 |  |  |  |  |  |  |
| SMTC configuration |  | 1,2,4,5 | SMTC.1 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |
|  |  | 3,6 | SMTC.1 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | 2,5 | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | 3,6 | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | 1,4 | CSI-RS.1.1 FDD |  |  |  |  | CSI-RS.1.1 FDD |  |  |  |  |  |  |
|  |  | 2,5 | CSI-RS.1.1 TDD |  |  |  |  | CSI-RS.1.1 TDD |  |  |  |  |  |  |
|  |  | 3,6 | CSI-RS.2.1 TDD |  |  |  |  | CSI-RS.2.1 TDD |  |  |  |  |  |  |
| reportConfigType |  | 1,2,3,4,5,6 | Periodic |  |  |  |  | periodic |  |  |  |  |  |  |
| reportQuantity |  | 1,2,3,4,5,6 | cri-RI-PMI-CQI |  |  |  |  | cri-RI-PMI-CQI |  |  |  |  |  |  |
| CSI reporting periodicity | slot | 1,2,4,5 | 5 |  |  |  |  | 5 |  |  |  |  |  |  |
|  |  | 3,6 | 10 |  |  |  |  | 10 |  |  |  |  |  |  |
| CSI reporting offset | slot | 1,2,4,5 | 2 |  |  |  |  | 2 |  |  |  |  |  |  |
|  |  | 3,6 | 4 |  |  |  |  | 4 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2,3,4,5,6 | 0 |  |  |  |  | 0 |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4,5,6 | -85 |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1,2,4,5 | -85 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3,6 | -82 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1,2,3,4,5,6 | -infinity | -1 |  | -infinity | -infinity | -infinity |  | -infinity | -infinity |  | -1 |  |
|  |  | 1,2,3,4,5,6 | -infinity | -1 |  | -infinity | -infinity | -infinity |  | -infinity | -infinity |  | -1 |  |
| SS-RSRPNote3 | dBm/SCS | 1,2,4,5 | -infinity | -86 |  | -infinity | -infinity | -infinity |  | -infinity | -infinity |  | -86 |  |
|  |  | 3,6 | -infinity | -83 |  | -infinity | -infinity | -infinity |  | -infinity | -infinity |  | -83 |  |
| IoNote3 | dBm/9.36 MHz | 1,2,4,5 | -57 | -55 |  | -57 | -57 | -57 |  | -57 | -57 |  | -55 |  |
|  | dBm/38.1 MHz | 3,6 | -57 | -49 |  | -57 | -57 | -57 |  | -57 | -57 |  | -49 |  |
| Propagation condition |  | 1,2,3,4,5,6 | AWGN |  |  |  |  | AWGN |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

TRRC_delay + TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_DU + 2 ms = 1040+10+62 ms=1112 ms from the start of T2.

The UE shall start to transmit the PRACH to Cell 3 less than TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_DU + 2 ms = 0+1040+10+62 ms=1112 ms from the start of T4.

All of the above test requirements shall be fulfilled in order for the observed conditional PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90 %.

<end of change>