3GPP TSG-RAN5 Meeting #109	R5-256969

# Dallas, USA, 17th – 21st November, 2025

Agenda item:	5.4.20

Source:	Ericsson

Title:	Discussion on Channel raster enhancement for RedCap Rel-17

Document for:	Discussion and Endorsement

# Introduction

The contribution seeks clarification on how to incorporate the mandatory feature of Channel raster enhancement for RedCap UE.

# Discussion

## Background

A new Work Item has been introduced in RAN5 for Channel raster enhancement for Rel-18 [1]. While this Work Item addresses the feature starting from Release 18 onward, clause 5.4I of 3GPP TS 38.101 Release 17 [2], specifies that mandatory support for Channel Raster Enhancement begins with Release 17 for RedCap UE introduced by RAN4 CR [7] implemented under TEI-17. This requirement is also reflected in clause 4.2.21.1 of 3GPP TS 38.306 Release 17 [3].

## Capability implementation of new Work Item for Rel-18

The new Rel-18 Work Item has not yet implemented the UE capability information in 38.508-2 [4], but it is assumed that it will be similar to below example item xxx:

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| .. | .. | .. | .. | .. |  |  |  |
| xxx | Support enhanced channel raster | 38.306, 4.2.7.2 | Rel-18 | pc_enhancedChannelRaster-r18 | CY |  | Mandatory for (e)RedCap UEs and optional for non-(e)RedCap UEs |

## Handling Rel-17 support

As stated in the background section (subclause 2.1), the Release 17 support for Channel Raster Enhancement is not currently included in the scope of the new Work Item, which targets Release 18. However, could it be considered for inclusion in this Work Item to also cover the Release 17 aspects of Channel Raster Enhancement?

There is no difference in the requirements between Release 17 and Release 18, meaning the same specifications apply to both RedCap in Release 17 and in Release 18 and onwards. This is confirmed in 3GPP TS 38.101 [5], subclause 5.4.2, Table 5.4.2.3-5.

Proposal 1. Handle Rel-17 aspects under maintenance work item for channel raster enhancements. An WorkPlan [6] created to handle the TEl-17 aspects.

## Implementation of Rel-17 support

It’s stated mandatory support of channel raster enhancement in 38.101 Rel-17 [2]  for RedCap, proposal is to add the Rel-17 support for RedCap like bellow example in 38.508-2:

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| .. | .. | .. | .. | .. |  |  |  |
| xxx | Support enhanced channel raster | 38.306, 4.2.7.2 | Rel-18 | pc_enhancedChannelRaster-r18 | CY |  | Mandatory for (e)RedCap UEs and optional for non-(e)RedCap UEs |
|  |  | 38.306 4.2.21.1 | Rel-17 |  | Yes |  | Mandatory for RedCap UEs |

Proposal 2. To add Mandatory Rel-17 UE support for RedCap can above example table be endorsed.

#### Test case implementation

The intention is to verify the functionality of channel raster enhancements for RedCap Release 17, for example, by adding an additional step in test case TS 38.521-1, clause 7.3I.2. An workplan [6] attached to this discussion paper describe the changes needed to add Rel-17 updates.

# Conclusion

These proposals were made in this contribution.

Seeking endorsement for Proposal 1 and Proposal 2.

Proposal 1. Handle Rel-17 aspects under maintenance work item for channel raster enhancements. An WorkPlan should be created to handle the REl-17 aspects.

Proposal 2. To add Mandatory Rel-17 UE support for RedCap can above example table be endorsed.

# References

- R5-253960, “New WID on UE Conformance - Channel raster enhancement”, China Telecom, Huawei, RAN5#108

- TS 38.101, “User Equipment (UE); Radio transmission and reception; Part 1: Range 1 Standalone”, v17.19.0

- TS 38.306, “User Equipment (UE) radio access capabilities”, v17.14.0

- TS 38.508-2, “Common Implementation Conformance Statement (ICS)”, v19.1.0

- TS 38.101 “User Equipment (UE); Radio transmission and reception; Part 1: Range 1 Standalone”, v19.2.0

- WorkPlan for enhanced channel raster Rel_17, included in zip for this discussion paper.

- R4-2414434, “(NR_redcap-Core) Correction of the channel raster for RedCap UEs by added entries”, Ericsson RAN4#112