# Title:		Test Tolerances for NR SA FR1 Selection / Reselection of relay UE for U2N case

Source:	CATT

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for Selection / Reselection of relay UE tests. The test cases in question, from TS 38.533, are:

- 9.1.7.1 NR SA FR1 selection / reselection of relay UE for U2N case

# 2. 	Test case in TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v17.19.0:

A.9.1.7	Selection / Reselection of relay UE

A.9.1.7.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to selection / reselection of relay UE defined in clauses 12.10. In the test the UE under test is configured with PCell and is configured with resource pools for NR sidelink discovery message as required for remote UE operation.

This test is applicable to UEs capable of NR sidelink communication and sidelink discovery, and further support the optional feature of sidelink remote UE operation.

The test parameters are given in Table A.9.1.71.1-1, Table A.9.1.7.1-2, Table A.9.1.7.1-3, Table A.9.1.7.1-4, and Table A.9.1.7.1-5 below. The test consists of one active serving cell (cell 1), one remote UE and two active Sidelink relay UEs (Sidelink Relay UE 1, Sidelink Relay UE 2). The relay UEs are configured in mode 2 to be transmitting relay discovery messages every discovery period, which is determined by resource reservation period indicated by sl-ResourceReservePeriodList.

The test system shall ensure that the remote UE under test has transmitted SidelinkUEInformationNR message and has been configured with the sidelink discovery resource pool and sidelink communication resource pool respectively for relay operation prior to the start of the test.

The tests consist of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively.

During T1, RSRP of cell 1 is kept higher than threshHighRemote (within sl-remoteUE-Config), and the remote UE is not required to perform relay UE selection.

During T2, RSRP of cell 1 is configured to be lower than threshHighRemote. The UE is expected to start looking for relay UE. The test system shall ensure that the UE under test has been configured the resource pool prior to end of T2 duration. During T2, the SD-RSRP of Sidelink Relay UE 1 and Sidelink Relay UE 2 is configured to be lower than the detection threshold and no relay UE will be available for the remote UE under test.

During T3, the SD-RSRP of Sidelink Relay UE 1 is raised above the threshold sl-RSRP-Thresh and the UE is expected to perform relay selection to Sidelink Relay UE 1. The test system can determine that the remote UE has selected a relay by monitoring the configured sidelink communication resource for the Prose direct link establishment request message to the relay UE.

During T4, the UE is expected to complete the sidelink connection establishment with the relay UE. Note that the RSRP of the serving cell (cell 1) and the SD-RSRP of sidelink relay UEs is kept unchanged during T3 and T4. The period T4 ends when Sidelink Relay UE1 sends the sidelink communication response message back to the remote UE.

During T5, SD-RSRP of Sidelink Relay UEs are modified such that the remote UE is expected to reselect to Sidelink Relay UE2.

Table A.9.1.7.1-0: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.9.1.7.1-1: Test parameters for selection / reselection of relay UE test for NR FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS | kHz | 30 |  |
| Active cell |  | Cell 1 | Serving NR cell will broadcasting SIB12 |
| Active Relay UEs |  | Relay UE 1, Relay UE 2 | Transmitting relay discovery message |
| CP length of Cell 1 |  | Normal |  |
| T1 | ms | 100 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to 1 sec if UE does not transmit SidelinkUEInformationNR during this period. |  |
| T3 | s | 1 |  |
| T4 | s | 2 |  |
| T5 | s | 1 |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |

Table A.9.1.7.1-2: Sidelink discovery configuration for selection / reselection of relay UE test for NR FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Resource pool configuration for sidelink discovery |  | As specified in Table A.3.21.2-2 Note 2 | IE values unless specified otherwise in this test. |
| sl-TimeResource included in sl-DiscTxPoolSelected-r17 and sl-DiscRxPool-r17 |  | 0000111100 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
| sl-NumSubchannel included in sl-DiscTxPoolSelected-r17 and sl-DiscRxPool-r17 |  | 2 | Indicates the number of sub-channels for TX resource pool |
| sl-SubchannelSize included in sl-DiscTxPoolSelected-r17 and sl-DiscRxPool-r17 |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| sl-ResourceReservePeriodList | s | 0.04 | Indicates the resource reservation period for discovery period |
| threshHighRemote within sl-remoteUE-Config |  | -91dBm |  |
| sl-RSRP-Thresh within sl-remoteUE-Config |  | -91dBm |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: This test is according to the principle defined in section A.3.21. |  |  |  |

Table A.9.1.7.1-3: Sidelink Communication configuration for selection / reselection of relay UE test

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | - | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Sidelink Communication configuration |  | - | As specified in section A.3.21.2 | IE values unless specified otherwise in this test. |
| sl-TimeResource |  | - | 0000000011 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
| Number of Active Sidelink Relay UEs |  | - | 2 | Active Sidelink UE i = 0, 1 |
| Active Sidelink Relay UEs (UE i = 0, 1) | PSCCH Reference Measurement Channel | - | CC.1A HD | As specified in Table A.3.21.3-1 |
|  | PSSCH Reference Measurement Channel | - | CD.1A HD | As specified in Table A.3.21.3-2 |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool | - | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize-r16 included in SL-ResourcePool |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
|  | sl-StartRB-Subchannel-r16 included in SL-ResourcePool | - | floor(i)x10 | Indicates the lowest RB index of the subchannel with the lowest index for Sidelink Relay UE i = 0 starts RB=0; Sidelink Relay UE i = 1 starts RB=10. |
|  | PSBCH-RSRP | dBm/30kHz | -95 |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

Table A.9.1.7.1-4: Sidelink Relay UE specific test parameters for selection / reselection of relay UE test for NR FR1

| Parameter | Unit | Relay UE 1 |  |  |  |  | Relay UE 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 (HD carrier in Band n47 or n38) |  |  |  |  |  |  |  |  |  |
| BWchannel Note 4 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |  |  |  |  |  |  |  |  |
| SCS | kHz | 30 |  |  |  |  |  |  |  |  |  |
| Sidelink Discovery resource pool configuration |  | As specified in Table A.9.1.7.1-2 |  |  |  |  |  |  |  |  |  |
| Transmission frequency |  | Every discovery period (40ms) |  |  |  |  |  |  |  |  |  |
| Resource allocation |  | Non-overlapping PRBs |  |  |  |  |  |  |  |  |  |
| Note1 | dBm/30 kHz | -97 |  |  |  |  |  |  |  |  |  |
|  | dB | -inf | -inf | 10.5 | 10.5 | -1.5 | -inf | -inf | -inf | -inf | 10.5 |
| SD-RSRP / SL-RSRP Note2, Note 3 | dBm/30 kHz | -inf | -inf | -86.5 | -86.5 | -98.5 | -inf | -inf | -inf | -inf | -86.5 |
| Antenna Configuration | - | 1x2 |  |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SD-RSRP / SL-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: PSCCH-DMRS Es/Iot is set the same as PSSCH-DMRS Es/Iot. Note 4: This test is according to the principle defined in section A.3.21.2. |  |  |  |  |  |  |  |  |  |  |  |

Table A.9.1.7.1-5: Cell specific test parameters for selection / reselection of relay UE test for NR FR1

| Parameter |  | Unit | Cell1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  | 2 |  |  |  |  |
| Serving/Non-serving |  |  | Serving |  |  |  |  |
| Duplex Mode | Config 1 |  | FDD |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| Channel Bandwidth (BWchannel) | Config 1,2 |  | 10: NRB,c = 52 |  |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |
| DRX Cycle |  |  | N/A |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| SMTC Configuration |  |  | SMTC.2 |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  |
| drx-Configuration |  |  | N/A |  |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  |  | R.11 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  |  | OP.16 FDD |  |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -97 |  |  |  |  |
| Note2 | Config 1,2, | dBm/SCS | -97 |  |  |  |  |
|  | Config 3 |  | -94 |  |  |  |  |
|  |  | dB | 10.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| RSRP Note3 |  | dBm/SCS | -86.5 | -95.5 | -95.5 | -95.5 | -95.5 |
|  |  |  | -83.5 | -92.5 | -92.5 | -92.5 | -92.5 |
| Propagation condition |  |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The test requirements in TS 38.133 A.9.1.7.1 refer back to the core requirements in clause 12.10.2.

12.10.2	Selection / Reselection of relay UE

For a remote UE configured by upper layer for relay operation, the remote UE shall search for candidate relay UEs for selection and/or reselection every discovery period which is determined by resource reservation period or SPS transmission periodicity configured by network.

If the remote UE has a selected sidelink relay UE, then the remote UE shall measure the SD-RSRP or SL-RSRP of the selected relay once in every four discovery periods and evaluate if it meets the relay selection criterion as defined in TS 38.331[2] (clause 5.8.15.3).

The remote UE shall measure SD-RSRP or SL-RSRP of the candidate relay UEs every Tmeasure, SL_Relay_Intra for relay UEs that are detected and measured according to the measurement rules.

For intra-frequency relay UEs that are detected, but that has not been selected or reselected to, the remote UE shall be capable of evaluating that the intra-frequency relay UE has met selection or reselection criterion defined in TS 38.331[2] (clause 5.8.15.3) within Tevaluate, SL_Relay_Intra as specified in table 12.10.2-1.

The minimum requirements are required to meet when the selected and candidate relay UEs are transmitting relay discovery message every discovery period.

Table 12.10.2-1: Tmeasure, SL_Relay_Intra and Tevaluate, SL_Relay_Intra

| Discovery Period [s] | Tmeasure,SL_Relay_Intra [s] (number of discovery periods) | Tevaluate, SL_Relay_Intra [s] (number of discovery periods) |
| --- | --- | --- |
| 0.04≤Discovery period≤10.24 | Note 1 (4) | Note 1 (16) |
| Note 1: Time depends upon the discovery period which is resource reservation period (in mode 2) or SPS transmission periodicity (in mode 1). Note 2: SL-RSRP or SD-RSRP can be derived from PSCCH-DMRS and/or PSSCH-DMRS. |  |  |

# 3. 	Discussion

The test consists of one active serving cell (cell 1), one remote UE and two active Sidelink relay UEs (Sidelink Relay UE 1, Sidelink Relay UE 2). The tests consist of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively..

During T1, RSRP of cell 1 is kept higher than threshHighRemote and the remote UE is not required to perform relay UE selection.

During T2, RSRP of cell 1 is configured to be lower than threshHighRemote. The UE is expected to start looking for relay UE.

During T3, the SD-RSRP of Sidelink Relay UE 1 is raised above the threshold sl-RSRP-Thresh and the UE is expected to perform relay selection to Sidelink Relay UE 1.

During T4, the UE is expected to complete the sidelink connection establishment with the Sidelink Relay UE 1.

During T5, SD-RSRP of Sidelink Relay UEs are modified such that the remote UE is expected to reselect to Sidelink Relay UE2.

# 4. 	Calculation of Test Tolerances

Two test configurations are given in test case A.9.1.7.1 (20 MHz CBW and 40 MHz CBW). Analysis are given in this clause are based on the test configuration of 40 MHz CBW.

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from TS 38.133 Table A.9.1.7.1-0, Table A.9.1.7.1-1, Table A.9.1.7.1-2, Table A.9.1.7.1-3, Table A.9.1.7.1-4 and Table A.9.1.7.1-5. The key parameters are selected as the minimum set to define the cell power levels and the sidelink relay UE power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In this test case, in order to verify the requirements related to selection / reselection of relay UE ,the following conditions for  power levels for Cell 1, SL relay UE 1 and SL relay UE 2 need to be satisfied.

During T1, Cell 1 SS-RSRP > threshHighRemote;

During T2, Cell 1 SS-RSRP < threshHighRemote ;

During T3, Sidelink Relay UE 1 SD-RSRP > sl-RSRP-Thresh;

During T4, Sidelink Relay UE 1 SD-RSRP > sl-RSRP-Thresh;

During T5, Sidelink Relay UE 1 SD-RSRP < sl-RSRP-Thresh, Sidelink Relay UE 2 SD-RSRP > sl-RSRP-Thresh;

Some further steps are done to check the selection/ reselection power level conditions:

- (Cell 1 SS-RSRP-threshHighRemote)

- (SL relay UE 1 SD-RSRP-sl-RSRP-Thresh)

- (SL relay UE 2 SD-RSRP-sl-RSRP-Thresh)

c) Uncertainties

The SS provides 2 AWGN, NR cell 1, 2 Sidelink relay UEs to the remote UE. We propose to control the following parameters:

- AWGN#1 Absolute power averaged over BWConfig, Noc ±1.5 dB

- AWGN#1 Absolute power averaged over measurement PRB, Noc ±1.5 dB

- AWGN#2 Absolute power averaged over BWConfig, Noc ±1.5 dB

- AWGN#2 Absolute power averaged over measurement PRB, Noc ±1.5 dB

- Ratio of NR cell 1 signal to noise averaged over BWConfig, Ês1 / Noc ±0.3 dB

- Ratio of NR cell 1 signal to noise averaged over measurement PRB, Ês1 / Noc ±0.3 dB

- Ratio of Sidelink relay UE 1 signal to noise averaged over BWConfig, Ês2 / Noc ±0.3 dB

- Ratio of Sidelink relay UE 1 signal to noise averaged over measurement PRB, Ês2 / Noc ±0.3 dB

- Ratio of Sidelink relay UE 2 signal to noise averaged over BWConfig, Ês3 / Noc ±0.3 dB

- Ratio of Sidelink relay UE 2 signal to noise averaged over measurement PRB, Ês3 / Noc ±0.3 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition, the SS-RSRP absolute measurement accuracy needs to be considered in T1 and T2. The UE measurement accuracy is defined in the following extract from TS 38.133:

10.1.2.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 is configured.

The accuracy requirements in Table 10.1.2.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 4.5 | 9 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_B, NR_FDD_FR1_C, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_SDL_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

Based on the Io calculated in step b), Io > -70dBm/Ch BW, Io < -50dBm/Ch BW, the UE measure SS-RSRP absolute accuracy is 8 dB in normal condition.

To ensure SS-RSRP absolute measurement accuracy, the conditions of S-SSB_RP and S-SSB Es/Iot in Annex B.2.2 need to be also fulfilled.

B.2.2	Conditions for NR intra-frequency measurements

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

In addition, the SD-RSRP absolute measurement accuracy needs to be considered in T3, T4 and T5. The UE measurement accuracy is defined in the following extract from TS 38.133:

10.4.5.1	Absolute Discovery Signal Measurement Accuracy

The requirements for absolute accuracy of discovery signal measurement in this clause apply to a sidelink UE performing SD-RSRP measurements for direct to indirect path swich or SL-RSRP measurements for indirect to direct path switch on the same frequency as used by the sidelink relay UE transmitting the relay Discovery message.

The accuracy requirements in Table 10.4.5.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSCCH and/or PSSCH are transmitted from one port.

-	Conditions defined in clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSCCH-RSRP|dBm and/or PSSCH-RSRP|dBm according to Annex B.4.4 for a corresponding Band are fulfilled.

Table 10.4.5.1-1: Intra-frequency discovery signal measurement absolute accuracy for a remote sidelink UE [2] capable of sidelink Communication and sidelink Discovery and configured by upper layers for relay operation.

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCS = 15kHz | SCS = 30kHz | SCS = 60kHz |  |  |
| 4.5 | 9 | 0 dB | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 | N/A | -70 |
| 8 | 11 | 0 dB | NR_TDD_FR1_B NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR sidelink operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. |  |  |  |  |  |  |  |  |

Based on the Io calculated in step b), Io > -70dBm/Ch BW, Io < -50dBm/Ch BW, the UE measure SD-RSRP absolute accuracy is 8 dB in normal condition.

To ensure SD-RSRP absolute measurement accuracy, the conditions of SL-RSRP and Es/Iot in Annex B.4.4 need to be also fulfilled.

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements

This clause defines the following condtions for L1 SL-RSRP measurement accuracy requirements applicable for a corresponding operating band.

The conditions are defined in Table B.4.4-1 for FR1.

Table B.4.4-1: Conditions for L1 SL-RSRP measurements in FR1

| Parameter | NR V2X operating band groups Note1 | Minimum L1 SL-RSRP |  |  | Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/SCS |  |  | dB |
|  |  | SCS= 15kHz | SCS= 30kHz | SCS = 60kHz |  |
|  | NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 | 0 |
|  | NR_TDD_FR1_J | -116.5 | -113.5 | -110.5 |  |
| NOTE 1: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 2: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS and/or PSSCH-DMRS. NOTE 3: The SCS is for PSCCH and/or PSSCH. |  |  |  |  |  |

In summary,

- NR Cell 1 Es/Iot in T3, T4 and T5  shall be no less than -6 dB;

- NR Cell1 S-SSB_RP in T3 shall be no less than -123.5 dBm/30kHz;

- SL Relay UE 1 Es/Iot in T3 and T4 shall be no less than 0 dB;

- SL Relay UE 2 Es/Iot in T5 shall be no less than 0 dB;

- SL Relay UE 1 SD-RSRP in T3, T4 and T5  shall be no less than -113.5 dBm/30kHz;

- SL Relay UE 2 SD-RSRP in T5  shall be no less than -113.5 dBm/30kHz;

- Io in PC5 Ch BWshall be no less than -70dBm/30kHz and no bigger than -50dBm/Ch BW;

- Io in Uu Ch BWshall be no less than -70dBm/30kHz and no bigger than -50dBm/Ch BW;

- measured (Cell 1 SS-RSRP-threshHighRemote) in T1 shall be bigger than 0dB.

- measured (Cell 1 SS-RSRP-threshHighRemote) in T2 shall be less than 0dB.

- measured (SL relay UE 1 SD-RSRP-sl-RSRP-Thresh) in T3 and T4 shall be bigger than 0dB.

- measured (SL relay UE 1 SD-RSRP-sl-RSRP-Thresh) in T5 shall be less than 0dB.

measured (SL relay UE 2 SD-RSRP-sl-RSRP-Thresh) in T5 shall be bigger than 0dB.

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133.

Having identified the parameters which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB.

Denoting critical parameter y as a function of test equipment uncertainties x1, x2, ..., i.e. y=fx1,x2,⋯. Then sensitivity factors of xi on y can be expressed as:

sensitivity factors of xi on y=∂y∂xi

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE measurement accuracies where relevant. This process follows the superposition principle.

For the test system uncertainties the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE measurement accuracies are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE measurement accuracies. If all the uncertainties were combined root-sum square, the resulting smaller test limits could case a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

During T1, the SS-RSRP measurement uncertainties take (Cell 1 SS-RSRP-threshHighRemote) outside the allowed ranges.

We observe that for the test configuration that the test equipment uncertainties could take the UE outside the allowed range, so in T1 the Es/Noc of Cell 1 is therefore increased by an amount sufficient to achieve (Cell 1 SS-RSRP-threshHighRemote)>0dB.

During T3 and T4, the SD-RSRP measurement uncertainties take (SL relay UE 1 SD-RSRP-sl-RSRP-Thresh) outside the allowed ranges.

We observe that for the test configuration that the test equipment uncertainties could take the UE outside the allowed range, so in T3 and T4 the Es/Noc of Sidelnk relay UE 1 is therefore increased by an amount sufficient to achieve (SL relay UE 1 SD-RSRP-sl-RSRP-Thresh)>0dB.

During T5, the SD-RSRP measurement uncertainties take (SL relay UE 2 SD-RSRP-sl-RSRP-Thresh) outside the allowed ranges.

We observe that for the test configuration that the test equipment uncertainties could take the UE outside the allowed range, so in T5 the Es/Noc of Sidelnk relay UE 2 is therefore increased by an amount sufficient to achieve (SL relay UE 2 SD-RSRP-sl-RSRP-Thresh)>0dB.

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances.

In T1, the SSB Es/Noc of Cell 1 is increased by an amount sufficient to achieve (Cell 1 SS-RSRP-threshHigh Remote) >= 0dB. The actual offset required is +3.7dB for cell 1.

In T3 and T4, the Sidelnk Es/Noc of Sidelink relay UE 1 is increased by an amount sufficient to achieve (SL relay UE 1 SD-RSRP-sl-RSRP-Thresh) >= 0dB. The actual offset required is +3.7dB for Sidelink relay UE 1.

In T5, the Sidelnk Es/Noc of Sidelink relay UE 2 is increased by an amount sufficient to achieve (SL relay UE 2 SD-RSRP-sl-RSRP-Thresh) >= 0dB. The actual offset required is +3.7dB for Sidelink relay UE 2.

These values are found empirically by observing the minimum/maximum parameter values in step g).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements. It is not necessary to calculate all parameters, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5.	Calculations for Sidelink relay UE selection / reselction delay for U2N case

The minimum requirements are required to meet when the selected and candidate relay UEs are transmitting relay discovery message every discovery period as specified in TS 38.133 Table 12.10.2-1:

Table 12.10.2-1: Tmeasure, SL_Relay_Intra and Tevaluate, SL_Relay_Intra

| Discovery Period [s] | Tmeasure,SL_Relay_Intra [s] (number of discovery periods) | Tevaluate, SL_Relay_Intra [s] (number of discovery periods) |
| --- | --- | --- |
| 0.04≤Discovery period≤10.24 | Note 1 (4) | Note 1 (16) |
| Note 1: Time depends upon the discovery period which is resource reservation period (in mode 2) or SPS transmission periodicity (in mode 1). Note 2: SL-RSRP or SD-RSRP can be derived from PSCCH-DMRS and/or PSSCH-DMRS. |  |  |

The sidelink relay UE selection delay = (Tevaluate, SL_Relay_Intra + 40ms).

The sidelink relay UE reselection delay = (Tmeasure, SL_Relay_Intra + Tevaluate, SL_Relay_Intra + 40ms).

The relay UEs are configured in mode 2 to be transmitting relay discovery messages every discovery period, which is determined by resource reservation period indicated by sl-ResourceReservePeriodList. And in test case the discovery period is configured as 40ms

Table A.9.1.7.1-2: Sidelink discovery configuration for selection / reselection of relay UE test for NR FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Resource pool configuration for sidelink discovery |  | As specified in Table A.3.21.2-2 Note 2 | IE values unless specified otherwise in this test. |
| sl-TimeResource included in sl-DiscTxPoolSelected-r17 and sl-DiscRxPool-r17 |  | 0000111100 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
| sl-NumSubchannel included in sl-DiscTxPoolSelected-r17 and sl-DiscRxPool-r17 |  | 2 | Indicates the number of sub-channels for TX resource pool |
| sl-SubchannelSize included in sl-DiscTxPoolSelected-r17 and sl-DiscRxPool-r17 |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| sl-ResourceReservePeriodList | s | 0.04 | Indicates the resource reservation period for discovery period |
| threshHighRemote within sl-remoteUE-Config |  | -91dBm |  |
| sl-RSRP-Thresh within sl-remoteUE-Config |  | -91dBm |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: This test is according to the principle defined in section A.3.21. |  |  |  |

The SL relay UE selection delay = (Tevaluate, SL_Relay_Intra + Every discovery period)

= 16* T discovery period+40ms

= 16*40+40

= 680ms

The SL relay UE reselection delay = (Tmeasure, SL_Relay_Intra + Tevaluate, SL_Relay_Intra + Every discovery period)

= 4* T discovery period + 16* T discovery period+40ms

= 20*40+40

= 840ms

# 6. Treatment of other configurations within the same test case

TC 9.1.7.1 contains configuration with different PC5 channel bandwidths. The Io of 20MHz CBW must be less than that of 40MHz CBW. Therefore, as long as the max Io of PC5 40MHz CBW is guaranteed to be fine, the max Io of PC5 20MHz CBW will definitely not be a problem.

TC 9.1.7.1 also contains configurations with different Uu SCS and Uu/SCS channel bandwidths. Analysis given in clause 4 also applies to test configuration 9.1.7.1-3.

And for test configuration 9.1.7.1-3, as the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Uu Channel bandwidth and SCS, section a)

- Noc on NR RF chanel 2, section a)

- Uu RBs in Channel bandwidth, section a) and associated comment

- SS-RSRP minimum requirement, section g)