3GPP TSG-RAN WG5 Meeting #109	R5-256054r1

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4203 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to editor notes for ATG RRM TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editors notes for ATG RRM TCs need to be updated since UE location in test environment has been added. Also initial test environment condition general description is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Editors notes for ATG RRM TCs have been updated. The related TCs are listed in the affected clauses below. - Editorial correction to the format issue in some ATG RRM TCs header and editor notes. - Add a new general chapter for ATG RRM TCs to specify initial test environment condition including UE positioning and update the initial condition of the related TCs accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.2.1.1, 19.2.1.2, 19.2.3.1.1, 19.2.3.1.2, 19.2.3.2.1, 19.2.3.2.2, 19.2.3.2.3, 19.2.3.2.4, 19.2.3.3.1, 19.3.1.1, 19.3.3.1, 19.4.1.1, 19.4.1.2, 19.4.1.3, 19.4.1.4, 19.4.3.1.1, 19.4.3.2.1, 19.4.4.1, 19.4.5.1, 19.5.1.1, 19.5.1.2, 19.5.1.3, 19.5.1.4, 19.5.2.1, 19.5.2.2, 19.5.2.3, 19.5.3.1, 19.5.3.2, 19.5.4.1, 19.5.4.2, 19.5.4.3, 19.5.5.1, 19.6.6.1.1, 19.6.6.1.2, 19.6.6.2.1, 19.6.6.2.2, 19.6.7.1, 19.6.7.2.1, 19.6.7.2.2, 19.6.8.1, 19.6.8.2.1, 19.6.8.2.2, 19.0 (new) and 19.0.1 (new), 19.2.2.1, 19.2.2.2, 19.6.1.1.1, 19.6.1.1.2, 19.6.1.2.1, 19.6.1.2.2, 19.6.2.2.1, 19.6.2.2.2, 19.6.3.1, 19.6.3.2.1, 19.6.3.2.2, 19.6.4.1.1, 19.6.4.1.2, 19.6.4.2.1, 19.6.4.2.2, 19.6.5.1.1, 19.6.5.1.2, 19.6.5.2, 19.6.5.3.1, 19.6.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR 3650 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related Test environment CR is R5-256053(CR 3650). R5-256053 is to define UE location for ATG RRM TCs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Add a new general chapter for ATG RRM TCs to specify the UE location. Undo the removal of the second part of the editor notes since only UE specific positioning is solved but gNB reference location and UE speed are still FFS. Also update the initial condition of the related TCs according to the newly added UE location description in cl 19.0.1. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

19	NR standalone tests for ATG

19.0	General

19.0.1	Initial test environment conditions for ATG RRM tests

The following initial conditions apply to all ATG RRM test cases specified in this section, unless otherwise specified in the test:

1. UE location according to TS 38.508-1 [14] clause 7.7.2 is provided to the UE through any preconfigured means.

19.1	RRC_IDLE state mobility for ATG

<< Unchanged Parts Skipped >>

19.2.1.1	Intra-frequency handover from FR1 to FR1; known target cell

Editor's Note: This test case is incomplete for the following configurations:

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.2.1.1.1	Test purpose

To verify the NR ATG UE’s ability to perform NR FR1-NR FR1 intra frequency handover for ATG in RRC_CONNECTED state by meeting the delay requirement and interruption length for handover to known target cell.

19.2.1.1.2	Test applicability

This test applies to all types of NR ATG UE release 18 and forward.

19.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.1.1.

19.2.1.1.4	Test description

19.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

This test shall be tested using any of the test configurations in Table 19.2.1.1.4.1-1.

Table 19.2.1.1.4.1-1: Intra-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.2.1.1.4.1-2

Table 19.2.1.1.4.1-2: Initial conditions for NR ATG SA FR1 handover with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table [TBD] and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.2.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 19.2.1.1.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Tables 19.2.1.1.4.1-3 and 19.2.1.1.5-1 below.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.2.1.2	Inter-frequency handover from FR1 to FR1; unknown target cell

Editor's Note: This test case is incomplete for the following configurations:

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.2.1.2.1	Test purpose

To verify the NR ATG UE’s ability to perform NR FR1-NR FR1 inter frequency handover for ATG in RRC_CONNECTED state by meeting the delay requirement and interruption length for handover to unknown target cell.

19.2.1.2.2	Test applicability

This test applies to all types of NR ATG UE release 18 and forward.

19.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.1.2.

19.2.1.2.4	Test description

19.2.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

This test shall be tested using any of the test configurations in Table 19.2.1.2.4.1-1.

Table 19.2.1.2.4.1-1: Inter-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.2.1.2.4.1-2

Table 19.2.1.2.4.1-2: Initial conditions for NR ATG SA FR1 handover with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table [TBD] and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.2.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 19.2.1.2.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Tables 19.2.1.2.4.1-3 and 19.2.1.2.5-1 below.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.2.2.1	Intra-frequency distance-based conditional Handover from FR1 to FR1

19.2.2.1.1	Test purpose

To verify the requirememt for the NR FR1-NR FR1 intra-frequency distance-based conditional handover for ATG requirements specified in 38.133 [6] 6.1E.2.2.

19.2.2.1.2	Test applicability

This test applies to all types of NR ATG UE release 18 onwards and supporting conditional handover.

19.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.2.1.

19.2.2.1.4	Test description

19.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 19.2.2.1.4.1-1.

Table 19.2.2.1.4.1-1: NR ATG SA FR1 distance-based conditional handover test configurations

| Config | Description |
| --- | --- |
| 19.2.2.1-1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.2.2.1-2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.2.2.1-3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.2.2.1.4.1-2

Table 19.2.2.1.4.1-2: Initial conditions for NR ATG SA FR1 distance-based conditional handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table [TBD] and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.2.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 19.2.2.1.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Tables 19.2.2.1.4.1-3 and 19.2.2.1.5-1 below.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.2.2.2	Inter-frequency distance-based conditional Handover from FR1 to FR1

19.2.2.2.1	Test purpose

To verify the requirememt for the NR FR1-NR FR1 inter-frequency distance-based conditional handover for ATG requirements specified in 38.133 [6] 6.1E.2.2.

19.2.2.2.2	Test applicability

This test applies to all types of NR ATG UE release 18 onwards and supporting distance-based conditional handover.

19.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.2.2.

19.2.2.2.4	Test description

19.2.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 19.2.2.2.4.1-1.

Table 19.2.2.2.4.1-1: NR ATG SA FR1 distance-based conditional handover test configurations

| Config | Description |
| --- | --- |
| 19.2.2.2-1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.2.2.2-2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.2.2.2-3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.2.2.2.4.1-2

Table 19.2.2.2.4.1-2: Initial conditions for NR ATG SA FR1 distance-based conditional handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table [TBD] and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.2.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 19.2.2.2.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Tables 19.2.2.2.4.1-3 and 19.2.2.2.5-1 below.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.2.3.1.1	Intra-frequency RRC Re-establishment in FR1 for ATG

Editor's Note: This test case is incomplete for the following configurations:

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.2.3.1.1.1	Test purpose

To verify the NR ATG UE’s ability to perform NR intra-frequency RRC re-establishment delay in FR1 with known target cell is within the specified limit for ATG.

19.2.3.1.1.2	Test applicability

This test applies to all types of NR ATG UE release 18 and forward.

19.2.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.2.3.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.3.1.1.

19.2.3.1.1.4	Test description

19.2.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

This test shall be tested using any of the test configurations in Table 19.2.3.1.1.4.1-1.

Table 19.2.3.1.1.4.1-1: Intra-frequency RRC re-establishment test configurations

| Config | Description |
| --- | --- |
| 19.2.3.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.2.3.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.2.3.1.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 19.2.3.1.1.4.1-2

Table 19.2.3.1.1.4.1-2: Initial conditions for NR ATG SA FR1 RRC re-establishment with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table [TBD] and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.2.3.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The test parameters are given in Tables 19.2.3.1.1.4.1-3.

2.	Message contents are defined in clause 19.2.3.1.1.4.3.

3.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.2.3.1.2	Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG

Editor's Note: This test case is incomplete for the following configurations:

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.2.3.1.2.1	Test purpose

To verify the NR ATG UE’s ability to perform NR inter-frequency RRC re-establishment delay in FR1 with unknown target cell and without serving cell timing are within the specified limits for ATG.

19.2.3.1.2.2	Test applicability

This test applies to all types of NR ATG UE release 18 and forward.

19.2.3.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.2.3.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.3.1.2.

19.2.3.1.2.4	Test description

19.2.3.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

This test shall be tested using any of the test configurations in Table 19.2.3.1.1.4.1-1.

Table 19.2.3.1.2.4.1-1: Inter-frequency RRC re-establishment test configurations

| Config | Description |
| --- | --- |
| 19.2.3.1.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.2.3.1.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.2.3.1.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 19.2.3.1.2.4.1-2

Table 19.2.3.1.1.4.1-2: Initial conditions for NR ATG SA FR1 RRC re-establishment with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.2.3.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The test parameters are given in Tables 19.2.3.1.2.4.1-3.

2.	Message contents are defined in clause 19.2.3.1.2.4.3.

3.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.2.3.2.1	NR SA FR1 4-step RA type contention based random access test for ATG

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- AT commands for UE positioning and UE speed setting are FFS.

- The specific gNB reference location emulated by test system is FFS.

- Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.2.3.2.1.1	Test purpose

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in TS 38.133 [6] clause 6.2D.2.2 and clause 7.1D.2 in an AWGN with constant residual doppler model.

19.2.3.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.2.3.2.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 19.2.3.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.3.2.1.

19.2.3.2.1.4	Test description

19.2.3.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.3.2.2.1.4.1 with following exceptions:

-	Table 6.3.2.2.1.4.1-1 is replaced by Table 19.2.3.2.1.4.1-1.

-	Table 6.3.2.2.1.4.1-2 is replaced by Table 19.2.3.2.1.4.1-2.

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.2.3.2.2	NR SA FR1 4-step RA type non-contention based random access test for ATG

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- AT commands for UE positioning and UE speed setting are FFS.

- The specific gNB reference location emulated by test system is FFS.

- Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.2.3.2.2.1	Test purpose

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in TS 38.133 [6] clause 6.2D.2.2 and clause 7.1D.2 in an AWGN with constant residual doppler model.

19.2.3.2.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards. Additionally Test 2 is applicable to UE that supports CSI-RS based Random Access Preamble which requires UE to support csi-RS-CFRA-ForHO.

19.2.3.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 19.2.3.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.3.2.2.

19.2.3.2.2.4	Test description

19.2.3.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.3.2.2.2.4.1 with following exceptions:

-	Table 6.3.2.2.2.4.1-1 is replaced by Table 19.2.3.2.2.4.1-1.

-	Table 6.3.2.2.2.4.1-2 is replaced by Table 19.2.3.2.2.4.1-2.

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.2.3.2.3	NR SA FR1 2-step RA type contention based random access test for ATG

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- AT commands for UE positioning and UE speed setting are FFS.

- The specific gNB reference location emulated by test system is FFS.

- Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.2.3.2.3.1	Test purpose

The purpose of this test is to verify that the behavior of the 2-step RA type random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits. This test will verify the requirements in TS 38.133 [6] clause 6.2D.2.3 and clause 7.1D.2 in an AWGN with constant residual doppler model.

19.2.3.2.3.2	Test applicability

This test applies to all types of NR ATG UE supporting 2-step RACH from Release 18 onwards.

19.2.3.2.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 19.2.3.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.3.2.3.

19.2.3.2.3.4	Test description

19.2.3.2.3.4.1	Initial conditions

Same as the initial conditions given in clause 6.3.2.2.3.4.1 with following exceptions:

-	Table 6.3.2.2.2.4.1-1 is replaced by Table 19.2.3.2.3.4.1-1.

-	Table 6.3.2.2.2.4.1-2 is replaced by Table 19.2.3.2.3.4.1-2.

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.2.3.2.4	NR SA FR1 2-step RA type non-contention based random access test for ATG

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- AT commands for UE positioning and UE speed setting are FFS.

- The specific gNB reference location emulated by test system is FFS.

- Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.2.3.2.4.1	Test purpose

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits. This test will verify the requirements in TS 38.133 [6] clause 6.2D.2.3 and clause 7.1D.2 in an AWGN with constant residual doppler model.

19.2.3.2.4.2	Test applicability

This test applies to all types of NR ATG UE supporting 2-step RACH from Release 18 onwards.

19.2.3.2.4.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 19.2.3.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clauses A.19.2.3.2.4.

19.2.3.2.4.4	Test description

19.2.3.2.4.4.1	Initial conditions

Same as the initial conditions given in clause 6.3.2.2.4.4.1 with following exceptions:

-	Table 6.3.2.2.4.4.1-1 is replaced by Table 19.2.3.2.4.4.1-1.

-	Table 6.3.2.2.4.4.1-2 is replaced by Table 19.2.3.2.4.4.1-2.

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.2.3.3.1	NR SA FR1-FR1 RRC connection release with Redirection for ATG

Editor's Note: This test case is incomplete:

-	 Test applicability is missing.

-	 AT commands for UE positioning and UE speed setting are FFS.

- 	The specific gNB reference location emulated by test system is FFS.

-	 Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.2.3.3.1.1	Test purpose

This test is to verify RRC connection release with redirection for ATG from NR to NR.

19.2.3.3.1.2	Test applicability

This test applies to all types of NR ATG UE release 18 and forward.

19.2.3.3.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 19.2.3.3.0.

The normative reference for this requirement is TS 38.133 [6] A.19.2.3.3.1.

19.2.3.3.1.4	Test description

19.2.3.3.1.4.1	Initial conditions

This test can be run in the configurations defined in Table19.2.3.3.1.4.1-1.

Table19.2.3.3.1.4.1-1: Redirection from NR to NR test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table19.2.3.3.1.4.1-2.

Table19.2.3.3.1.4.1-2: Initial conditions for Redirection from NR to NR test case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table19.2.3.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameters for PCell and neighbour cell are given in Table19.2.3.3.1.4.1-3 below.

2. Message contents are defined in clause19.2.3.3.1.4.3.

3. There are two carriers and two NR cells specified in the test. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.3.1.1	NR SA FR1 UE Transmit Timing Test for ATG

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- T_GNSS_margin need to be subtracted for the final test requirement.

- AT commands for UE positioning and UE speed setting are FFS.

- The specific gNB reference location emulated by test system is FFS.

- Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.3.1.1.1	Test purpose

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in TS 38.133 [6] clause 7.1D.2.

19.3.1.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.3.1.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 19.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.3.1.1

19.3.1.1.4	Test description

19.3.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.4.1.1.4.1 with following exceptions:

-	Table 6.4.1.1.4.1-1 is replaced by Table 19.3.1.1.4.1-1.

-	Table 6.4.1.1.4.1-2 is replaced by Table 19.3.1.1.4.1-2.

-	During the test, UE location should be changed with the mobility assumption of 1200km/h, the specific UE location should be emulated by test system and provided to UE by AT command or GNSS simulator.

-	During the test, the specific gNB reference location is emulated by test system.

-	During the test, the transmit timing is verified by the UE transmitting SRS using the configuration SRSconfig.1 defined in Table 6.4.1.1.5-2.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.3.3.1	NR SA FR1 timing advance adjustment accuracy for ATG

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- AT commands for UE positioning and UE speed setting are FFS.

- The specific gNB reference location emulated by test system is FFS.

- Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.3.3.1.1	Test purpose

The purpose of the test is to verify UE Timing Advance adjustment delay and accuracy requirement defined in TS 38.133 [6] clause 7.3D.

19.3.3.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.3.3.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 19.3.3.0.

The normative reference for this requirement is TS 38.133 [6] clause A.19.3.3.1.

19.3.3.1.4	Test description

19.3.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.4.3.1.4.1 with following exceptions:

-	Table 6.4.3.1.4.1-1 is replaced by Table 19.3.3.1.4.1-1.

-	Table 6.4.3.1.4.1-2 is replaced by Table 19.3.3.1.4.1-2.

-	During the test, UE location should be changed with the mobility assumption of 1200km/h, the specific UE location should be emulated by test system and provided to UE by AT command or GNSS simulator.

-	During the test, the specific gNB reference location is emulated by test system.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- AT Command to set the UE speed and position is not specified in the test procedure

- The specific gNB reference location is missing in test procedure

- Antenna array is FFS

19.4.1.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in TS 38.133 [6] clause 8.1D.

19.4.1.1.2	Test applicability

This test applies to all types of NR ATG UE from release 18.

19.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.4.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.4.1.1.

19.4.1.1.4	Test description

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure 19.4.1.1.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40 ms) in test 1.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

Figure 19.4.1.1.4-1: SNR variation for out-of-sync testing

19.4.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.4.1.1.4.1-1

Table 19.4.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.4.1.1-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.4.1.1-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.4.1.1-3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Test environment parameters are given in Table 19.4.1.1.4.1-2.

Table 19.4.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.4.1.1.4.1-3 and 19.4.1.1.4.1-4.

2.	Message contents are defined in clause 19.4.1.1.4.3.

3.	There is one cell (Cell 1), which is the active NR cell, in the test.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.4.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

Editor's Note: This test case is incomplete in following aspects:

TT analysis is missing.

AT Command to set the UE speed and position is not specified in the test procedure

The specific gNB reference location is missing in test procedure

Antenna array is FFS

19.4.1.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in ts 38.133 [6] clause 8.1D.

19.4.1.2.2	Test applicability

This test applies to all types of NR UE from release 18 onwards.

19.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.4.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.4.1.2.

19.4.1.2.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.19.4.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms.

Figure 19.4.1.2.4: SNR variation for in-sync testing

19.4.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.4.1.2.4.1-1

Table 19.4.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.4.1.2-1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 19.4.1.2-2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 19.4.1.2-3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Test environment parameters are given in Table 19.4.1.2.4.1-2.

Table 19.4.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.4.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 19.4.1.2.4.1-3.

2.	Message contents are defined in clause 19.4.1.2.4.3.

3.	There is one cell (Cell 1), which is the active cell, in the test.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- AT Command to set the UE speed and position is not specified in the test procedure

- The specific gNB reference location is missing in test procedure

- Antenna array is FFS

19.4.1.3.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in TS 38.133 [6] clause 8.1D.

19.4.1.3.2	Test applicability

This test applies to all types of NR UE from release 18 onwards.

19.4.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.4.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.4.1.3.

19.4.1.3.4	Test description

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure 19.4.1.3.4-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting of 5 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40 ms) in test. In the test, SSB0 is configured as the BFD-RS.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

Figure 19.4.1.3.4-1: SNR variation for CSI-RS out-of-sync testing

19.4.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.4.1.3.4.1-1

Table 19.4.1.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.4.1.3-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.4.1.3-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.4.1.3-3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Test environment parameters are given in Table 19.4.1.3.4.1-2.

Table 19.4.1.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.4.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.4.1.3.4.1-3 and 19.4.1.3.4.1-4.

2.	Message contents are defined in clause 19.4.1.3.4.3.

3.	There is one cell, Cell 1 which is the PCell, in the test.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.4.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in non-DRX mode

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- AT Command to set the UE speed and position is not specified in the test procedure

- The specific gNB reference location is missing in test procedure

- Antenna array is FFS

19.4.1.4.1	Test purpose

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in TS 38.133 [6] clause 8.1D.

19.4.1.4.2	Test applicability

This test applies to all types of NR ATG UE from release 18 onwards.

19.4.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.4.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.4.1.4.

19.4.1.4.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 19.4.1.4.4-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled. In the test, SSB0 is configured as the BFD-RS.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

Figure 19.4.1.4.4-1: SNR variation for CSI-RS in-sync testing

19.4.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.4.1.4.4.1-1

Table 19.4.1.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.4.1.4-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.4.1.4-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.4.1.4-3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Test environment parameters are given in Table 19.4.1.4.4.1-2.

Table 19.4.1.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.4.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 19.4.1.4.4.1-3.

2.	Message contents are defined in clause 19.4.1.4.4.3.

3.	There is one cell (Cell 1), which is the PCell in the test.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.4.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for ATG

Editor's Note: This test is incomplete in the following aspects:

- MU and TT analysis is incomplete

- Speed setting is FFS

- Antenna array is FFS

19.4.3.1.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 19.4.3.1.0.1.

19.4.3.1.1.2	Test applicability

This test applies to all types of NR ATG UE release 18 onwards supporting BWP adaptation of at least 2 BWPs, DCI and timer-based active BWP switching delay Type1 or Type2.

19.4.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.4.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.4.3.1.1.

19.4.3.1.1.4	Test description

19.4.3.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.4.3.1.1.4.1-1.

Table 19.4.3.1.1.4.1-1: Supported test configurations for NR SA FR1 DCI-based DL active BWP switch in non-DRX for ATG

| Config | Description |
| --- | --- |
| 19.4.3.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.4.3.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.4.3.1.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: A UE which fulfils the requirements in test case A.19.4.3.1.1 can skip the test cases in A.19.4.3.1.1. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.4.3.1.1.4.1-2.

Table 19.4.3.1.1.4.1-2: Initial conditions for NR SA FR1 DCI-based DL active BWP switch in non-DRX for ATG

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.4.3.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 19.4.3.1.1.4.1-3.

2. Message contents are defined in clause 19.4.3.1.1.4.3.

3. The test scenario comprises of one NR PCell (Cell 1). Cell 1 is configured according to Annex C.1.2 and C.1.3.

4. By step 4 of the test procedure:

-	UE is connected to Cell 1 on radio channel 1.

-	UE is configured with 2 different UE-specific downlink bandwidth parts, BWP-1 and BWP-2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

-	UE is configured with a bwp-InactivityTimer timer value for Cell 1.

5.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.4.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for ATG

Editor's Note: This test is incomplete in the following aspects:

- MU and TT analysis is incomplete

- Speed setting is FFS

- Antenna array is FFS

19.4.3.2.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in clause 19.4.3.2.0.1.

19.4.3.2.1.2	Test applicability

This test applies to all types of NR ATG UE release 18 onwards supporting BWP adaptation of at least 2 BWPs.

19.4.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.4.3.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.19.4.3.2.1.

19.4.3.2.1.4	Test description

19.4.3.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.4.3.2.1.4.1-1.

Table 19.4.3.2.1.4.1-1: Supported test configurations for NR SA FR1 RRC-based DL active BWP switch in non-DRX for ATG

| Config | Description |
| --- | --- |
| 19.4.3.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.4.3.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.4.3.2.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 19.4.3.2.1.4.1-2.

Table 19.4.3.2.1.4.1-2: Initial conditions for NR SA FR1 RRC-based DL active BWP switch in non-DRX for ATG

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.4.3.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 19.4.3.2.1.4.1-3.

2. Message contents are defined in clause 19.4.3.2.1.4.3.

3. The test scenario comprises of one NR PCell (Cell 1). Cell 1 is configured according to Annex C.1.2 and C.1.3.

4. By step 4 of the test procedure:

-	UE is connected to Cell 1 on radio channel 1.

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in Cell 1.

5.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.4.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for ATG

Editor's Note: This test is incomplete in the following aspects:

- MU and TT analysis is incomplete

- Speed setting is FFS

- Antenna array is FFS

19.4.4.1.1	Test purpose

The purpose of this test is to verify the UE specific CBW change delay requirement defined in clause 19.4.4.0.1.

19.4.4.1.2	Test applicability

This test applies to all types of NR ATG UE release 18 onwards.

19.4.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.4.4.0.

The normative reference for this requirement is TS 38.133 [6] clause A.19.4.4.1.

19.4.4.1.4	Test description

The test scenario comprises of one Cell, PCell (Cell 1), which has constant signal levels throughout the test. The test consists of 1 time period with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration containing SCS-SpecificCarrier with updated UE specific CBW, sent from the test equipment to the UE, is completely received at the UE side in PCell’s slot # denoted i. The UE shall reconfigure its UE specific CBW with the updated CBW-2 for the final condition.

The UE shall be able to receive PDSCH on PCell from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWcℎangeDelayRRCNR Slot lengtℎ as defined in clause 19.4.4.0 and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWcℎangeDelayRRCNR Slot lengtℎ+k1 on the PCell’s BWP-1 on CBW-2 for the final condition. The UE shall be continuously scheduled on the PCell’s BWP-1 on CBW-2  for the final condition starting from the first DL slot right after slot i+TRRCprocessingDelay+TCBWcℎangeDelayRRCNR Slot lengtℎ.

TRRCprocessingDelay and TCBWcℎangeDelayRRC are defined in clause 19.4.4.0.

The test equipment verifies the UE specific CBW switching delay in PCell by estimating the time from the moment the RRC Reconfiguration message including updated UE specific CBW configuration is sent until the moment a vaild ACK/NACK is received.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.4.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.4.4.1.4.1-1.

Table 19.4.4.1.4.1-1: Supported test configurations for NR SA FR1 UE specific CBW change on PCell in non-DRX for ATG

| Configuration | Description |
| --- | --- |
| 19.4.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.4.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.4.4.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 19.4.4.1.4.1-2.

Table 19.4.4.1.4.1-2: Initial conditions for NR SA FR1 UE specific CBW change on PCell in non-DRX for ATG

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.17-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.4.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 19.4.4.1.4.1-3.

2. Message contents are defined in clause 19.4.4.1.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

4. By step 3 of the test procedure:

-	UE is connected to Cell 1 (PCell) on radio channel 1.

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1 (PCell).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PCell.

-	UE has been configured with UE specific CBW (CBW-1).

-	UE is indicated in SCS-SpecificCarrier [13] that the UE specific CBW is CBW-1 as the initial condition in Cell 1 (PCell).

5.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.4.5.1	NR SA FR1 MAC-CE based pathloss reference signal switch delay for ATG

Editor’s Note: This test case has been completed for the following configurations:

- MU and TT are missing

- Speed setting is FFS

- Antenna array is FFS

- Several parameters are still in brackets

19.4.5.1.1	Test purpose

The purpose of this test is to verify the MAC-CE based pathloss reference signal switch delay requirement defined in clause 19.4.5.0.1.

19.4.5.1.2	Test applicability

This test applies to all types of NR ATG UE release 18 onwards.

19.4.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.4.5.0.

The normative reference for this requirement is TS 38.133 [6] clause A.19.4.5.1.

19.4.5.1.4	Test description

The test scenario comprises of one Cell, PCell (Cell 1).

During T1,

-	The UE shall track SSB #1 so that SSB #1 as a pathloss reference signal is known to the UE.

Time period T2 starts when the UE is configured of the power headroom reporting functionality by upper layers by the test equipment and the UE shall transmit a PHR during T2.

During T2,

-	UE is configured with a phr-ProhibitTimer timer value for Cell 1.

-	UE is configured with a phr-Tx-PowerFactorChange value for Cell 1.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.4.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.4.5.1.4.1-1.

Table 19.4.5.1.4.1-1: Supported test configurations for NR SA FR1 MAC-CE based pathloss reference signal switch delay for ATG

| Config | Description |
| --- | --- |
| 19.4.5.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.4.5.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.4.5.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.4.5.1.4.1-2.

Table 19.4.5.1.4.1-2: Initial conditions for NR SA FR1 MAC-CE based pathloss reference signal switch delay for ATG

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.4.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 19.4.5.1.4.1-3.

2. Message contents are defined in clause 19.4.5.1.4.3.

3. The test scenario comprises of one cell (Cell 1). Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

4. By step 2 of the test procedure:

-	UE is connected to Cell 1 on radio channel 1.

-	UE shall be fully synchronized to SSB #0.

5. The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.1.1	SA FR1 event triggered reporting tests without gap without SSB index reading under non-DRX

Editor's Note: This test case is incomplete in following aspects:

Test parameters are missing in TS 38.133

- TT analysis is missing.

Message contents are FFS

Test procedure is missing

- AT Command to set the UE speed is not specified

- The specific gNB reference location is missing

- Antenna array is FFS

19.5.1.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.2D.5.1 and 9.2D.5.2.

19.5.1.1.2	Test applicability

This test applies to all types of NR ATG UE from release 18.

19.5.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.1.1.

19.5.1.1.4	Test description

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.5.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.5.1.1.4.1-1

Table 19.5.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.5.1.1-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.1-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.1-3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations |  |

Test environment parameters are given in Table 19.5.1.1.4.1-2.

Table 19.5.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.5.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.5.1.1.4.1-3 and 19.5.1.1.4.1-4.

2.	Message contents are defined in clause 19.5.1.1.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in the Table TBD.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.1.2	SA FR1 event triggered reporting tests with per-UE gaps under non-DRX

Editor's Note: This test case is incomplete in following aspects:

Test parameters are missing in TS 38.133

- TT analysis is missing.

Message contents are FFS

Test procedure is missing

- AT Command to set the UE speed is not specified

- The specific gNB reference location is missing

- Antenna array is FFS

19.5.1.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.2D.6.2 and 9.2D.6.3.

19.5.1.2.2	Test applicability

This test applies to all types of NR ATG UE from release 18.

19.5.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.1.2.

19.5.1.2.4	Test description

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.5.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.5.1.2.4.1-1

Table 19.5.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.5.1.2-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.2-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.2-3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations |  |

Test environment parameters are given in Table 19.5.1.2.4.1-2.

Table 19.5.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.5.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.5.1.2.4.1-3 and 19.5.1.2.4.1-4.

2.	Message contents are defined in clause 19.5.1.2.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in the Table TBD.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.1.3	SA FR1 event triggered reporting tests without gap under non-DRX with SSB index reading

Editor's Note: This test case is incomplete in following aspects:

Test parameters are missing in TS 38.133

- TT analysis is missing.

Message contents are FFS

Test procedure is missing

- AT Command to set the UE speed is not specified

- The specific gNB reference location is missing

- Antenna array is FFS

19.5.1.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.2D.5.1 and 9.2D.5.2.

19.5.1.3.2	Test applicability

This test applies to all types of NR ATG UE from release 18.

19.5.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.1.3.

19.5.1.3.4	Test description

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.5.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.5.1.3.4.1-1

Table 19.5.1.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.5.1.3-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.3-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.3-3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Test environment parameters are given in Table 19.5.1.3.4.1-2.

Table 19.5.1.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.5.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.5.1.3.4.1-3 and 19.5.1.3.4.1-4.

2.	Message contents are defined in clause 19.5.1.3.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in the Table TBD.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.1.4	SA FR1 event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading

Editor's Note: This test case is incomplete in following aspects:

Test parameters are missing in TS 38.133

- TT analysis is missing.

Message contents are FFS

Test procedure is missing

- AT Command to set the UE speed is not specified

- The specific gNB reference location is missing

- Antenna array is FFS

19.5.1.4.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.2D.6.2 and 9.2D.6.3.

19.5.1.4.2	Test applicability

This test applies to all types of NR ATG UE from release 18.

19.5.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.1.4.

19.5.1.4.4	Test description

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.5.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.5.1.4.4.1-1

Table 19.5.1.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.5.1.4-1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.4-2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 19.5.1.4-3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations |  |

Test environment parameters are given in Table 19.5.1.4.4.1-2.

Table 19.5.1.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.5.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.5.1.4.4.1-3 and 19.5.1.4.4.1-4.

2.	Message contents are defined in clause 19.5.1.4.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in the Table TBD.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.2.1	SA FR1-FR1 event triggered reporting tests without SSB time index detection when DRX is used

Editor's Note: This test case is incomplete in following aspects:

Test parameters are missing in TS 38.133

- TT analysis is missing.

Message contents are FFS

Test procedure is missing

- AT Command to set the UE speed is not specified

- The specific gNB reference location is missing

- Antenna array is FFS

19.5.2.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.3D.4.

19.5.2.1.2	Test applicability

This test applies to all types of NR ATG UE from release 18.

19.5.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.2.1.

19.5.2.1.4	Test description

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.5.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.5.2.1.4.1-1

Table 19.5.2.1.4.1-1: Supported test configurations

TBD

Test environment parameters are given in Table 19.5.2.1.4.1-2.

Table 19.5.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.5.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.5.2.1.4.1-3 and 19.5.2.1.4.1-4.

2.	Message contents are defined in clause 19.5.2.1.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the different frequency than the PCell. The test parameters for PCell and neighbour cell are given in the Table TBD.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.2.2	SA FR1-FR1 event triggered reporting tests with SSB time index detection when DRX is not used

Editor's Note: This test case is incomplete in following aspects:

- Test parameters are missing in TS 38.133

- TT analysis is missing.

- Message contents are FFS

- Test procedure is missing

- AT Command to set the UE speed is not specified

- The specific gNB reference location is missing

- Antenna array is FFS

19.5.2.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.3D.4 and 9.3D.5.

19.5.2.2.2	Test applicability

This test applies to all types of NR ATG UE from release 18.

19.5.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.2.2.

19.5.2.2.4	Test description

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.5.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.5.2.2.4.1-1

Table 19.5.2.2.4.1-1: Supported test configurations

TBD

Test environment parameters are given in Table 19.5.2.1.4.1-2.

Table 19.5.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.5.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.5.2.2.4.1-3 and 19.5.2.2.4.1-4.

2.	Message contents are defined in clause 19.5.2.2.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the different frequency than the PCell. The test parameters for PCell and neighbour cell are given in the Table TBD.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

19.5.2.3	SA FR1-FR1 event triggered reporting tests without gap when DRX is not used

Editor's Note: This test case is incomplete in following aspects:

- Test parameters are missing in TS 38.133

- TT analysis is missing.

- Message contents are FFS

- Test procedure is missing

- AT Command to set the UE speed is not specified

- The specific gNB reference location is missing

- Antenna array is FFS

19.5.2.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in 38.133 [6] clauses 9.3D.9.

19.5.2.3.2	Test applicability

This test applies to all types of NR ATG UE from release 18.

19.5.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.2.3.

19.5.2.3.4	Test description

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

19.5.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.5.2.3.4.1-1

Table 19.5.2.3.4.1-1: Supported test configurations

TBD

Test environment parameters are given in Table 19.5.2.1.4.1-2.

Table 19.5.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.5.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to tables 19.5.2.3.4.1-3 and 19.5.2.3.4.1-4.

2.	Message contents are defined in clause 19.5.2.3.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the different frequency than the PCell. The test parameters for PCell and neighbour cell are given in the Table TBD.

4.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.3.1	NR SA FR1 SSB based L1-RSRP measurement in non-DRX for ATG

Editor’s Note: This test case is incomplete in following aspects.

- 	TT analysis is missing

-	 AT commands for UE positioning and UE speed setting are FFS.

-	 The specific gNB reference location emulated by test system is FFS.

-	 Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.5.3.1.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

19.5.3.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward.

19.5.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.3.1.

19.5.3.1.4	Test description

19.5.3.1.4.1	Initial conditions

Same sas the initial conditions given in clause 6.6.4.1.4.1 with following exceptions:

-	Table 6.6.4.1.4.1-1, Table 6.6.4.1.4.1-2 and Table 6.6.4.1.4.1-3 are replaced by Table 19.5.3.1.4.1-1, Table 19.5.3.1.4.1-2 and Table 19.5.3.1.4.1-3 respectively;

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.3.2	NR SA FR1 CSI-RS based L1-RSRP measurement in non-DRX for ATG

Editor’s Note: This test case is incomplete in following aspects.

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.5.3.2.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

19.5.3.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward.

19.5.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.3.2.

19.5.3.2.4	Test description

19.5.3.2.4.1	Initial conditions

Same sas the initial conditions given in clause 6.6.4.3.4.1 with following exceptions:

-	Table 6.6.4.3.4.1-1, Table 6.6.4.3.4.1-2 and Table 6.6.4.3.4.1-3 are replaced by Table 19.5.3.2.4.1-1, Table 19.5.3.2.4.1-2 and Table 19.5.3.2.4.1-3 respectively;

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.4.1	NR SA FR1 L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured in non-DRX for ATG

Editor’s Note: This test case is incomplete in following aspects.

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.5.4.1.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement in non-DRX within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.1.

19.5.4.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward and supporting L1-SINR measurement.

19.5.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.4.1.

19.5.4.1.4	Test description

19.5.4.1.4.1	Initial conditions

Same sas the initial conditions given in clause 6.6.8.1.4.1 with following exceptions:

-	Table 6.6.8.1.4.1-1, Table 6.6.8.1.4.1-2 and Table 6.6.8.1.4.1-3 are replaced by Table 19.5.4.1.4.1-1, Table 19.5.4.1.4.1-2 and Table 19.5.4.1.4.1-3 respectively;

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.4.2	NR SA FR1 L1-SINR measurement with SSB based CMR and dedicated IMR in non-DRX for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.5.4.2.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement in non-DRX within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.2.

19.5.4.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward and supporting L1-SINR measurement.

19.5.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.4.2.

19.5.4.2.4	Test description

19.5.4.2.4.1	Initial conditions

Same sas the initial conditions given in clause 6.6.8.2.4.1 with following exceptions:

-	Table 6.6.8.2.4.1-1, Table 6.6.8.2.4.1-2 and Table 6.6.8.2.4.1-3 are replaced by Table 19.5.4.2.4.1-1, Table 19.5.4.2.4.1-2 and Table 19.5.4.2.4.1-3 respectively;

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.4.3	NR SA FR1 L1-SINR measurement with CSI-RS based CMR and dedicated IMR in non-DRX for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.5.4.3.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement in non-DRX within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.3.

19.5.4.3.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward and supporting L1-SINR measurement.

19.5.4.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.5.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.4.3.

19.5.4.3.4	Test description

19.5.4.3.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.8.3.4.1 with following exceptions:

-	Table 6.6.8.3.4.1-1, Table 6.6.8.3.4.1-2 and Table 6.6.8.3.4.1-3 are replaced by Table 19.5.4.3.4.1-1, Table 19.5.4.3.4.1-2 and Table 19.5.4.3.4.1-3 respectively;

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.5.5.1	NR SA FR1 CGI identification of NR neighbor cell for ATG

Editor’s Note: This test case is incomplete in following aspects.

- 	TT analysis is missing

-	Message contents are missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.5.5.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of intra-frequency CGI identification of an NR neighbour cell in FR1 with autonomous gaps. This test shall partly verify the measurement requirements in Clause 9.11 in TS 38.133 [6].

19.5.5.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward and supporting acquisition of CGI from neighbour NR cell using autonomous gap.

19.5.5.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 19.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.5.5.1.

19.5.5.1.4	Test description

Same as the test description given in clause 6.6.7.1.4.

19.5.5.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.6.7.1.4.1 with following exceptions:

-	Table 6.6.7.1.4.1-1 and Table 6.6.7.1.4.1-2 are replaced by Table 19.5.5.1.4.1-1 and Table 19.5.5.1.4.1-2 respectively;

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.1.1	Intra-frequency measurements for ATG

19.6.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy for ATG

19.6.1.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

19.6.1.1.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.1.1.

19.6.1.1.1.4	Test description

19.6.1.1.1.4.1	Initial conditions

Same as the Initial conditions given in clause 6.7.1.1.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.1.1.1.4.1-1 is replaced by Table 19.6.1.1.1.4.1-1;

-	Table 6.7.1.1.1.4.1-2 is replaced by Table 19.6.1.1.1.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy for ATG

19.6.1.1.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

19.6.1.1.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.1.1.

19.6.1.1.2.4	Test description

19.6.1.1.2.4.1	Initial conditions

Same as the Initial conditions given in clause 6.7.1.1.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.1.1.2.4.1-1 is replaced by Table 19.6.1.1.2.4.1-1;

-	Table 6.7.1.1.2.4.1-2 is replaced by Table 19.6.1.1.2.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG

19.6.1.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

19.6.1.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.1.2.

19.6.1.2.1.4	Test description

19.6.1.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.1.2.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.1.2.1.4.1-1 is replaced by Table 19.6.1.2.1.4.1-1.

-	Table 6.7.1.2.1.4.1-2 is replaced by Table 19.6.1.2.1.4.1-2.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG

19.6.1.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

19.6.1.2.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.1.2.

19.6.1.2.2.4	Test description

19.6.1.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.1.2.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.1.2.2.4.1-1 is replaced by Table 19.6.1.2.2.4.1-1.

-	Table 6.7.1.2.2.4.1-2 is replaced by Table 19.6.1.2.2.4.1-2.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.2.2	Inter-frequency SS-RSRQ measurements for ATG

19.6.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG

19.6.2.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRQ absolute measurement accuracy is within the specified limits for all bands.

19.6.2.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.2.2.

19.6.2.2.1.4	Test description

19.6.2.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.2.2.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.2.2.1.4.1-1 is replaced by Table 19.6.2.2.1.4.1-1.

-	Table 6.7.2.2.1.4.1-2 is replaced by Table 19.6.2.2.1.4.1-2.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG

19.6.2.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

19.6.2.2.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.1.2.

19.6.2.2.2.4	Test description

19.6.2.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.2.2.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.2.2.2.4.1-1 is replaced by Table 19.6.2.2.2.4.1-1.

-	Table 6.7.2.2.2.4.1-2 is replaced by Table 19.6.2.2.2.4.1-2.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.3.1	NR SA FR1 SS-SINR absolute measurement accuracy for ATG

19.6.3.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-SINR measurement accuracy is within the specified limits for all bands.

19.6.3.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting ss-SINR-Meas.

19.6.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.3.1.

19.6.3.1.4	Test description

19.6.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.3.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.3.1.4.1-1 is replaced by Table 19.6.3.1.4.1-1;

-	Table 6.7.3.1.4.1-2 is replaced by Table 19.6.3.1.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG

19.6.3.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-SINR absolute measurement accuracy is within the specified limits for all bands.

19.6.3.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting ss-SINR-Meas.

19.6.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.3.2.

19.6.3.2.1.4	Test description

19.6.3.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.3.2.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.3.2.1.4.1-1 is replaced by Table 19.6.3.2.1.4.1-1.

-	Table 6.7.3.2.1.4.1-2 is replaced by Table 19.6.3.2.1.4.1-2.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG

19.6.3.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-SINR relative measurement accuracy is within the specified limits for all bands.

19.6.3.2.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting ss-SINR-meas.

19.6.3.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.1.2.

19.6.3.2.2.4	Test description

19.6.3.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.3.2.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.3.2.2.4.1-1 is replaced by Table 19.6.3.2.2.4.1-1.

-	Table 6.7.3.2.2.4.1-2 is replaced by Table 19.6.3.2.2.4.1-2.

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for ATG

19.6.4.1.1.1	Test purpose

The purpose of this test is to verify that the SSB based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

19.6.4.1.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.4.1.

19.6.4.1.1.4	Test description

19.6.4.1.1.4.1	Initial conditions

Same as the Initial conditions given in clause 6.7.4.1.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.4.1.1.4.1-1 is replaced by Table 19.6.4.1.1.4.1-1;

-	Table 6.7.4.1.1.4.1-2 is replaced by Table 19.6.4.1.1.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy for ATG

19.6.4.1.2.1	Test purpose

The purpose of this test is to verify that the SSB based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

19.6.4.1.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.4.1.

19.6.4.1.2.4	Test description

19.6.4.1.2.4.1	Initial conditions

Same as the Initial conditions given in clause 6.7.4.1.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

Table 6.7.4.1.2.4.1-1 is replaced by Table 19.6.4.1.2.4.1-1;

-	Table 6.7.4.1.2.4.1-2 is replaced by Table 19.6.4.1.2.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG

19.6.4.2.1.1	Test purpose

The purpose of this test is to verify that the CSI-RS based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

19.6.4.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.4.2.

19.6.4.2.1.4	Test description

19.6.4.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.2.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.4.2.1.4.1-1 is replaced by Table 19.6.4.2.1.4.1-1;

-	Table 6.7.4.2.1.4.1-2 is replaced by Table 19.6.4.2.1.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for ATG

19.6.4.2.2.1	Test purpose

The purpose of this test is to verify that the CSI-RS based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

19.6.4.2.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.4.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.4.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.4.2.

19.6.4.2.2.4	Test description

19.6.4.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.4.2.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.4.2.2.4.1-1 is replaced by Table 19.6.4.2.2.4.1-1;

-	Table 6.7.4.2.2.4.1-2 is replaced by Table 19.6.4.2.2.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.5.1.1	NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG

19.6.5.1.1.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

19.6.5.1.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting L1-SINR measurements.

19.6.5.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.5.1.

19.6.5.1.1.4	Test description

19.6.5.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.9.1.1.4.1 with following exceptions:

- During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

- During the test, the specific gNB reference location is emulated by test system.

- Table 6.7.9.1.1.4.1-1 is replaced by Table 19.6.5.1.1.4.1-1;

- Table 6.7.9.1.1.4.1-2 is replaced by Table 19.6.5.1.1.4.1-2;

- The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.5.1.2	NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG

19.6.5.1.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR relative measurement accuracy is within the specified limits.

19.6.5.1.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting L1-SINR measurements.

19.6.5.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.5.1.

19.6.5.1.2.4	Test description

19.6.5.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.9.1.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.9.1.2.4.1-1 is replaced by Table 19.6.5.1.2.4.1-1;

-	Table 6.7.9.1.2.4.1-2 is replaced by Table 19.6.5.1.2.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.5.2	NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG

19.6.5.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

19.6.5.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting L1-SINR measurements.

19.6.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.5.2.

19.6.5.2.4	Test description

19.6.5.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.9.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.9.2.4.1-1 is replaced by Table 19.6.5.2.4.1-1;

-	Table 6.7.9.2.4.1-2 is replaced by Table 19.6.5.2.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.5.3.1	NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG

19.6.5.3.1.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

19.6.5.3.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting L1-SINR measurements.

19.6.5.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.5.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.5.3.

19.6.5.3.1.4	Test description

19.6.5.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.9.3.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.9.3.1.4.1-1 is replaced by Table 19.6.5.3.1.4.1-1;

-	Table 6.7.9.3.1.4.1-2 is replaced by Table 19.6.5.3.1.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.5.3.2	NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG

19.6.5.3.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR relative measurement accuracy is within the specified limits.

19.6.5.3.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting L1-SINR measurements.

19.6.5.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.5.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.5.3.

19.6.5.3.2.4	Test description

19.6.5.3.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.9.3.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.9.3.2.4.1-1 is replaced by Table 19.6.5.3.2.4.1-1;

-	Table 6.7.9.3.2.4.1-2 is replaced by Table 19.6.5.3.2.4.1-2;

-	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.6.1.1	NR SA FR1 CSI-RSRP absolute measurement accuracy for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	Test requirements are missing

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.6.6.1.1.1	Test purpose

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clauses 10.1.2.3.1 for CSI-RS intra-frequency measurements

19.6.6.1.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward supporting CSI-RS based RSRP and RSRQ measurement.

19.6.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 19.6.6.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.6.1.

19.6.6.1.1.1.4	Test description

19.6.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.6.6.1.1.4.1-1.

Table 19.6.6.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.6.6.1.1-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.6.1.1-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.6.1.1-3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.6.6.1.1.4.1-2.

Table 19.6.6.1.1.4-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 19.6.6.1.1.4.3.

2.	There are two cells on the same frequency. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-RSRP measurements.

3.	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

4.	During the test, the specific gNB reference location is emulated by test system.

5.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.6.1.2	NR SA FR1 CSI-RSRP relative measurement accuracy for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	Test requirements are missing

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.6.6.1.2.1	Test purpose

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clauses 10.1.2.3.2 for CSI-RS intra-frequency measurements

19.6.6.1.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward supporting CSI-RS based RSRP and RSRQ measurement.

19.6.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 19.6.6.0.2

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.6.1.

19.6.6.1.2.1.4	Test description

19.6.6.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.6.6.1.2.4.1-1.

Table 19.6.6.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.6.6.1.2-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.6.1.2-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.6.1.2-3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.6.6.1.2.4.1-2.

Table 19.6.6.1.2.4-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 19.6.6.1.2.4.3.

2.	There are two cells on the same frequency. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-RSRP measurements.

3.	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

4.	During the test, the specific gNB reference location is emulated by test system.

5.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.6.2.1	NR SA FR1-FR1 CSI-RSRP absolute measurement accuracy for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	Message content is missing

-	Test procedure is missing

-	Test requirements are missing

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.6.6.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency CSI-RSRP absolute measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133 [6] clause 10.1.4.3.1.

19.6.6.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward supporting CSI-RS based RSRP and RSRQ measurement.

19.6.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 19.6.6.0.3

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.6.2.

19.6.6.2.1.1.4	Test description

19.6.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.6.6.2.1.4.1-1.

Table 19.6.6.2.1.4.1-1: Supported test canfigurations

| Configuration | Description |
| --- | --- |
| 19.6.6.2.1-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.6.2.1-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.6.2.1-3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.6.6.2.1.4.1-2.

Table 19.6.6.2.1.4-3: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 19.6.6.2.1.4.3.

2.	There are two cells on different frequencies. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-RSRP measurements.

3.	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

4.	During the test, the specific gNB reference location is emulated by test system.

5.	The initial test environment conditions are setup according to section 19.0.1.

19.6.6.2.1.4.2	Test procedure

<< Unchanged Parts Skipped >>

19.6.6.2.2	NR SA FR1-FR1 CSI-RSRP relative measurement accuracy

Editor’s Note: This test case is incomplete. The following still is missing:

- Message content is missing

- Test procedure is missing

- Cell mapping is missing

- Antenna connection diagram is missing

- Applicability is missing

- TT is missing

19.6.6.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency CSI-RSRP relative measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133 [6] clause 10.1.4.3.2.

19.6.6.2.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward supporting CSI-RS based RSRP and RSRQ measurement.

19.6.6.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 19.6.6.0.4

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.6.2.

19.6.6.2.2.1.4	Test description

19.6.6.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.6.6.2.2.4.1-1.

Table 19.6.6.2.2.4.1-1: Supported test canfigurations

| Configuration | Description |
| --- | --- |
| 19.6.6.2.2-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.6.2.2-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.6.2.2-3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.6.6.2.2.4.1-2.

Table 19.6.6.2.2.4-3: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.6.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 19.6.6.2.2.4.3.

2.	There are two cells on different frequencies. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-RSRP measurements.

3.	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

4.	During the test, the specific gNB reference location is emulated by test system.

5.	The initial test environment conditions are setup according to section 19.0.1.

19.6.6.2.2.4.2	Test procedure

<< Unchanged Parts Skipped >>

19.6.7.1	NR SA FR1 CSI-RSRQ absolute measurement accuracy for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	Test requirements are missing

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.6.7.1.1	Test purpose

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] sub-clause 10.1.7.2 for CSI-RS intra-frequency measurements.

19.6.7.1.2			Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward supporting CSI-RS based RSRP and RSRQ measurement.

19.6.7.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in Clause 19.6.7.0.1.

The normative reference for this requirement is in TS 38.133 [6] A.19.6.7.1.

19.6.7.1.4	Test description

19.6.7.1.4.1	Initial conditions

Supported test configuration are shown in Table 19.6.7.1.4.1-1.

Table 19.6.7.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 19.6.7.1-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.7.1-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.7.1-3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.6.7.1.4.1-2.

Table 19.6.7.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] Clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] Clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.7.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | Without the LTE link |  |  |

1.	Message contents are defined in Clause 19.6.7.2.4.3.

2.	In this test case all cells are on the same carrier frequency. Cell 1 is the NR serving cell (PCell). Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for CSI-RSRQ measurements..

3.	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

4.	During the test, the specific gNB reference location is emulated by test system.

5.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.7.2.1	NR SA FR1-FR1 CSI-RSRQ absolute measurement accuracy for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	Message content is missing

-	Test procedure is missing

-	Test requirements are missing

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.6.7.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency CSI-RSRQ absolute measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clauses 10.1.9.2.1.

19.6.7.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward supporting CSI-RS based RSRP and RSRQ measurement.

19.6.7.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 19.6.7.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.7.2.

19.6.7.2.1.1.4	Test description

19.6.7.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.6.7.2.1.4.1-1.

Table 19.6.7.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.6.7.2.1-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.7.2.1-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.7.2.1-3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.6.7.2.1.4.1-2.

Table 19.6.7.2.1.4-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.7.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 19.6.7.2.1.4.3.

2.	There are two cells on the same frequency. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-RSRP measurements.

3.	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

4.	During the test, the specific gNB reference location is emulated by test system.

5.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.7.2.2	NR SA FR1-FR1 CSI-RSRQ relative measurement accuracy for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	Message content is missing

-	Test procedure is missing

-	Test requirements are missing

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.6.7.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency CSI-RSRQ relative measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clauses 10.1.9.2.2.

19.6.7.2.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward supporting CSI-RS based RSRP and RSRQ measurement.

19.6.7.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 19.6.7.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.7.2.

19.6.7.2.2.1.4	Test description

19.6.7.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.6.7.2.2.4.1-1.

Table 19.6.7.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.6.7.2.2-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.7.2.2-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.7.2.2-3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.6.7.2.2.4.1-2.

Table 19.6.7.2.2.4-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.7.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 19.6.7.2.2.4.3.

2.	There are two cells on the same frequency. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-RSRQ measurements.

3.	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

4.	During the test, the specific gNB reference location is emulated by test system.

5.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.8.1	NR SA FR1 CSI-SINR absolute measurement accuracy for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	Test requirements are missing

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.6.8.1.1	Test purpose

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] sub-clause 10.1.12.2.1 for CSI-RS intra-frequency measurements.

19.6.8.1.2			Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward supporting CSI-RS based SINR measurements.

19.6.8.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in Clause 19.6.8.0.1.

The normative reference for this requirement is in TS 38.133 [6] A.19.6.8.1.

19.6.8.1.4	Test description

19.6.8.1.4.1	Initial conditions

Supported test configuration are shown in Table 19.6.8.1.4.1-1.

Table 19.6.8.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 19.6.8.1-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.8.1-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.8.1-3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.6.8.1.4.1-2.

Table 19.6.8.1.4.1-2: Initial conditions for CSI-SINR intra frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] Clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] Clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.8.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1.	Message contents are defined in Clause 19.6.8.1.4.3.

2.	In this test case all cells are on the same carrier frequency. Cell 1 is the NR serving cell (PCell). Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for CSI-RSRQ measurements..

3.	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

4.	During the test, the specific gNB reference location is emulated by test system.

5.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.8.2.1	NR SA FR1-FR1 CSI-SINR absolute measurement accuracy for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	Message content is missing

-	Test procedure is missing

-	Test requirements are missing

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.6.8.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency CSI-SINR absolute measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clauses 10.1.14.2.1.

19.6.8.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward supporting CSI-RS based SINR measurements.

19.6.8.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 19.6.8.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.8.2.

19.6.8.2.1.1.4	Test description

19.6.8.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.6.8.2.1.4.1-1.

Table 19.6.8.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.6.8.2.1-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.8.2.1-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.8.2.1-3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.6.8.2.1.4.1-2.

Table 19.6.8.2.1.4-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.8.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 19.6.8.2.1.4.3.

2.	There are two cells on the same frequency. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-SINR measurements.

3.	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

4.	During the test, the specific gNB reference location is emulated by test system.

5.	The initial test environment conditions are setup according to section 19.0.1.

<< Unchanged Parts Skipped >>

19.6.8.2.2	NR SA FR1-FR1 CSI-SINR relative measurement accuracy for ATG

Editor’s Note: This test case is incomplete in following aspects.

-	Message content is missing

-	Test procedure is missing

-	Test requirements are missing

- 	TT analysis is missing

-	AT commands for UE positioning and UE speed setting are FFS.

-	The specific gNB reference location emulated by test system is FFS.

-	Test procrdures for providing network assistance on ATG cells reference locations are FFS.

19.6.8.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency CSI-SINR relative measurement accuracy is within the specified limits. This test will verify the requirements in TS 38.133 [6] clauses 10.1.14.2.2.

19.6.8.2.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 and forward supporting CSI-RS based SINR measurements.

19.6.8.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 19.6.8.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.8.2.

19.6.8.2.2.1.4	Test description

19.6.8.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 19.6.8.2.2.4.1-1.

Table 19.6.8.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 19.6.8.2.2-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.8.2.2-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.8.2.2-3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 19.6.8.2.2.4.1-2.

Table 19.6.8.2.2.4-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.8.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 19.6.8.2.2.4.3.

2.	There are two cells on the same frequency. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-SINR measurements.

3.	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

4.	During the test, the specific gNB reference location is emulated by test system.

5.	The initial test environment conditions are setup according to section 19.0.1.

<< END OF CHANGES >>