Reason for change:

In slot#0, it is not possible to send paging to UE as it will clash with the SSB blocks. So for SSB#0 when UE is in RRC_IDLE or RRC_INACTIVE, paging shall be changed to slot#1.

Summary of changes:

Corrected paging slot to slot#1 for SSB#0

### 7.3.4	Paging and Short Message

SS can be configured with a DCI including Short Messages. In that case, when SS is triggered to transmit a Paging message, the Short Message indication shall be included in the DCI.

SS can be triggered to transmit Short Messages alone, this is achieved in TTCN with the DciTrigger type. SS is triggered to send the Paging message or a Short Message indication at a calculated Paging Occasion provided in the activation time and optionally a list slot offsets when multiple paging occasions are applied. Discontinuous Reception (DRX) is applied for the transmission of a Paging message or a Short Message indication to the UE in the RRC_IDLE or RRC_INACTIVE states. The paging frame calculation is according to TS 38.304 [24] clause 7. The following default values are provided in TS 38.508-1[5]:

-	defaultPagingCycle = 128

-	Ns = one

-	N = 64

-	PF_Offset = 0

When these default values are applied, the Paging Frame calculation is:

-	Paging Frame: SFN mod 128 =2*(UE_ID mod 64) so depending on UE_ID, the Paging Frame is set to 0,2,4,6,… 126 of the  paging cycle.

When UE is in RRC_IDLE or RRC_INACTIVE and when PDCCH monitoring occasions with default association is applied, the Paging Occasion is set to:

-	slot#2 of the Paging Frame when the cell is configured with SSB#1 or

-	slot#1 of the Paging Frame when the cell is configured with SSB#0.

When UE is in RRC_CONNECTED and when PDCCH monitoring occasions with default association is applied, the Paging Occasion is set to:

-	slot#1 and slot#2 of the Paging Frame when the cell is configured with SSB#1 or

-	slot#0 and slot#1 of the Paging Frame when the cell is configured with SSB#0.