3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210179

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1374 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test cases 8.1.1.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - PDU session info is sent twice to EUTRA, once from the test case and then again inside f_NR5GC_TrackingAreaUpdateFromN1() - No EUTRA DRB is configured on the SS before the call to f_EUTRA_CheckDataPath() |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove call to f_NR5GC_SendPduSessionInfoListToEUTRA before step 2 - Add SS configuration for EUTRA DRB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_1_3_4_TestBody |
| --- | --- |
| Reason for change | PDU session info is sent twice to EUTRA, once from the test case and then again inside f_NR5GC_TrackingAreaUpdateFromN1() |
| Summary of change | Remove call to f_NR5GC_SendPduSessionInfoListToEUTRA before step 2 |
| TTCN module | RRC_ConnRel_NR5GC_NR |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_1_3_4_TestBody(EUTRA_NR_CoOrd_SysInfo_Type p_CoOrd_SysInfo) runs on NR5GC_PTC { var template (value) CellReselectionPriorities v_CellReselectionPriorities; var PDUSessionInfoList_Type v_PDUSessionInfoList; var integer v_DRBId; //@siclog "Step 1" siclog@ f_NR_SetCellPower (nr_Cell1, -88, -88); // FR2 FFS f_NR5GC_SendPduSessionInfoListToEUTRA(); // @sic R5-204399 sic@ //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message containing the IE freqPriorityListEUTRA v_CellReselectionPriorities:= cs_CellReselectionPriorities({cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[0].Arfcn, 3), - cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[1].Arfcn, 5)}, {cs_FreqPriorityNR(f_NR_CellInfo_GetFrequencySSB(nr_Cell1), 4)}, omit); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, v_CellReselectionPriorities)); // @sic R5-202623 sic@ //@siclog "Step 3" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); //@siclog "Step 5" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Step 4"); f_NR5GC_IRAT_S1ToN1_IdleMode (nr_Cell1, noRrcConnectionRelease, false); // @sic R5-202623 sic@ //@siclog "Step 5AA - 5AB" siclog@ f_NR_RRCReconfigExistingDRBs(nr_Cell1, omit); // @sic R5-202623 sic@ // @siclog "Step 5A" siclog@ v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), defaultBearer, v_DRBId, "Step 5A"); //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message containing the IE freqPriorityListEUTRA and T320 v_CellReselectionPriorities:= cs_CellReselectionPriorities({cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[0].Arfcn, 3)}, {cs_FreqPriorityNR(f_NR_CellInfo_GetFrequencySSB(nr_Cell1), 4)}, // @sic R5-202623 sic@ min5); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, v_CellReselectionPriorities)); // @sic R5-202623 sic@ //@siclog "Step 7" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Step 7")); //@siclog "Step 8" siclog@ if (not f_NG_CheckNoRRCSetupReq(nr_Cell1, f_NR_SetTimerToleranceMin (nr_Cell1, rrcTimer, 300.0))) { // @sic R5-202623 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } //@siclog "Step 9" siclog@ // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }

After Change

function fl_TC_8_1_1_3_4_TestBody(EUTRA_NR_CoOrd_SysInfo_Type p_CoOrd_SysInfo) runs on NR5GC_PTC { var template (value) CellReselectionPriorities v_CellReselectionPriorities; var PDUSessionInfoList_Type v_PDUSessionInfoList; var integer v_DRBId; //@siclog "Step 1" siclog@ f_NR_SetCellPower (nr_Cell1, -88, -88); // FR2 FFS //REMOVE f_NR5GC_SendPduSessionInfoListToEUTRA(); // @sic R5-204399 sic@ //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message containing the IE freqPriorityListEUTRA v_CellReselectionPriorities:= cs_CellReselectionPriorities({cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[0].Arfcn, 3), cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[1].Arfcn, 5)}, {cs_FreqPriorityNR(f_NR_CellInfo_GetFrequencySSB(nr_Cell1), 4)}, omit); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, v_CellReselectionPriorities)); // @sic R5-202623 sic@ //@siclog "Step 3" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); //@siclog "Step 5" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Step 4"); f_NR5GC_IRAT_S1ToN1_IdleMode (nr_Cell1, noRrcConnectionRelease, false); // @sic R5-202623 sic@ //@siclog "Step 5AA - 5AB" siclog@ f_NR_RRCReconfigExistingDRBs(nr_Cell1, omit); // @sic R5-202623 sic@ // @siclog "Step 5A" siclog@ v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), defaultBearer, v_DRBId, "Step 5A"); //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message containing the IE freqPriorityListEUTRA and T320 v_CellReselectionPriorities:= cs_CellReselectionPriorities({cs_FreqPriorityEUTRA_NRDef(p_CoOrd_SysInfo.Eutra[0].Arfcn, 3)}, {cs_FreqPriorityNR(f_NR_CellInfo_GetFrequencySSB(nr_Cell1), 4)}, // @sic R5-202623 sic@ min5); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, -, v_CellReselectionPriorities)); // @sic R5-202623 sic@ //@siclog "Step 7" siclog@ // NR power level (for FR1, FR2 FFS) doesn't change f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Step 7")); //@siclog "Step 8" siclog@ if (not f_NG_CheckNoRRCSetupReq(nr_Cell1, f_NR_SetTimerToleranceMin (nr_Cell1, rrcTimer, 300.0))) { // @sic R5-202623 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } //@siclog "Step 9" siclog@ // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }

## Change 2

| Function name | fl_TAU_N1ToS1_CheckDataPath |
| --- | --- |
| Reason for change | No EUTRA DRB is configured on the SS before the call to f_EUTRA_CheckDataPath() |
| Summary of change | Add SS configuration for EUTRA DRB |
| TTCN module | RRC_ConnRel_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change

function fl_TAU_N1ToS1_CheckDataPath(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, charstring p_StepNo1 := "", charstring p_StepNo2 := "", template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific, boolean p_FirstN1_S1Transition := true) runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var template (value) RadioBearerId_Type v_DRBId; var EUTRA38_DRBInfoList_Type v_ExistingDRBs; var DRB_ToAddModList v_DRB_ToAddModList; var template (omit) SRB_ToAddModList v_SRB_ToAddModList := {cs_508_SRB2_ToAddMod_DEFAULT}; var integer i; v_NasInd := f_EUTRA38_TAUReqFromN1 (p_CellId, p_RRC_TI, -, -, p_CellIdPreviouslyCampedOn, -, p_FirstN1_S1Transition); // @sic R5s201062 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, p_StepNo1); v_SecurityParams := f_EUTRA_Security_Get(); v_SecurityParams := f_EUTRA_RRC_ActivateSecurity(p_CellId, v_SecurityParams, v_NasInd.SecurityProtection.NasCount, omit); // @sic R5s200878, R5-204399 sic@ f_EUTRA_Security_Set(v_SecurityParams); //Saving security parameters f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(p_CellId, v_NasInd, -, noRrcConnectionRelease); // @sic R5-204399 sic@ get all DRBs transferred from NR v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); for (i:=0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DRB_ToAddModList[i] := v_ExistingDRBs[i].DRB_ToAdd; } f_EUTRA38_MultiPDN_RRCConnectionReconfiguration_Common(p_CellId, p_RRC_TI, v_SRB_ToAddModList, v_DRB_ToAddModList); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(p_CellId, p_RRC_TI); v_DRBId := cs_RadioBearerId_DRB(v_ExistingDRBs[0].DRB_Id); f_EUTRA_CheckDataPath(p_CellId, PDCP, defaultBearer, p_StepNo2, v_DRBId, -, true); // @sic R5-204439 sic@ }

After Change

function fl_TAU_N1ToS1_CheckDataPath(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, charstring p_StepNo1 := "", charstring p_StepNo2 := "", template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific, boolean p_FirstN1_S1Transition := true) runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var template (value) RadioBearerId_Type v_DRBId; var EUTRA38_DRBInfoList_Type v_ExistingDRBs; var DRB_ToAddModList v_DRB_ToAddModList; var template (omit) SRB_ToAddModList v_SRB_ToAddModList := {cs_508_SRB2_ToAddMod_DEFAULT}; var integer i; v_NasInd := f_EUTRA38_TAUReqFromN1 (p_CellId, p_RRC_TI, -, -, p_CellIdPreviouslyCampedOn, -, p_FirstN1_S1Transition); // @sic R5s201062 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, p_StepNo1); v_SecurityParams := f_EUTRA_Security_Get(); v_SecurityParams := f_EUTRA_RRC_ActivateSecurity(p_CellId, v_SecurityParams, v_NasInd.SecurityProtection.NasCount, {cs_OneDRB_ConfigAM (tsc_DRB2)}); // @sic R5s200878, R5-204399 sic@ f_EUTRA_Security_Set(v_SecurityParams); //Saving security parameters f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(p_CellId, v_NasInd, -, noRrcConnectionRelease); // @sic R5-204399 sic@ get all DRBs transferred from NR v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); for (i:=0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DRB_ToAddModList[i] := v_ExistingDRBs[i].DRB_ToAdd; } f_EUTRA38_MultiPDN_RRCConnectionReconfiguration_Common(p_CellId, p_RRC_TI, v_SRB_ToAddModList, v_DRB_ToAddModList); f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(p_CellId, p_RRC_TI); v_DRBId := cs_RadioBearerId_DRB(v_ExistingDRBs[0].DRB_Id); f_EUTRA_CheckDataPath(p_CellId, PDCP, defaultBearer, p_StepNo2, v_DRBId, -, true); // @sic R5-204439 sic@ }