3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210186

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1395 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GMM test case 9.1.5.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During this test case the function f_NR5GC_PDUSessionEstablishment is called at several steps following the completion of the registration procedure. This function will only consider PDU session establishment based on pc_noOf_PDUsSameConnection, and will exit immediately if this is set to 0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace f_NR5GC_PDUSessionEstablishment with f_NR5GC_RRC_Idle_Steps16_20 at step 24, 46, 68. This function will also handle PDU session establishment based on pc_noOf_PDUsNewConnection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case in the configuration pc_noOf_PDUsSameConnection=0 pc_noOf_PDUsNewConnection>0 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_9_1_5_1_1_Body |
| --- | --- |
| Reason for change | During this test case the function f_NR5GC_PDUSessionEstablishment is called at several steps following the completion of the registration procedure. This function will only consider PDU session establishment based on pc_noOf_PDUsSameConnection, and will exit immediately if this is set to 0. |
| Summary of change | Replace f_NR5GC_PDUSessionEstablishment with f_NR5GC_RRC_Idle_Steps16_20 at step 24, 46, 68. This function will also handle PDU session establishment based on pc_noOf_PDUsNewConnection. |
| TTCN module | Initial_Registration_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_9_1_5_1_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellH := f_NR5GC_MapNASCell(ngc_CellH, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; …….. //@siclog Steps 24 - 27a1 siclog@ // No SRB2 so must be on SRB1 f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); //@siclog "Step 28" siclog@ //Prose does not mention to handle the DEREGISTRATION, a Prose CR is needed. f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_CONNECTED_3A); //@siclog "Step 29" siclog@ …… //@siclog Steps 46 - 49a1 siclog@ // No SRB2 so must be on SRB1 f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellH, -, tsc_NR_RbId_SRB1); //@siclog "Step 50" siclog@ //Prose does not mention to handle the DEREGISTRATION, a Prose CR is needed. f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellH, STATE_CONNECTED_3A); ………. //@siclog Steps 68 - 71a1 siclog@ // No SRB2 so must be on SRB1 f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); //@siclog "Step 72" siclog@ ……. } //End of fl_TC_9_1_5_1_1_Body

After Change

function fl_TC_9_1_5_1_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_CellsOnDifferentPLMN); var NR_CellId_Type v_NGC_NASCellH := f_NR5GC_MapNASCell(ngc_CellH, NG_CellsOnDifferentPLMN); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; …….. //@siclog Steps 24 - 27a1 siclog@ // No SRB2 so must be on SRB1 f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA); //KS R5sxxx change_91511 //@siclog "Step 28" siclog@ //Prose does not mention to handle the DEREGISTRATION, a Prose CR is needed. f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_CONNECTED_3A); //@siclog "Step 29" siclog@ …… //@siclog Steps 46 - 49a1 siclog@ // No SRB2 so must be on SRB1 f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellH);//KS R5s-xxxxx //@siclog "Step 50" siclog@ //Prose does not mention to handle the DEREGISTRATION, a Prose CR is needed. f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellH, STATE_CONNECTED_3A); ………. //@siclog Steps 68 - 71a1 siclog@ // No SRB2 so must be on SRB1 f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA); //KS R5sxxx change_91511 //@siclog "Step 72" siclog@ ……. } //End of fl_TC_9_1_5_1_1_Body