3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210201

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1425 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Change 2 of CR R5s200995 removed the starting of UL grant from the test body function f_TC_7_1_1_5_1_NR_TestBody(). For ENDC UL grant needs to be started in the function f_TC_7_1_1_5_1_ENDC_NR() |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UL grant configuration before f_TC_7_1_1_5_1_NR_TestBody() to match NR5GC operation. Note : This was change 3 of R5s200995, accepted but not applied in the subsequent delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case for EN-DC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 	Change 1

| Function name | function f_TC_7_1_1_5_1_ENDC_NR() |
| --- | --- |
| Reason for change | Change 2 of CR R5s200995 removed the starting of UL grant from the test body function f_TC_7_1_1_5_1_NR_TestBody(). For ENDC UL grant needs to be started in the function f_TC_7_1_1_5_1_ENDC_NR() |
| Summary of change | Add UL grant configuration before f_TC_7_1_1_5_1_NR_TestBody() to match NR5GC operation. Note : This was change 3 of R5s200995, accepted but not applied in the subsequent delivery. |
| TTCN module | DRX_MAC_ENDC_NR |
| MCC160 Comment |  |

Before change:

function f_TC_7_1_1_5_1_ENDC_NR() runs on ENDC_NR_PTC

{// DRX operation / Short cycle not configured / Parameters configured by RRC

var DRB_Identity v_DRBId :=tsc_NR_DRB2;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)};

var template (value) CellGroupConfig v_CellGroupConfig;

//Init Cell parameters

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only);

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, v_SS_Drb_ConfigList);

// Wait for Trigger from EUTRA indicating start of Test Body

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_TestBody_Set(true);

//@siclog steps 1-2 logsic@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1);

v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup ( cs_NR_DRX_Config_Def( cs_NR_DRX_OnDurationTimerMs20, ms10, 56, 56, sl8, sl8, cs_NR_DRX_LongCycleStartOffsetMs640_7, -, 0));

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG,

-,

-,

v_MAC_CellGroupConfig);

f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig);

v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList);

//@siclog steps 3-20 logsic@

f_TC_7_1_1_5_1_NR_TestBody(v_DRBId);

f_NR_TestBody_Set(false);

// Send Trigger to EUTRA indicating Finish of Test Body

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger);

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

}

After change:

function f_TC_7_1_1_5_1_ENDC_NR() runs on ENDC_NR_PTC

{// DRX operation / Short cycle not configured / Parameters configured by RRC

var DRB_Identity v_DRBId :=tsc_NR_DRB2;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)};

var template (value) CellGroupConfig v_CellGroupConfig;

//Init Cell parameters

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only);

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, v_SS_Drb_ConfigList);

// Wait for Trigger from EUTRA indicating start of Test Body

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_TestBody_Set(true);

//@siclog steps 1-2 logsic@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1);

v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup ( cs_NR_DRX_Config_Def( cs_NR_DRX_OnDurationTimerMs20, ms10, 56, 56, sl8, sl8, cs_NR_DRX_LongCycleStartOffsetMs640_7, -, 0));

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG,

-,

-,

v_MAC_CellGroupConfig);

f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig);

f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, -,-, cs_NR_UplinkTimeAlignment_Stop);

v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList);

//@siclog steps 3-20 logsic@

f_TC_7_1_1_5_1_NR_TestBody(v_DRBId);

f_NR_TestBody_Set(false);

// Send Trigger to EUTRA indicating Finish of Test Body

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger);

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

}