3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210718

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1791 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC idle mode test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following R5-211378 and R5-213446, the need to clear RPLMN information for these test cases is not referenced in the pre-test conditions as specific values for LOCI fields were added to these SIM profiles. Specifying values for these fields is problematic because the UE can update these files during test case execution (after registering to any PLMN), meaning that at the end of the test case the USIM is no longer compliant to the profile and cannot be used for a further execution without re-flashing the card. This places extra requirements on test operators to re-flash the card between test executions which is not required for other USIM profiles and should be avoided. For this reason it is proposed to reinstate the clearing of RPLMN information in the UE through registration reject signalling in the preamble. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reinstate the call to f_NR5GC_PreambleReject() to clear the UE's Last Registered PLMN (RPLMN) using Registration Reject procedure as defined in TS 38.508-1 [4] Subclause 4.9.8 Note : Associated prose CRs on 38.523-1 and 38.508-1 will be raised at RAN5#92 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases on subsequent executions due to USIM configuration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1.NR5GC, 6.2.1.1.NR5GC, 6.2.1.2.NR5GC, 6.2.1.3.NR5GC, 6.3.1.2.NR5GC, 6.3.1.1.NR5GC, 6.3.1.3.NR5GC, 6.3.1.4.NR5GC, 6.3.1.5.NR5GC, 6.3.1.7.NR5GC, 6.3.1.8.NR5GC and 6.3.1.9.NR5GC . |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1, 38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | e.g. f_TC_6_3_1_1_NR5GC() |
| --- | --- |
| Reason for change | Following R5-211378 and R5-213446, the need to clear RPLMN information for these test cases is not referenced in the pre-test conditions as specific values for LOCI fields were added to these SIM profiles. Specifying values for these fields is problematic because the UE can update these files during test case execution (after registering to any PLMN), meaning that at the end of the test case the USIM is no longer compliant to the profile and cannot be used for a further execution without re-flashing the card. This places extra requirements on test operators to re-flash the card between test executions which is not required for other USIM profiles and should be avoided. For this reason it is proposed to reinstate the clearing of RPLMN information in the UE through registration reject signalling in the preamble. |
| Summary of change | Reinstate the call to f_NR5GC_PreambleReject() to clear the UE's Last Registered PLMN (RPLMN) using Registration Reject procedure as defined in TS 38.508-1 [4] Subclause 4.9.8 Note : Same correction is required in test cases 6.1.1.1, 6.2.1.1, 6.2.1.2, 6.2.1.3, 6.3.1.2, 6.3.1.3, 6.3.1.4, 7.3.1.5, 6.3.1.7, 6.3.1.8 and 6.3.1.9 Note : Associated prose CRs on 38.523-1 and 38.508-1 will be raised at RAN5#92. |
| TTCN module |  |
| MCC160 Comment |  |

Before Change

function f_TC_6_3_1_1_NR5GC() runs on NR5GC_PTC

{ //Steering of UE in roaming during registration/security check successful using List Type 1

//@sic R5-198877 sic@

f_NR5GC_Init(NR_4); // @sic R5s201368 sic@

// Set Sys info and cell info for cell 11& 12 & 13 to be used

f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//PLMN2

f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_002_81);//PLMN13//@sic R5-202596 sic@

f_NR_CellInfo_SetPLMN(nr_Cell13, tsc_NR_HPLMN_002_91);//PLMN14//@sic R5-202596 sic@

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell11);

f_NR_CellConfig_Def(nr_Cell12);

f_NR_CellConfig_Def(nr_Cell13);

// Equip UE with the USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1

f_UT_USIM_Insert(UT, "USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1");

f_UT_SwitchOnUE(UT, false, CNF_REQUIRED);

// Ensure that the UE has cleared the RPLMN @sic R5-211378 sic@Delted in @sic R5-213457 sic@

f_UT_AutomaticPLMN_Select (UT);

f_UT_SwitchOffUE(UT, false, CNF_REQUIRED);

f_NR5GC_Preamble (nr_Cell13, STATE_OFF_0A);     // @sic R5-211378 sic@

f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_TestBody_Set(true);

fl_TC_6_3_1_1_TestBody();

f_NR_TestBody_Set(false);

// Finish

f_NR_Postamble(nr_Cell11, STATE_IDLE_1A);

}

After Change

function f_TC_6_3_1_1_NR5GC() runs on NR5GC_PTC

{ //Steering of UE in roaming during registration/security check successful using List Type 1

//@sic R5-198877 sic@

f_NR5GC_Init(NR_4); // @sic R5s201368 sic@

// Set Sys info and cell info for cell 11& 12 & 13 to be used

f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//PLMN2

f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_002_81);//PLMN13//@sic R5-202596 sic@

f_NR_CellInfo_SetPLMN(nr_Cell13, tsc_NR_HPLMN_002_91);//PLMN14//@sic R5-202596 sic@

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell11);

f_NR_CellConfig_Def(nr_Cell12);

f_NR_CellConfig_Def(nr_Cell13);

// Equip UE with the USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1

f_UT_USIM_Insert(UT, "USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1");

f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT);

//UE's Last Registered PLMN (RPLMN) needs to be cleared by using Registration Reject procedure as defined in TS 38.508-1 [4] Subclause 4.9.8

f_NR5GC_PreambleReject(nr_Cell13);

f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_SetCellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_TestBody_Set(true);

fl_TC_6_3_1_1_TestBody();

f_NR_TestBody_Set(false);

// Finish

f_NR_Postamble(nr_Cell11, STATE_IDLE_1A);

}