3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210846

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1862 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC testcases 8.2.4.3.1.1 and 8.2.4.3.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the current TTCN implementation, there is a redundant co-ord message waiting for “RB” parameters from NR PTC. However, this has already been received earlier. - At Step 7 – 8, the old SCell is being released which becomes the new neighbour cell. However, the local end config does not include the correct Cell ID to release the SCell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed the extra line of code waiting for “RB” parameters from NR PTC. - Included the correct Cell ID whilst releasing the SCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.3.1.1.ENDC, 8.2.4.3.1.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to function fl_TC_8_2_4_3_1_x_TestBody ()	3

1.2	Correction to function fl_TC_8_2_4_3_1_x_TestBody ()	4

# Corrections required

## Correction to function fl_TC_8_2_4_3_1_x_TestBody ()

| Function name | Function fl_TC_8_2_4_3_1_x_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, there is a redundant co-ord message waiting for “RB” parameters from NR PTC. However, this has already been received earlier. |
| Summary of change | Removed the extra line of code waiting for “RB” parameters from NR PTC. |
| TTCN module | RRCMeasurement_CA_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_4_3_1_x_TestBody() runs on EUTRA_5GS_PTC { var template (omit) octetstring v_NR_SCGConfig := omit; var template (omit) octetstring v_NR_RBConfig := omit; // @sic R5s210453, R5-213480 sic@ var EUTRA38_SecurityParamsExtd_Type v_Security_Params; //Send a trigger to NR to change the power f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("ChangePower")); //@siclog "Steps 1-2" siclog@ //SS transmits an RRCConnectionReconfiguration message containing RRCReconfiguration message to configure SCell (NR Cell 3) and SRB3 //The UE transmits an RRCConnectionReconfigurationComplete message containing RRCReconfigurationComplete message v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); //@sic R5s210453, R5-213480 sic@ f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); f_EUTRA38_508_ENDC_SendReconfiguration(eutra_Cell1, -, -, v_NR_SCGConfig, v_NR_RBConfig, //@sic R5s210453, R5-213480 sic@ -, omit); //@sic R5s210160, R5-210399 sic@ v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_Security_Set(v_Security_Params); <<SKIPPED CODE>>

After Change:

function fl_TC_8_2_4_3_1_x_TestBody() runs on EUTRA_5GS_PTC { var template (omit) octetstring v_NR_SCGConfig := omit; var template (omit) octetstring v_NR_RBConfig := omit; // @sic R5s210453, R5-213480 sic@ var EUTRA38_SecurityParamsExtd_Type v_Security_Params; //Send a trigger to NR to change the power f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("ChangePower")); //@siclog "Steps 1-2" siclog@ //SS transmits an RRCConnectionReconfiguration message containing RRCReconfiguration message to configure SCell (NR Cell 3) and SRB3 //The UE transmits an RRCConnectionReconfigurationComplete message containing RRCReconfigurationComplete message v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); //@sic R5s210453, R5-213480 sic@ //REMOVED f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); f_EUTRA38_508_ENDC_SendReconfiguration(eutra_Cell1, -, -, v_NR_SCGConfig, v_NR_RBConfig, //@sic R5s210453, R5-213480 sic@ -, omit); //@sic R5s210160, R5-210399 sic@ v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_Security_Set(v_Security_Params); <<SKIPPED CODE>>

## Correction to function fl_TC_8_2_4_3_1_x_TestBody ()

| Function name | Function fl_TC_8_2_4_3_1_x_TestBody |
| --- | --- |
| Reason for change | At Step 7 – 8, the old SCell is being released which becomes the new neighbour cell. However, the local end config does not include the correct Cell ID to release the SCell. |
| Summary of change | Included the correct Cell ID whilst releasing the SCell |
| TTCN module | RRCMeasurement_CA_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_4_3_1_x_TestBody(NR_CellId_Type p_SCellId, NR_CellId_Type p_NeigbourCellId) runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@siclog "Steps 7-8" siclog@ //The SS transmits an RRCReconfiguration message including sCellToReleaseList with NR Cell 3 as SCell release and sCellToAddModList with NR Cell 12 as SCell addition via SRB3. //Check: Does the UE transmits an RRCConnectionReconfigurationComplete message on NR Cell 1? //Table 8.2.4.3.1.1.3.3-9: CellGroupConfig v_SCGConfig := f_NR_GetSCGConfig_SCellAdd(p_NeigbourCellId, tsc_SCellIndex_3); v_SCGConfig.rlc_BearerToAddModList := v_RLC_BearerConfigList; //@sic R5s201309 sic@ v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.servingCellMO := tsc_NR_MeasObjectId2; //same as measObjectId for cell 3, Table 8.2.4.3.1.1.3.3-6 @sic R5s201309 sic@ v_Timing_NextSendOccasion := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_Timing_AddSCell := f_SubFrameTiming_AddMilliSeconds(v_Timing_NextSendOccasion, 5); // 5 ms later //Release old SCell: now SCell becomes neighbour cell //f_NR_SS_CommonCellConfig(p_SCellId, cads_NR_ServingCellConfig_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing_AddSCell), cs_NR_ServingCellConfig_None)); <<SKIPPED CODE>>

After Change:

function fl_TC_8_2_4_3_1_x_TestBody(NR_CellId_Type p_SCellId, NR_CellId_Type p_NeigbourCellId) runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@siclog "Steps 7-8" siclog@ //The SS transmits an RRCReconfiguration message including sCellToReleaseList with NR Cell 3 as SCell release and sCellToAddModList with NR Cell 12 as SCell addition via SRB3. //Check: Does the UE transmits an RRCConnectionReconfigurationComplete message on NR Cell 1? //Table 8.2.4.3.1.1.3.3-9: CellGroupConfig v_SCGConfig := f_NR_GetSCGConfig_SCellAdd(p_NeigbourCellId, tsc_SCellIndex_3); v_SCGConfig.rlc_BearerToAddModList := v_RLC_BearerConfigList; //@sic R5s201309 sic@ v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.servingCellMO := tsc_NR_MeasObjectId2; //same as measObjectId for cell 3, Table 8.2.4.3.1.1.3.3-6 @sic R5s201309 sic@ v_Timing_NextSendOccasion := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_Timing_AddSCell := f_SubFrameTiming_AddMilliSeconds(v_Timing_NextSendOccasion, 5); // 5 ms later //Release old SCell: now SCell becomes neighbour cell f_NR_SS_CommonCellConfig(p_SCellId, cads_NR_ServingCellConfig_REQ(p_SCellId, cs_TimingInfo_NR(v_Timing_AddSCell), cs_NR_ServingCellConfig_None)); <<SKIPPED CODE>>