3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211392

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1833 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-DC test case 8.2.5.4.2 in FR1+FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR-DC TC 8.2.5.4.2 Reconfiguration failure / SCG change failure/ NR-DC |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR-DC test case 8.2.5.4.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications 38.523-1 |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches Executed	8

4	Execution Log Files	8

4.1	Qualcomm SM 8450 + SDX65	8

5	References	8

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 8.2.5.4.2 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.2.5.4.2

ATS Version:	iwd-TTCN3-B2020-09_D21wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM 8450 + SDX65

Verification Status:	PASS

# Corrections required

Change 1,5,6 of R5s210716(Addition of NR-DC test case 8.2.2.4.2 in FR1+FR2 ) are also required in addition to the below changes.

## Change 1

| Function name | f_TC_8_2_5_4_2_NR5GC() |
| --- | --- |
| Reason for change | OSI removal to align with prose. Also, FR2 Cell cannot be advertised on OSI as it cannot be a Stand Alone cell. |
| Summary of change | Changed system info to NR_1 when initializing the NR cells. |
| TTCN module | RRC_ReconfigFailure_NRDC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_2_5_4_2_NR5GC() runs on NR5GC_PTC { // Reconfiguration failure / SCG change failure / NR-DC f_NR5GC_Init_NRDC(NR_4, {nr_Cell1, nr_Cell10}); // System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.3 is used in NR cells. //Create and configure NR cell1 and NR cell 10. f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10); // The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC) with DC bearer(MCG and SCG) according to TS 38.508-1 [4], Table 4.5.1-1. f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC(nr_Cell1, nr_Cell10, MCG_SCG); f_NR_TestBody_Set(true); fl_TC_8_2_5_4_2_NR5GC_TestBody (); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_8_2_5_4_2_NR5GC() runs on NR5GC_PTC { // Reconfiguration failure / SCG change failure / NR-DC f_NR5GC_Init_NRDC(NR_1, {nr_Cell1, nr_Cell10}); // System information combination NR-4 as defined in TS 38.508-1 clause 4.4.3.1.3 is used in NR cells. //Create and configure NR cell1 and NR cell 10. f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell10); // The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC) with DC bearer(MCG and SCG) according to TS 38.508-1 [4], Table 4.5.1-1. f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower(nr_Cell10, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_RRC_ConnectedState3N_NRDC(nr_Cell1, nr_Cell10, MCG_SCG); f_NR_TestBody_Set(true); fl_TC_8_2_5_4_2_NR5GC_TestBody (); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

## Change 2

| Function name | fl_TC_8_2_5_4_2_TestBody() |
| --- | --- |
| Reason for change | The variables v_IsFR1, v_ParametersTX, v_ParametersRx, v_RLC_BearerConfig, v_SpCellConfig, v_MAC_CellGroupConfig, and v_PhysicalCellGroupConfig are getting configured based on nr_Cell1 (instead of nr_Cell10) and are redundant because the variable v_SCGConfig is already present. To align to TS38.523 Table 8.2.5.4.2.3.2-1 Step 2, nr_Cell10 (not nr_Cell1) must be configured to not respond to RACH preambles. The original TTCN configures the function f_NR_SendRRCReconfiguration_NRDC to wait for ReconfigurationComplete message and check for successful RACH at the same time. This defeats the purpose of the test. |
| Summary of change | Removed variables v_IsFR1, v_ParametersTX, v_ParametersRx, v_RLC_BearerConfig, v_SpCellConfig, v_MAC_CellGroupConfig, and v_PhysicalCellGroupConfig. Conifigured nr_Cell10 to not respond to RACH preamble. Configured f_NR_SendRRCReconfiguration_NRDC to not check for RACH and to handle RRCReconfigurationComplete outside f_NR_SendRRCReconfiguration_NRDC. |
| TTCN module | RRC_ReconfigFailure_NRDC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_5_4_2_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var DRB_Identity v_DrbId := f_NR_GetDefaultDRB_SCG(); var float v_T304Min_1sec := f_NR_SetTimerToleranceMin (nr_Cell1, rrcTimer, 1.0); var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1); var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1); var template (value) RLC_BearerConfig v_RLC_BearerConfig; var template (value) SpCellConfig v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); timer t_T304Expiry; timer t_WatchDog; f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); // @siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell. // Table 8.2.5.4.2.3.3-3 v_SpCellConfig.reconfigurationWithSync.t304 := ms1000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; v_SpCellConfig.reconfigurationWithSync.newUE_Identity := f_NR_CellInfo_GetRNTI(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n200; // Table 8.2.5.4.2.3.3-7 v_RLC_BearerConfig := cs_38508_RLC_BearerConfig_DRB(v_DrbId, cs_38508_RLC_Config_AM_DRB(v_ParametersTX, v_ParametersRX), cs_38508_LogicalChannelConfig_DRB); v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, {v_RLC_BearerConfig}, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(v_SCGConfig); t_T304Expiry.start(v_T304Min_1sec); // t_T304Expiry will expire v_T304Min_1sec after RRCConnectionReconfiguration has been sent t_WatchDog.start(v_T304Min_1sec + 5.0); // 5.0s for the UE to send FailureReportSCG only after expiry of T304 f_NR_SendRRCReconfiguration_NRDC (nr_Cell1, nr_Cell10,-, v_V1530Ext,-,-,-,noRACH_Check); … }

After Change:

function fl_TC_8_2_5_4_2_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var DRB_Identity v_DrbId := f_NR_GetDefaultDRB_SCG(); var float v_T304Min_1sec := f_NR_SetTimerToleranceMin (nr_Cell1, rrcTimer, 1.0); timer t_T304Expiry; timer t_WatchDog; f_NR_SS_RachProcedureConfig(nr_Cell10, cs_NR_RachProcedureConfig_NoResponse); // @siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell. // Table 8.2.5.4.2.3.3-3 v_SCGConfig := f_NR_GetSCGConfigNRDC_SCG(nr_Cell10, v_DrbId, -); v_SCGConfig.spCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n200; // Table 8.2.5.4.2.3.3-7 v_V1530Ext := f_NRDC_BuildRRCReconfiguration_v1530(v_SCGConfig); t_T304Expiry.start(v_T304Min_1sec); // t_T304Expiry will expire v_T304Min_1sec after RRCConnectionReconfiguration has been sent t_WatchDog.start(v_T304Min_1sec + 5.0); // 5.0s for the UE to send FailureReportSCG only after expiry of T304 f_NR_SendRRCReconfiguration_NRDC (nr_Cell1, nr_Cell10,-, v_V1530Ext,-,-,-,-,false); SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); … }

# Branches Executed

The test case was executed on NR band DC_n78A_n257A.

# Execution Log Files

## Qualcomm SM 8450 + SDX65

The Qualcomm SM 8450 + SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_2_5_4_2_LOG.xml

- PICS/PIXIT parameter file:

TC_8_2_5_4_2_PICS_PIXIT.txt

# References

| [1] | R5s210803 |
| --- | --- |