3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220091

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2295 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the current TTCN implementation , in the TAU REQ expected at Step 9, the TTCN expects the ePCO bit set to 1 in the IE UE Network Capability (Octet 8, bit 8). According to 24.301 cl 5.5.3.2.2, Quote If the UE supports NB-S1 mode, Non-IP PDN type, or N1 mode, then the UE shall support the extended protocol configuration options IE. Unquote Also, according to 24.501 cl 4.8.4, Quote If the UE is capable of both N1 mode and S1 mode, when the UE needs to use one or more functionalities not supported in 5GS but supported in EPS and the UE is in 5GMM-IDLE mode, the UE may disable the N1 mode capability for 3GPP access (see subclause 4.9.2). Unquote Thus, when a UE disables N1 mode capability, it can set the ePCO to not supported. Current TTCN always expects the UE to set the ePCO bit to “supported”, and can fail a conformant UE. This needs to be corrected |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_EUTRA38_TAUReqFromN1, set v_UENWCap.cIoT_proSeCap := ? for the case where establishment cause is set as emergency. Please see below for detailed change. Note : A Prose CR will raised for this at RAN5#94e |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## Change 1

| Function name | f_EUTRA38_TAUReqFromN1 |
| --- | --- |
| Reason for change | According to the current TTCN implementation , in the TAU REQ expected at Step 9, the TTCN expects the ePCO bit set to 1 in the IE UE Network Capability (Octet 8, bit 8). According to 24.301 cl 5.5.3.2.2, Quote If the UE supports NB-S1 mode, Non-IP PDN type, or N1 mode, then the UE shall support the extended protocol configuration options IE. Unquote Also, according to 24.501 cl 4.8.4, Quote If the UE is capable of both N1 mode and S1 mode, when the UE needs to use one or more functionalities not supported in 5GS but supported in EPS and the UE is in 5GMM-IDLE mode, the UE may disable the N1 mode capability for 3GPP access (see subclause 4.9.2). Unquote Thus, when a UE disables N1 mode capability, it can set the ePCO to not supported. Current TTCN always expects the UE to set the ePCO bit to “supported”, and can fail a conformant UE. This needs to be corrected |
| Summary of change | In the function f_EUTRA38_TAUReqFromN1, set v_UENWCap.cIoT_proSeCap := ? for the case where establishment cause is set as emergency. Please see below for detailed change. Note : A Prose CR will raised for this at RAN5#94e |
| TTCN module | EUTRA38_NR5GCProcedures.ttcn |
| MCC160 Comment |  |

Before Change

<<SKIPPED CODE>> if (valueof(p_EstablishmentCause) == emergency) { // @sic R5-217714 sic@ v_UENWCap.cIoT_V2X := ?; } <<SKIPPED CODE>>

After change

<<SKIPPED CODE>> if (valueof(p_EstablishmentCause) == emergency) { // @sic R5-217714 sic@ v_UENWCap.cIoT_V2X := ?; v_UENWCap.cIoT_proSeCap := ? //WA#11_1_7 : Prose CR needed } <<SKIPPED CODE>>