3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220095

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2297 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test cases 7.1.1.4.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current implementation in the main loop of this test csase is, on received Scheduling Request indication, to activate the UL grant and then disable SR indications. It is observed that the delay in disabling SR indication can cause the test case to often fail due to a 2nd SR indication being received at TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the disable of scheduling request system indication to before configuring UL grant, for increased reliability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.1.ENDC, 7.1.1.4.2.1.NR5GC, 7.1.1.4.2.3.ENDC, 7.1.1.4.2.3.NR5GC, 7.1.1.4.2.5.ENDC, 7.1.1.4.2.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody() |
| --- | --- |
| Reason for change | Current implementation in the main loop of this test csase is, on received Scheduling Request indication, to activate the UL grant and then disable SR indications. It is observed that the delay in disabling SR indication can cause the test case to often fail due to a 2nd SR indication being received at TTCN. |
| Summary of change | Move the disable of scheduling request system indication to before configuring UL grant, for increased reliability. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, integer p_Max_Imcs := 28, //@sic R5s201144sic@ boolean p_TransformPrecoder := false) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection */ ……………………………… //@siclog "Step 4" siclog@ //v_Timing := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_Timing, 60); t_Watchdog.start; alt { [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { f_NR_ULGrantConfiguration_ULTBS(nr_Cell1, cs_TimingInfo_Now, v_I_MCS, v_N_PRB, int2bit(v_TDRA, 4), resourceAllocationType1); //@sic R5s190895 sic@ //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); f_NR_Rx_N_PDCP_SDUs_InOneTTI(v_NR_PDCP_SDUList, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } }; }//End for TDRA 0 to 13 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_2_1

After Change

function f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, integer p_Max_Imcs := 28, //@sic R5s201144sic@ boolean p_TransformPrecoder := false) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection */ …………….. //@siclog "Step 4" siclog@ //v_Timing := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_Timing, 60); t_Watchdog.start; alt { [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); f_NR_ULGrantConfiguration_ULTBS(nr_Cell1, cs_TimingInfo_Now, v_I_MCS, v_N_PRB, int2bit(v_TDRA, 4), resourceAllocationType1); //@sic R5s190895 sic@ // MOVED f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs_InOneTTI(v_NR_PDCP_SDUList, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } }; }//End for TDRA 0 to 13 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_2_1

## Change 2

| Function name | f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody() |
| --- | --- |
| Reason for change | Current implementation in the main loop of this test csase is, on received Scheduling Request indication, to activate the UL grant and then disable SR indications. It is observed that the delay in disabling SR indication can cause the test case to often fail due to a 2nd SR indication being received at TTCN. |
| Summary of change | Move the disable of scheduling request system indication to before configuring UL grant, for increased reliability. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, NR_ResourceAllocationType_Type p_NR_ResourceAllocationType,//@sic R5s211564 sic@ integer p_Max_Imcs := 28) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection DCI 0_1 */ ………………… //@siclog "Step 4" siclog@ t_Watchdog.start; alt { [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { f_NR_ULGrantConfiguration_ULTBS(nr_Cell1, cs_TimingInfo_Now, v_I_MCS, v_N_PRB, int2bit(v_TDRA, 4), v_NR_ResourceAllocationType); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs_InOneTTI(v_NR_PDCP_SDUList, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } }; }//End for TDRA 0 to 13 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 };//f_NR_7_1_1_4_2_x

After Change

function f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, NR_ResourceAllocationType_Type p_NR_ResourceAllocationType,//@sic R5s211564 sic@ integer p_Max_Imcs := 28) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection DCI 0_1 */ …………………. v_NR_PDCP_SDUList)); //@siclog "Step 4" siclog@ t_Watchdog.start; alt { [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); f_NR_ULGrantConfiguration_ULTBS(nr_Cell1, cs_TimingInfo_Now, v_I_MCS, v_N_PRB, int2bit(v_TDRA, 4), v_NR_ResourceAllocationType); //MOVED f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs_InOneTTI(v_NR_PDCP_SDUList, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } }; }//End for TDRA 0 to 13 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 };//f_NR_7_1_1_4_2_x