3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220109

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2311 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.1.6.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For some test cases where SS is initiating deregistration, the IMS component should be reset to allow for new IMS registration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reset IMS component |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant IMS-enabled UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.6.2.1.NR5GC, 9.1.6.1.2.NR5GC, 9.1.6.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 1 Change 1

| Function name | e.g. fl_TC_9_1_6.2.1 |
| --- | --- |
| Reason for change | At step 1 The SS transmits a DEREGISTRATION REQUEST and so an IMS enabled UE will perform IMS registration again during the step 7-23, following the initial Registration procedure. |
| Summary of change | Reset IMS component to allow new IMS registration. Note : Similar changes are needed for the test cases 9.1.6.1.2 and 9.1.6.2.2 |
| TTCN module | Mobility_Registration_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_6_2_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //The SS transmits a DEREGISTRATION REQUEST with indicates "re-registration required". //@siclog "Step 1" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_DeregistrationReqMTMsg(crs_DeregisterType('0'B, '1'B), omit)))); // @sic R5-201202 sic@ // Check: Does the UE transmit a DEREGISTRATION ACCEPT message? //@siclog "Step 2" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered), cr_NG_DEREGISTRATION_ACCEPT))); f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 2: DEREGISTRATION ACCEPT received"); //The SS releases RRC connection. //@siclog "Step 3" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); // Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE f_NR5GC_MobileInfo_Init();//@sic R5s210059 sic@ //@siclog "Step 4" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(v_NGC_NASCellA); //@siclog "Step 5" siclog@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (v_NGC_NASCellA); //@siclog "Step 6" siclog@ //The SS transmits an RRCSetup message. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(v_NGC_NASCellA, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 7-23" siclog@ //Steps 5-20 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF); }//End of fl_TC_9_1_6_2_1_TestBody

After Change:

function fl_TC_9_1_6_2_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //The SS transmits a DEREGISTRATION REQUEST with indicates "re-registration required". //@siclog "Step 1" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_DeregistrationReqMTMsg(crs_DeregisterType('0'B, '1'B), omit)))); // @sic R5-201202 sic@ // Check: Does the UE transmit a DEREGISTRATION ACCEPT message? //@siclog "Step 2" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind((tsc_SHT_IntegrityProtected,tsc_SHT_IntegrityProtected_Ciphered), cr_NG_DEREGISTRATION_ACCEPT))); f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 2: DEREGISTRATION ACCEPT received"); //The SS releases RRC connection. //@siclog "Step 3" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); // Need to 'reset' all stored lists as all DRBs and PDU sessions no longer exist in UE f_NR5GC_MobileInfo_Init();//@sic R5s210059 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[f_PDN_PdnIndex2Integer(PDN_1)], cms_IPCAN_IMS_Reset); //@siclog "Step 4" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(v_NGC_NASCellA); //@siclog "Step 5" siclog@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (v_NGC_NASCellA); //@siclog "Step 6" siclog@ //The SS transmits an RRCSetup message. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(v_NGC_NASCellA, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 7-23" siclog@ //Steps 5-20 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, TESTMode_OFF); }//End of fl_TC_9_1_6_2_1_TestBody