3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220160

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2195 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC R16 SON-MDT test case 8.1.6.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC RRC test case 8.1.6.1.2.1 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC RRC test case 8.1.6.1.2.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

3.	Branches executed	5

4.	Execution Log Files	5

4.1	Samsung SM-G996B	5

5.	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.2.1 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Verification Test Summary

Test Case: 	8.1.6.1.2.1.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Samsung SM-G996B

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_2_1_NR5GC() |
| --- | --- |
| Reason for change | - Power level for both NR cell1 and cell11 is set to -78 during the test case - Power levels for time T0 are currently set before the preamble function, this means cell 11 is active during the preamble and the UE may select this cell. |
| Summary of change | - Set initial max reference power for the 2 NR cells - Move T0 power level setting to after preamble function |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_2_1_NR5GC() runs on NR5GC_PTC { // Logged MDT / RRC_IDLE / Logging and reporting / Intra-frequency measurement // System information combination NR-2 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR Cells. f_NR5GC_Init(NR_2); //Set Table 8.1.6.1.2.1.3.3-1: SIB2 f_NR_CellInfo_SetSIB2_T_ReselectionNR(nr_Cell1, 7); // NR Cell 1 and NR Cell 11 have different tracking areas according to TS 38.508-1 Table 4.4.2-3. //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); //Set SS/PBCH EPRE levels acc. to row "T0" f_NR_SetCellPower (nr_Cell1, -78, -82); f_NR_SetCellPower (nr_Cell11, -85, -91); // Preamble: The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1, clause 4.4A.2 Table 4.4A.2-3 f_NR5GC_Preamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); f_TC_8_1_6_1_2_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell11, STATE_CONNECTED_3A); } //end of f_TC_8_1_6_1_2_1_NR5GC

After Change:

function f_TC_8_1_6_1_2_1_NR5GC() runs on NR5GC_PTC { // Logged MDT / RRC_IDLE / Logging and reporting / Intra-frequency measurement // System information combination NR-2 as defined in TS 38.508-1 clause 4.4.3.1.2 is used in NR Cells. f_NR5GC_Init(NR_2); //Set initial max ref power f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, -78); // FR2 FFS f_NR_CellInfo_InitMaxReferencePower(nr_Cell11, -78, -78); // FR2 FFS //Set Table 8.1.6.1.2.1.3.3-1: SIB2 f_NR_CellInfo_SetSIB2_T_ReselectionNR(nr_Cell1, 7); // NR Cell 1 and NR Cell 11 have different tracking areas according to TS 38.508-1 Table 4.4.2-3. //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); //Set SS/PBCH EPRE levels acc. to row "T0" /* MOVED f_NR_SetCellPower (nr_Cell1, -78, -82); f_NR_SetCellPower (nr_Cell11, -85, -91); */ // Preamble: The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1, clause 4.4A.2 Table 4.4A.2-3 f_NR5GC_Preamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_SetCellPower (nr_Cell1, -78, -82); f_NR_SetCellPower (nr_Cell11, -85, -91); f_NR_TestBody_Set(true); f_TC_8_1_6_1_2_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell11, STATE_CONNECTED_3A); } //end of f_TC_8_1_6_1_2_1_NR5GC

# Branches executed

The test case was executed on NR band n78 uing NR ciphering nea1 and integrity algorithm nia1

# Execution Log Files

## Samsung SM-G996B

The Samsung SM-G996B  UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_8_1_6_1_2_1_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_8_1_6_1_2_1_PIXIT.xml

# References

| [1] | R5s211643 : NR5GC FR1 : Supporting information for addition of NR5GC R16 SON-MDT test case 8.1.6.1.2.1 |
| --- | --- |