3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220174

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2326 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GMM Emergency Services TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | After UE performs the generic test procedure for IMS MO Emergency Call Release as specified in TS 38.508-1[4] subclause 4.9.12A, UE may immediately transmit DEREGISTRATION REQUEST (Steps 8a1-8a2). This can trigger a race condition and cause TC to fail, since functions in the IP component are still being executed when DEREGISTRATION REQUEST is received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Handling of DEREGISTRATION Request for all UEs that may transmit this is moved inside the Emergency Call Release function. A possible flag pixit p_DeregisterAfterPduRelease is added to ensure existing functionality of the function is not affected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.2.NR5GC, 11.4.3.NR5GC, 11.4.5.NR5GC, 11.4.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | function f_NR5GC_EmergencyCallRelease |
| --- | --- |
| Reason for change | Optional stetps 8a1-8a2 are moved inside Emergency Call Release function to avoid race conditions and support immediate transmission of UE of a Deregistration Message |
| Summary of change | Handling of DEREGISTRATION Request for all UEs that may transmit this is moved inside the Emergency Call Release function |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before change:

* @param     p_NR_CellId

* @status    APPROVED (IMS, NR5GC)

*/

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC

{

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@

var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it

//f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

f_NR5GC_PDUSessionRelease (p_NR_CellId,

v_PDUSessionId,

cs_GMM_GSM_Cause (omit, '00100100'B));

f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit);  // @sic R5s210363 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release");

}

After change:

* @param     p_NR_CellId

* @status    APPROVED (IMS, NR5GC)

*/

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId, boolean p_DeregisterAfterPduRelease := false) runs on NR5GC_PTC

{

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@

var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it

//f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);

f_NR5GC_PDUSessionRelease (p_NR_CellId,

v_PDUSessionId,

cs_GMM_GSM_Cause (omit, '00100100'B),-,-,-,-,-,-, p_DeregisterAfterPduRelease);

f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit);  // @sic R5s210363 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release");

}

## 2.2 Change 2

| Function name | f_NR5GC_PDUSessionRelease |
| --- | --- |
| Reason for change | Optional stetps 8a1-8a2 are moved inside Emergency Call Release function to avoid race conditions and support immediate transmission of UE of a Deregistration Message |
| Summary of change | Handling of DEREGISTRATION Request for all UEs that may transmit this is moved inside the Emergency Call Release function |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before change:

function f_NR5GC_PDUSessionRelease (NR_CellId_Type p_NR_CellId,

O1_Type p_PDU_SessionId,

template (value) GMM_GSM_Cause p_Cause := cs_GMM_GSM_Cause (omit, '00011010'B),

template (omit) GPRS_Timer3 p_BackOff := omit,

template (omit) NGSM_CongestionReattemptInd p_NGSM_CongestionReattemptInd := omit, // @sic R5s190543 sic@,

template (omit) ExtdProtocolConfigOptions p_ExtdPCO := omit,

template (omit) EAP_Message p_EAP := omit,

ProcedureTransactionIdentifier p_PTI := '00'O,

boolean p_ReceiveComplete := true, boolean p_DeregisterAfterPduRelease := false)  runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(p_PDU_SessionId));

var NR_DRBInfo_Type v_DRBInfo;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RLC_BearerToReleaseList_Type v_RLC_BearerToReleaseList;

var template (value) DRB_ToReleaseList v_DRB_ToReleaseList;

var template (value) NR_RadioBearerList_Type v_SS_Config;

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

var template (omit) PDN_Index_Type v_PDN_Index := omit;

var integer i;

timer t_Wait := 10.0;

// Build NAS command

v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand(p_PDU_SessionId,

p_Cause,

p_BackOff,

p_NGSM_CongestionReattemptInd,

p_ExtdPCO,

p_EAP,

p_PTI);

// Build RRC command for all the DRBs linked to this PDU Session @sic R5-202550 sic@

for (i := 0; i < lengthof(v_DRBIdList); i := i+1) {

v_DRBInfo := valueof(f_NR_GetDRBInfo_FromId(v_DRBIdList[i]));

v_DRB_ToReleaseList[i] := v_DRBInfo.DRB_Id;

v_RLC_BearerToReleaseList[i] := v_DRBInfo.RLC_Config.logicalChannelIdentity;

v_SS_Config[i] := cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBInfo.DRB_Id));

}

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, v_DRB_ToReleaseList);

v_CellGroupConfig := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(p_NR_CellId), omit, v_RLC_BearerToReleaseList);  //@sic R5s201387 sic@

// The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION RELEASE COMMAND

f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId,

v_RadioBearerConfig,

v_SS_Config,

v_CellGroupConfig,

{ cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) },

tsc_NR_RbId_SRB1, -, false);

if (p_ReceiveComplete) {

// @sic R5-206266 sic@

interleave { // @sic R5-215678 sic@

// The UE transmit an RRCReconfigurationComplete message

[] SRB.receive(car_NR_SRB_RrcPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)){}

// The UE transmits an ULInformationTransfer message and a UL NAS TRANSPORT containing PDU SESSION RELEASE COMPLETE.

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId,

tsc_NR_RbId_SRB2,

cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp{}

}

// Handling of optional UE behavior as specified in Steps 8a1-8b3 Table 11.4.2.3.2-1. Some UEs will send a DeRegistration request to the NW after the Release procedure

if (p_DeregisterAfterPduRelease) {

//Start Timer.

t_Wait.start;

//@siclog "Step 8a1-8b3" siclog@

//EXCEPTION: Step 8a1 and 8b3 describes optional behaviour that depends on the UE implementation.

alt {

//@siclog "Step 8a1" siclog@

//The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration".

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered,

cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) {

//@siclog "Step 8a2" siclog@

//The SS transmits a DEREGISTRATION ACCEPT message.

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT)));

//@siclog "Step 8a3" siclog@

//Stop Timer.

t_Wait.stop;

}

//@siclog "Step 8b1" siclog@

//Timer expires.

[] t_Wait.timeout {} //@sic R5-216198 sic@

}

}

v_PDN_Index := f_Get_PDN_IndexForPDUSessionId (v_PDUSessionInfoList, p_PDU_SessionId); // @sic R5s211524 sic@

if (isvalue(v_PDN_Index)) {  // @sic R5s211524 sic@

f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(valueof(v_PDN_Index)));

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Cannot find PDN Index of PDU Session being released");

}

if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, p_PDU_SessionId)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed");

}

}

f_NR5GC_MobileInfo_SetSessionInfoList (v_PDUSessionInfoList); // @sic R5-206266 sic@

}

After change:

function f_NR5GC_PDUSessionRelease (NR_CellId_Type p_NR_CellId,

O1_Type p_PDU_SessionId,

template (value) GMM_GSM_Cause p_Cause := cs_GMM_GSM_Cause (omit, '00011010'B),

template (omit) GPRS_Timer3 p_BackOff := omit,

template (omit) NGSM_CongestionReattemptInd p_NGSM_CongestionReattemptInd := omit, // @sic R5s190543 sic@,

template (omit) ExtdProtocolConfigOptions p_ExtdPCO := omit,

template (omit) EAP_Message p_EAP := omit,

ProcedureTransactionIdentifier p_PTI := '00'O,

boolean p_ReceiveComplete := true, boolean p_DeregisterAfterPduRelease := false)  runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(p_PDU_SessionId));

var NR_DRBInfo_Type v_DRBInfo;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RLC_BearerToReleaseList_Type v_RLC_BearerToReleaseList;

var template (value) DRB_ToReleaseList v_DRB_ToReleaseList;

var template (value) NR_RadioBearerList_Type v_SS_Config;

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

var template (omit) PDN_Index_Type v_PDN_Index := omit;

var integer i;

timer t_Wait := 10.0;

// Build NAS command

v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand(p_PDU_SessionId,

p_Cause,

p_BackOff,

p_NGSM_CongestionReattemptInd,

p_ExtdPCO,

p_EAP,

p_PTI);

// Build RRC command for all the DRBs linked to this PDU Session @sic R5-202550 sic@

for (i := 0; i < lengthof(v_DRBIdList); i := i+1) {

v_DRBInfo := valueof(f_NR_GetDRBInfo_FromId(v_DRBIdList[i]));

v_DRB_ToReleaseList[i] := v_DRBInfo.DRB_Id;

v_RLC_BearerToReleaseList[i] := v_DRBInfo.RLC_Config.logicalChannelIdentity;

v_SS_Config[i] := cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBInfo.DRB_Id));

}

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, v_DRB_ToReleaseList);

v_CellGroupConfig := cs_38508_CellGroupConfig(f_NR_CellInfo_Get_CellGroupId(p_NR_CellId), omit, v_RLC_BearerToReleaseList);  //@sic R5s201387 sic@

// The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION RELEASE COMMAND

f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId,

v_RadioBearerConfig,

v_SS_Config,

v_CellGroupConfig,

{ cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) },

tsc_NR_RbId_SRB1, -, false);

if (p_ReceiveComplete) {

// @sic R5-206266 sic@

interleave { // @sic R5-215678 sic@

// The UE transmit an RRCReconfigurationComplete message

[] SRB.receive(car_NR_SRB_RrcPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)){}

// The UE transmits an ULInformationTransfer message and a UL NAS TRANSPORT containing PDU SESSION RELEASE COMPLETE.

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId,

tsc_NR_RbId_SRB2,

cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp{}

}

// Handling of optional UE behavior as specified in Steps 8a1-8b3 Table 11.4.2.3.2-1. Some UEs will send a DeRegistration request to the NW after the Release procedure

if (p_DeregisterAfterPduRelease) {

//Start Timer.

t_Wait.start;

//@siclog "Step 8a1-8b3" siclog@

//EXCEPTION: Step 8a1 and 8b3 describes optional behaviour that depends on the UE implementation.

alt {

//@siclog "Step 8a1" siclog@

//The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration".

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered,

cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) {

//@siclog "Step 8a2" siclog@

//The SS transmits a DEREGISTRATION ACCEPT message.

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT)));

//@siclog "Step 8a3" siclog@

//Stop Timer.

t_Wait.stop;

}

//@siclog "Step 8b1" siclog@

//Timer expires.

[] t_Wait.timeout {} //@sic R5-216198 sic@

}

}

v_PDN_Index := f_Get_PDN_IndexForPDUSessionId (v_PDUSessionInfoList, p_PDU_SessionId); // @sic R5s211524 sic@

if (isvalue(v_PDN_Index)) {  // @sic R5s211524 sic@

f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(valueof(v_PDN_Index)));

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Cannot find PDN Index of PDU Session being released");

}

if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, p_PDU_SessionId)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed");

}

}

f_NR5GC_MobileInfo_SetSessionInfoList (v_PDUSessionInfoList); // @sic R5-206266 sic@

}

## 2.3 Change 3

| Function name | fl_TC_11_4_2_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Optional stetps 8a1-8a2 are moved inside Emergency Call Release function to avoid race conditions and support immediate transmission of UE of a Deregistration Message |
| Summary of change | Handling of DEREGISTRATION Request for all UEs that may transmit this is moved inside the Emergency Call Release function |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before change:

function fl_TC_11_4_2_NR5GC_TestBody() runs on NR5GC_PTC

{

timer t_Wait := 10.0;

//@siclog "Step 1" siclog@

//The SS configures:- "NR Cell 1 as "Serving cell".

f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

//@siclog "Step 2" siclog@

//Switch the UE on.

f_UT_SwitchOnUE(UT);

//@siclog "Step 3" siclog@

//Make the UE attempt an IMS emergency call dialling a number which is stored on the ME.

f_UT_RequestIMSEmergencyCall(UT);

//@siclog "Step 4" siclog@

//Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12?

f_NR5GC_EmergencyCallLimitedService(nr_Cell1);

//@siclog "Step 5-6" siclog@

//@siclog "Step 7" siclog@

//@siclog "Step 8a1-8b3" Step 8a1 and 8b3 describes optional behaviour that depends on the UE implementation.

//Make the UE release the emergency call.

f_NR5GC_EmergencyCallRelease(nr_Cell1, true);

//@siclog "Step 7" siclog@

//Start Timer.

/*t_Wait.start;

//@siclog "Step 8a1-8b3" siclog@

//EXCEPTION: Step 8a1 and 8b3 describes optional behaviour that depends on the UE implementation.

alt {

//@siclog "Step 8a1" siclog@

//The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration".

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered,

cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) {

//@siclog "Step 8a2" siclog@

//The SS transmits a DEREGISTRATION ACCEPT message.

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT)));

//@siclog "Step 8a3" siclog@

//Stop Timer.

t_Wait.stop;

}

//@siclog "Step 8b1" siclog@

//Timer expires.

[] t_Wait.timeout {} //@sic R5-216198 sic@

} */

//@siclog "Step 9" siclog@

//The SS releases RRC connection.

f_NR_RRCRelease(nr_Cell1);

//@siclog "Steps 9A" siclog@

//@sic R5-216198 sic@

// Make the UE attempt an IMS none-emergency call.

f_UT_RequestIMSCall(UT);

//@siclog "Step 10" siclog@

//@sic R5-216198 sic@

//Check: Does the UE transmit an RRCSetupRequest message in the next 30sec?

f_NR_CheckNoRRCSetupReq (nr_Cell1, 10.0);

}

After change:

function fl_TC_11_4_2_NR5GC_TestBody() runs on NR5GC_PTC

{

timer t_Wait := 10.0;

//@siclog "Step 1" siclog@

//The SS configures:- "NR Cell 1 as "Serving cell".

f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

//@siclog "Step 2" siclog@

//Switch the UE on.

f_UT_SwitchOnUE(UT);

//@siclog "Step 3" siclog@

//Make the UE attempt an IMS emergency call dialling a number which is stored on the ME.

f_UT_RequestIMSEmergencyCall(UT);

//@siclog "Step 4" siclog@

//Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12?

f_NR5GC_EmergencyCallLimitedService(nr_Cell1);

//@siclog "Step 5-6" siclog@

//@siclog "Step 7" siclog@

//@siclog "Step 8a1-8b3" Step 8a1 and 8b3 describes optional behaviour that depends on the UE implementation.

//Make the UE release the emergency call.

f_NR5GC_EmergencyCallRelease(nr_Cell1, true);

//@siclog "Step 7" siclog@

//Start Timer.

/*t_Wait.start;

//@siclog "Step 8a1-8b3" siclog@

//EXCEPTION: Step 8a1 and 8b3 describes optional behaviour that depends on the UE implementation.

alt {

//@siclog "Step 8a1" siclog@

//The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration".

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered,

cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) {

//@siclog "Step 8a2" siclog@

//The SS transmits a DEREGISTRATION ACCEPT message.

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT)));

//@siclog "Step 8a3" siclog@

//Stop Timer.

t_Wait.stop;

}

//@siclog "Step 8b1" siclog@

//Timer expires.

[] t_Wait.timeout {} //@sic R5-216198 sic@

} */

//@siclog "Step 9" siclog@

//The SS releases RRC connection.

f_NR_RRCRelease(nr_Cell1);

//@siclog "Steps 9A" siclog@

//@sic R5-216198 sic@

// Make the UE attempt an IMS none-emergency call.

f_UT_RequestIMSCall(UT);

//@siclog "Step 10" siclog@

//@sic R5-216198 sic@

//Check: Does the UE transmit an RRCSetupRequest message in the next 30sec?

f_NR_CheckNoRRCSetupReq (nr_Cell1, 10.0);

}

## 2.4 Change 4

| Function name | fl_TC_11_4_3_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Optional stetps 8a1-8a2 are moved inside Emergency Call Release function to avoid race conditions and support immediate transmission of UE of a Deregistration Message |
| Summary of change | Handling of DEREGISTRATION Request for all UEs that may transmit this is moved inside the Emergency Call Release function |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before change:

function fl_TC_11_4_3_NR5GC_TestBody() runs on NR5GC_PTC

{

timer t_Wait := 5.0; // @sic R5-217821 sic@

//@siclog "Step 1" siclog@

//Switch the UE on.

f_UT_SwitchOnUE(UT);

//@siclog "Step 2" siclog@

//Make the UE attempt an IMS emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1)

f_UT_RequestIMSEmergencyCall(UT);

//@siclog "Step 3" siclog@

//Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12?

f_NR5GC_EmergencyCallLimitedService(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "step 3");//@sic R5-217821 sic@

//@siclog "Step 4-5" siclog@

//Make the UE release the emergency call.

f_NR5GC_EmergencyCallRelease(nr_Cell1);

//@siclog "Step 6" siclog@

//Start Timer.

t_Wait.start;

//@siclog "Step 7a1-7b1" siclog@

//EXCEPTION: Step 7a1 and 7b1 describes optional behaviour that depends on the UE implementation.

alt {

//@siclog "Step 7a1" siclog@

//The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration".

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered,

cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) {

//@siclog "Step 7a2" siclog@

//The SS transmits a DEREGISTRATION ACCEPT message.

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT)));

//@siclog "Step 7a3" siclog@

//Stop Timer.

t_Wait.stop; }

//@siclog "Step 7b1" siclog@

//Timer expires.

[] t_Wait.timeout {}

}

}

After change:

function fl_TC_11_4_3_NR5GC_TestBody() runs on NR5GC_PTC

{

timer t_Wait := 5.0; // @sic R5-217821 sic@

//@siclog "Step 1" siclog@

//Switch the UE on.

f_UT_SwitchOnUE(UT);

//@siclog "Step 2" siclog@

//Make the UE attempt an IMS emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1)

f_UT_RequestIMSEmergencyCall(UT);

//@siclog "Step 3" siclog@

//Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12?

f_NR5GC_EmergencyCallLimitedService(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "step 3");//@sic R5-217821 sic@

//@siclog "Step 4-5" siclog@

//Make the UE release the emergency call.

f_NR5GC_EmergencyCallRelease(nr_Cell1, true);

//@siclog "Step 6" siclog@

//Start Timer.

/*t_Wait.start;

//@siclog "Step 7a1-7b1" siclog@

//EXCEPTION: Step 7a1 and 7b1 describes optional behaviour that depends on the UE implementation.

alt {

//@siclog "Step 7a1" siclog@

//The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration".

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered,

cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) {

//@siclog "Step 7a2" siclog@

//The SS transmits a DEREGISTRATION ACCEPT message.

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT)));

//@siclog "Step 7a3" siclog@

//Stop Timer.

t_Wait.stop; }

//@siclog "Step 7b1" siclog@

//Timer expires.

[] t_Wait.timeout {}

}*/

}

## 2.5 Change 5

| Function name | fl_TC_11_4_5_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Optional stetps 8a1-8a2 are moved inside Emergency Call Release function to avoid race conditions and support immediate transmission of UE of a Deregistration Message |
| Summary of change | Handling of DEREGISTRATION Request for all UEs that may transmit this is moved inside the Emergency Call Release function |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before change:

function fl_TC_11_4_5_NR5GC_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NR_SecurityParams_Type v_RRC_SecurityParams;

timer t_T1:= 5.0;//@sic R5-216201 R5-217820 sic@

//@siclog "Step 1" siclog@

//The SS configures:NR Cell 11 as "Serving cell" NR Cell 1 as "Non-Suitable "Off" cell".//@sic R5s210875 R5-217820 sic@

f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);

//@siclog "Step 2" siclog@

//The UE transmits an RRCSetupRequest message.    //@sic R5-216201 sic@

//@siclog "Step 3" siclog@

//SS transmit an RRCSetupRequest message.//@sic R5-216201 sic@

//@siclog "Step 4" siclog@

//The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area.

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell11, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Req failed");

}

//@siclog "Step 5" siclog@

//SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area).

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

-,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_NoSuitableCellsInTA)))));

//@siclog "Step 6" siclog@

//The SS transmits an RRCRelease message.//@sic R5-216201 sic@

f_NR_RRCRelease(nr_Cell11);

//@siclog "Steps 7" siclog@

//Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1)

f_UT_RequestIMSEmergencyCall(UT);//@sic R5s210875 sic@

//@siclog "Step 8" siclog@

//Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11?

f_NR5GC_EmergencyCallLimitedService(nr_Cell11);//@sic R5-216201 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "step 8");//@sic R5-216201 sic@

//@siclog "Step 9-10" siclog@

//Make the UE release the emergency call. (NOTE 1)

//The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place

f_NR5GC_EmergencyCallRelease(nr_Cell11);//@sic R5-216201 sic@

//@siclog "Step 11" siclog@

//void//@sic R5-216201 sic@

//@siclog "Step 12" siclog@

//Start Timer T1=10 seconds.

//NOTE: This is an arbitrary value to wait for UE to initiate deregistration.//@sic R5-216201 sic@

t_T1.start;//@sic R5-216201 sic@//@sic R5s210875 sic@

//EXCEPTION: Steps 12a1-12b1 describes optional behaviour that depends on the UE implementation.

alt {//@sic R5s210875 sic@

//@siclog "Step 12a1" siclog@

//The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration".//@sic R5-216201 sic@

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered,

cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) {

//@siclog "Step 12a2" siclog@

//The SS transmits a DEREGISTRATION ACCEPT message.//@sic R5-216201 sic@

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT)));

//@siclog "Step 12a3" siclog@

//Stop Timer.

t_T1.stop;//@sic R5-216201 sic@

}

//@siclog "Step 12b1" siclog@

//Timer expires.//@sic R5-216201 sic@

[] t_T1.timeout {}

}

//@siclog "Step 12A" siclog@

//SS releases the RRC connection

f_NR_RRCRelease(nr_Cell11);//@sic R5-216201 sic@//@sic R5s210875 sic@

…

}

After change:

function fl_TC_11_4_5_NR5GC_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NR_SecurityParams_Type v_RRC_SecurityParams;

timer t_T1:= 5.0;//@sic R5-216201 R5-217820 sic@

//@siclog "Step 1" siclog@

//The SS configures:NR Cell 11 as "Serving cell" NR Cell 1 as "Non-Suitable "Off" cell".//@sic R5s210875 R5-217820 sic@

f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);

//@siclog "Step 2" siclog@

//The UE transmits an RRCSetupRequest message.    //@sic R5-216201 sic@

//@siclog "Step 3" siclog@

//SS transmit an RRCSetupRequest message.//@sic R5-216201 sic@

//@siclog "Step 4" siclog@

//The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area.

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell11, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Req failed");

}

//@siclog "Step 5" siclog@

//SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area).

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

-,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_NoSuitableCellsInTA)))));

//@siclog "Step 6" siclog@

//The SS transmits an RRCRelease message.//@sic R5-216201 sic@

f_NR_RRCRelease(nr_Cell11);

//@siclog "Steps 7" siclog@

//Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1)

f_UT_RequestIMSEmergencyCall(UT);//@sic R5s210875 sic@

//@siclog "Step 8" siclog@

//Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11?

f_NR5GC_EmergencyCallLimitedService(nr_Cell11);//@sic R5-216201 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "step 8");//@sic R5-216201 sic@

//@siclog "Step 9-10" siclog@

//Make the UE release the emergency call. (NOTE 1)

//The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place

f_NR5GC_EmergencyCallRelease(nr_Cell11,true);//@sic R5-216201 sic@

//@siclog "Step 11" siclog@

//void//@sic R5-216201 sic@

//@siclog "Step 12" siclog@

//Start Timer T1=10 seconds.

//NOTE: This is an arbitrary value to wait for UE to initiate deregistration.//@sic R5-216201 sic@

/*t_T1.start;//@sic R5-216201 sic@//@sic R5s210875 sic@

//EXCEPTION: Steps 12a1-12b1 describes optional behaviour that depends on the UE implementation.

alt {//@sic R5s210875 sic@

//@siclog "Step 12a1" siclog@

//The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration".//@sic R5-216201 sic@

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered,

cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) {

//@siclog "Step 12a2" siclog@

//The SS transmits a DEREGISTRATION ACCEPT message.//@sic R5-216201 sic@

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT)));

//@siclog "Step 12a3" siclog@

//Stop Timer.

t_T1.stop;//@sic R5-216201 sic@

}

//@siclog "Step 12b1" siclog@

//Timer expires.//@sic R5-216201 sic@

[] t_T1.timeout {}

}*/

//@siclog "Step 12A" siclog@

//SS releases the RRC connection

f_NR_RRCRelease(nr_Cell11);//@sic R5-216201 sic@//@sic R5s210875 sic@

…

}

## 2.6 Change 6

| Function name | fl_TC_11_4_9_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Optional stetps 8a1-8a2 are moved inside Emergency Call Release function to avoid race conditions and support immediate transmission of UE of a Deregistration Message |
| Summary of change | Handling of DEREGISTRATION Request for all UEs that may transmit this is moved inside the Emergency Call Release function |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before change:

function fl_TC_11_4_9_NR5GC_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

timer t_T1:= 5.0;//@sic R5-216202 R5-217823 sic@

//@siclog "Step 1" siclog@

//Switch the UE on.

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2-4" siclog@

//Steps 2-4 of Table 4.5.2.2-2 NR RRC_IDLE in TS38.508-1 [4] take place (the UE initiates an initial registration procedure).

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Req failed");

}

//@siclog "Steps 5" siclog@

//SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area).

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

-,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_NoSuitableCellsInTA)))));

//@siclog "Step 6" siclog@

//The SS transmits an RRCRelease message.//@sic R5-216202 sic@

f_NR_RRCRelease(nr_Cell1);

//@siclog "Step 7" siclog@

//Make the UE attempt an IMS emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1)

f_UT_RequestIMSEmergencyCall (UT);//Defaut value is px_EmergencyCallNumber

//@siclog "Step 8" siclog@

//Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12?

f_NR5GC_EmergencyCallLimitedService(nr_Cell1);

//@siclog "Step 9" siclog@

//Make the UE release the emergency call. (NOTE 1)

//@siclog "Step 10" siclog@

//The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place.

f_NR5GC_EmergencyCallRelease(nr_Cell1);

//@siclog "Step 11" siclog@

//Start T1=10 seconds. NOTE: This is an arbitrary value to wait for UE initiated detach.

t_T1.start;//@sic R5-216202 sic@

//EXCEPTION: Steps 12a1-12b3 describes optional behaviour that depends on the UE implementation.

alt {

//@siclog "Step 12a1" siclog@

//The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration".

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered,

cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) {

//@siclog "Step 12a2" siclog@

//The SS transmits a DEREGISTRATION ACCEPT message.

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT)));

//@siclog "Step 12a3" siclog@

//Stop Timer.

t_T1.stop;//@sic R5-216202 sic@

}

//@siclog "Step 12b1" siclog@

//Timer expires.

[] t_T1.timeout {//@sic R5-216202 sic@

//@siclog "Step 12b2" siclog@

//Void @sic R5-216202 sic@

//@siclog "Step 12b3" siclog@

//Void @sic R5-216202 sic@

}

}

//@siclog "Step 13" siclog@

//SS releases the RRC connection

f_NR_RRCRelease(nr_Cell1);

//@siclog "Step 13A" siclog@//@sic R5-216202 sic@

//Make the UE attempt an IMS none-emergency call. (NOTE 2)

f_UT_RequestIMSCall(UT);

//@siclog "Step 14" siclog@

//Check: Does the UE transmit an RRCSetupRequest message in the next 10second?//@sic R5-216202 sic@

//NOTE: This is an arbitrary value to wait for catching not allowed UE behaviour.

f_NR_CheckNoRRCSetupReq (nr_Cell1, 10.0);//@sic R5-216202 R5s211323 sic@

}

}After change:

function fl_TC_11_4_9_NR5GC_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

timer t_T1:= 5.0;//@sic R5-216202 R5-217823 sic@

//@siclog "Step 1" siclog@

//Switch the UE on.

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2-4" siclog@

//Steps 2-4 of Table 4.5.2.2-2 NR RRC_IDLE in TS38.508-1 [4] take place (the UE initiates an initial registration procedure).

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Req failed");

}

//@siclog "Steps 5" siclog@

//SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area).

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

-,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_NoSuitableCellsInTA)))));

//@siclog "Step 6" siclog@

//The SS transmits an RRCRelease message.//@sic R5-216202 sic@

f_NR_RRCRelease(nr_Cell1);

//@siclog "Step 7" siclog@

//Make the UE attempt an IMS emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (NOTE 1)

f_UT_RequestIMSEmergencyCall (UT);//Defaut value is px_EmergencyCallNumber

//@siclog "Step 8" siclog@

//Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12?

f_NR5GC_EmergencyCallLimitedService(nr_Cell1);

//@siclog "Step 9" siclog@

//Make the UE release the emergency call. (NOTE 1)

//@siclog "Step 10" siclog@

//The Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A takes place.

f_NR5GC_EmergencyCallRelease(nr_Cell1,true);

//@siclog "Step 11" siclog@

//Start T1=10 seconds. NOTE: This is an arbitrary value to wait for UE initiated detach.

/*t_T1.start;//@sic R5-216202 sic@

//EXCEPTION: Steps 12a1-12b3 describes optional behaviour that depends on the UE implementation.

alt {

//@siclog "Step 12a1" siclog@

//The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration".

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB2,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered,

cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) {

//@siclog "Step 12a2" siclog@

//The SS transmits a DEREGISTRATION ACCEPT message.

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, -,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT)));

//@siclog "Step 12a3" siclog@

//Stop Timer.

t_T1.stop;//@sic R5-216202 sic@

}

//@siclog "Step 12b1" siclog@

//Timer expires.

[] t_T1.timeout {//@sic R5-216202 sic@

//@siclog "Step 12b2" siclog@

//Void @sic R5-216202 sic@

//@siclog "Step 12b3" siclog@

//Void @sic R5-216202 sic@

}

}*/

//@siclog "Step 13" siclog@

//SS releases the RRC connection

f_NR_RRCRelease(nr_Cell1);

//@siclog "Step 13A" siclog@//@sic R5-216202 sic@

//Make the UE attempt an IMS none-emergency call. (NOTE 2)

f_UT_RequestIMSCall(UT);

//@siclog "Step 14" siclog@

//Check: Does the UE transmit an RRCSetupRequest message in the next 10second?//@sic R5-216202 sic@

//NOTE: This is an arbitrary value to wait for catching not allowed UE behaviour.

f_NR_CheckNoRRCSetupReq (nr_Cell1, 10.0);//@sic R5-216202 R5s211323 sic@

}

}