3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220198

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2343 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Reestablisment procedure (add SRB1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | upon RLF, the UE releases the lower layer configuration (specifically, the SR and the PUCCH resoures are released). After the reestablishment complete is received the RRCReconfiguration message is sent to add physical resoureces and SRB2 and DRB configuration. In this reconfiguration message SRB1 is not added, therefore any SRB1 transmission will be based on RACH procedure (restablishment complete, RRCReconfigurationComplete, UEInformationResponse) It is proposed to add SRB1 to the RRCReconfiguration message to avoid all SRB1 tranmission to be triggered via RACH procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SRB1 Is added to the RLC_BearerToAddModList Added SRB1 to srb_ToAddModList |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SRB1 transmission will not be based on SR and therefore will trigger RACH procedure to send the SRB1 data |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ALL Resestablishment test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections to restablisment procedure (add SRB1)	2

1.1	Correction to f_NR5GC_RRCReestablishment_Steps8To13	2

1.2	Correction to f_NR_GetCellGroupConfigREEST	6

# Corrections to restablisment procedure (add SRB1)

## Correction to f_NR5GC_RRCReestablishment_Steps8To13

| Function name | f_NR5GC_RRCReestablishment_Steps8To13 |
| --- | --- |
| Reason for change | upon RLF, the UE releases the lower layer configuration (specifically, the SR and the PUCCH resoures are released). After the reestablishment complete is received the RRCReconfiguration message is sent to add physical resoureces and SRB2 and DRB configuration. In this reconfiguration message SRB1 is not added,therefore any SRB1 transmission will be based on RACH procedure (restablishment complete, RRCReconfigurationComplete, UEInformationResponse) It is proposed to add SRB1 to the RRCReconfiguration message to avoid all SRB1 tranmission to be triggered via RACH procedure. |
| Summary of change | Added SRB1 to srb_ToAddModList |
| TTCN module |  |
| MCC160 Comment | NR5GC_RRCSteps |

Before Change:

function f_NR5GC_RRCReestablishment_Steps8To13(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template (omit) NR_SCellToAddModList_Type p_SCellToAddModList := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { //@sic R5-206369 R5-213516: added p_MacBearerRoutingCA sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId); var IntegerList_Type v_DrbIdList; var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(true); var template (value) CellGroupConfig v_CellGroupConfig := f_NR_GetCellGroupConfigREEST(p_NR_TargetCellId, p_SCellToAddModList, p_RlfTimersAndConstants); var template (value) NR_RadioBearerList_Type v_SS_RadioBearerConfig := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); // Step 8. Target Cell: Reconfigure UL DCCH DCI on USS f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_SearchSpaceUlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceUlDciAssignment(v_NR_PhysicalParameters))); // @sic R5-217830 sic@ // Step 8a. Target Cell // If the Source Cell is different from the Target Cell // Configure UL grant "OnSR" and stop periodic TA // Else // Stop periodic TA if (p_NR_SourceCellId != p_NR_TargetCellId) { f_NR_ULGrantConfiguration_Start(p_NR_TargetCellId, -, -, -, cs_NR_UplinkTimeAlignment_Stop); } else { f_NR_ULGrantConfiguration_Common(p_NR_SourceCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop, cs_UL_GrantConfig_OnSR); } // Step 8b. Target Cell Reconfigure DRBs // Step 9. Target Cell: Send RRCReconfiguration // Step 10. Target Cell: Receive RRCReconfigurationComplete // Set RadioBearerConfig as per Table 4.6.3-132: RadioBearerConfig cond REEST v_RadioBearerConfig.srb_ToAddModList := {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}; //@sic R5-205334 sic@ f_ReconfigureDRBs(p_NR_TargetCellId, omit, v_RadioBearerConfig, v_CellGroupConfig, v_SS_RadioBearerConfig, -, -, p_ReceiveForRRCreconfigurationComplete, -, -, -, p_MacBearerRoutingCA); //@sic R5-205336 sic@ // 11. Target Cell: Reconfigure DL DCCH/DTCH DCI on USS //NOTE 2: The FollowOnFlag is set to true in the ASP reconfiguring the target Cell. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_SearchSpaceDlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceDlDciAssignment_USS(v_NR_PhysicalParameters), tsc_NoCnfReq, tsc_FollowOnFlag)); // 12. Target Cell: Re-configure RACH procedure as for initial access, enable TA transmissions. //NOTE 3: The FollowOnFlag is set to true in the ASP reconfiguring the target PCell. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedureConfig_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), tsc_NoCnfReq, tsc_FollowOnFlag)); f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_UplinkTimeAlignment_REQ(p_NR_TargetCellId, -, cs_NR_UplinkTimeAlignment_Start)); // 13. Source Cell: If the Source Cell is different from the Target Cell: // Release security. // Reset SRBs and release DRBs. // (Re-)configure SRBs. // Stop periodic UL grant and periodic TA. if (p_NR_SourceCellId != p_NR_TargetCellId) { f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId, cs_TimingInfo_Now); //@sic R5s211165 sic@ v_DrbIdList := f_NR_GetActiveDRBs(p_NR_TargetCellId); f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList, tsc_NoCnfReq); //@sic R5-211417 R5s211165 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, tsc_FollowOnFlag); f_NR_ULGrantConfiguration_Stop(p_NR_SourceCellId, -, cs_NR_UplinkTimeAlignment_Stop); } }

After Change:

function f_NR5GC_RRCReestablishment_Steps8To13(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template (omit) NR_SCellToAddModList_Type p_SCellToAddModList := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { //@sic R5-206369 R5-213516: added p_MacBearerRoutingCA sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId); var IntegerList_Type v_DrbIdList; var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(true); var template (value) CellGroupConfig v_CellGroupConfig := f_NR_GetCellGroupConfigREEST(p_NR_TargetCellId, p_SCellToAddModList, p_RlfTimersAndConstants); var template (value) NR_RadioBearerList_Type v_SS_RadioBearerConfig := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); // Step 8. Target Cell: Reconfigure UL DCCH DCI on USS f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_SearchSpaceUlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceUlDciAssignment(v_NR_PhysicalParameters))); // @sic R5-217830 sic@ // Step 8a. Target Cell // If the Source Cell is different from the Target Cell // Configure UL grant "OnSR" and stop periodic TA // Else // Stop periodic TA if (p_NR_SourceCellId != p_NR_TargetCellId) { f_NR_ULGrantConfiguration_Start(p_NR_TargetCellId, -, -, -, cs_NR_UplinkTimeAlignment_Stop); } else { f_NR_ULGrantConfiguration_Common(p_NR_SourceCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop, cs_UL_GrantConfig_OnSR); } // Step 8b. Target Cell Reconfigure DRBs // Step 9. Target Cell: Send RRCReconfiguration // Step 10. Target Cell: Receive RRCReconfigurationComplete // Set RadioBearerConfig as per Table 4.6.3-132: RadioBearerConfig cond REEST //WA# added SRB1 to the list to use SR configuration v_RadioBearerConfig.srb_ToAddModList := {cs_NR_SRB_ToAddMod(tsc_SRB1),cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}; //@sic R5-205334 sic@ f_ReconfigureDRBs(p_NR_TargetCellId, omit, v_RadioBearerConfig, v_CellGroupConfig, v_SS_RadioBearerConfig, -, -, p_ReceiveForRRCreconfigurationComplete, -, -, -, p_MacBearerRoutingCA); //@sic R5-205336 sic@ // 11. Target Cell: Reconfigure DL DCCH/DTCH DCI on USS //NOTE 2: The FollowOnFlag is set to true in the ASP reconfiguring the target Cell. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_SearchSpaceDlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceDlDciAssignment_USS(v_NR_PhysicalParameters), tsc_NoCnfReq, tsc_FollowOnFlag)); // 12. Target Cell: Re-configure RACH procedure as for initial access, enable TA transmissions. //NOTE 3: The FollowOnFlag is set to true in the ASP reconfiguring the target PCell. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedureConfig_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), tsc_NoCnfReq, tsc_FollowOnFlag)); f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_UplinkTimeAlignment_REQ(p_NR_TargetCellId, -, cs_NR_UplinkTimeAlignment_Start)); // 13. Source Cell: If the Source Cell is different from the Target Cell: // Release security. // Reset SRBs and release DRBs. // (Re-)configure SRBs. // Stop periodic UL grant and periodic TA. if (p_NR_SourceCellId != p_NR_TargetCellId) { f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId, cs_TimingInfo_Now); //@sic R5s211165 sic@ v_DrbIdList := f_NR_GetActiveDRBs(p_NR_TargetCellId); f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList, tsc_NoCnfReq); //@sic R5-211417 R5s211165 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, tsc_FollowOnFlag); f_NR_ULGrantConfiguration_Stop(p_NR_SourceCellId, -, cs_NR_UplinkTimeAlignment_Stop); } }

## Correction to f_NR_GetCellGroupConfigREEST

| Function name | f_NR_GetCellGroupConfigREEST |
| --- | --- |
| Reason for change | upon RLF, the UE releases the lower layer configuration (specifically, the SR and the PUCCH resoures are released). After the reestablishment complete is received the RRCReconfiguration message is sent to add physical resoureces and SRB2 and DRB configuration. In this reconfiguration message SRB1 is not added,therefore any SRB1 transmission will be based on RACH procedure (restablishment complete, RRCReconfigurationComplete, UEInformationResponse) It is proposed to add SRB1 to the RRCReconfiguration message to avoid all SRB1 tranmission to be triggered via RACH procedure. |
| Summary of change | SRB1 Is added to the RLC_BearerToAddModList |
| TTCN module | NR5GC_RRCSteps |
| MCC160 Comment |  |

Before Change:

function f_NR_GetCellGroupConfigREEST( NR_CellId_Type p_TargetCellId, template (omit) NR_SCellToAddModList_Type p_SCellToAddModList := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit ) runs on NR5GC_PTC return template(value) CellGroupConfig { //@sic R5-206297 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_TargetCellId); var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_TargetCellId); var template (omit) ServingCellConfig v_ServingCellConfig := f_NR_GetServinCellConfig(p_TargetCellId); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModListREEST; //@sic R5s210345 sic@ var integer i; //Remove SRB1 - entry 0 - in BearerToAddModList for (i:=0; i < lengthof(v_RLC_BearerToAddModList)-1; i:=i+1) { v_RLC_BearerToAddModListREEST[i] := v_RLC_BearerToAddModList[i+1]; } v_SpCellConfig := f_NR_GetSpCellConfigDef(p_TargetCellId, v_ServingCellConfig); v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, v_RLC_BearerToAddModListREEST, omit, // NR_RLC_BearerToReleaseList_Type v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); //@sic R5s210345 sic@ if (isvalue(p_SCellToAddModList)) { v_CellGroupConfig.sCellToAddModList := p_SCellToAddModList; } if (isvalue(p_RlfTimersAndConstants)) { v_CellGroupConfig.spCellConfig.rlf_TimersAndConstants := p_RlfTimersAndConstants; } return v_CellGroupConfig; }

After Change:

function f_NR_GetCellGroupConfigREEST( NR_CellId_Type p_TargetCellId, template (omit) NR_SCellToAddModList_Type p_SCellToAddModList := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit ) runs on NR5GC_PTC return template(value) CellGroupConfig { //@sic R5-206297 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SpCellConfig v_SpCellConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_TargetCellId); var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_TargetCellId); var template (omit) ServingCellConfig v_ServingCellConfig := f_NR_GetServinCellConfig(p_TargetCellId); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModListREEST; //@sic R5s210345 sic@ var integer i; //Remove SRB1 - entry 0 - in BearerToAddModList /*for (i:=0; i < lengthof(v_RLC_BearerToAddModList)-1; i:=i+1) { v_RLC_BearerToAddModListREEST[i] := v_RLC_BearerToAddModList[i+1]; }*/ //WA# keep SRB1 as part of the configuration. v_RLC_BearerToAddModList[0].reestablishRLC := omit; v_SpCellConfig := f_NR_GetSpCellConfigDef(p_TargetCellId, v_ServingCellConfig); v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, v_RLC_BearerToAddModList, //WA keep SRB0 as part of the configuration. v_RLC_BearerToAddModListREEST, omit, // NR_RLC_BearerToReleaseList_Type v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); //@sic R5s210345 sic@ if (isvalue(p_SCellToAddModList)) { v_CellGroupConfig.sCellToAddModList := p_SCellToAddModList; } if (isvalue(p_RlfTimersAndConstants)) { v_CellGroupConfig.spCellConfig.rlf_TimersAndConstants := p_RlfTimersAndConstants; } return v_CellGroupConfig; }