3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220210

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2350 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC reestablishment test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some UEs that support MDT feature may set the rlf-InfoAvailable-r16 IE in nonCriticalExtension/ue-MeasurementsAvaialble-r16 as true in RRCReestablishmentCopmlete message when it has some RLF info available. The current template for nonCriticalExtension in RRCReestablishmentComplete has the value set as omit, which contradicts with TS 38.508 table 4.6.1-11, which has the value as 'not checked': #### – RRCReestablishmentComplete Table 4.6.1-11: RRCReestablishmentComplete Derivation Path: TS 38.331 [6], clause 6.2.2. Information Element; Value/remark; Comment; Condition. RRCReestablishmentComplete ::= SEQUENCE {. rrc-TransactionIdentifier; RRC-TransactionIdentifier. criticalExtensions CHOICE {. rrcReestablishmentComplete SEQUENCE {. lateNonCriticalExtension; Not checked. nonCriticalExtension; Not checked. }. }. }. |
| Summary of change: |  | Changed the default value for p_RRCReestablishmentComplete_v1610_IEs to ‘*’, so the nonCriticalExtension, when included by a UE, is allowed but not checked. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some conformant Rel-16 UEs may fail test cases involving RRC reestablishment steps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.3.NR5GC and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_RRCReestablishment_Def |
| --- | --- |
| Reason for change | Some UEs that support MDT feature may set the rlf-InfoAvailable-r16 IE in nonCriticalExtension/ue-MeasurementsAvaialble-r16 as true in RRCReestablishmentCopmlete message when it has some RLF info available. The current template for nonCriticalExtension in RRCReestablishmentComplete has the value set as omit, which contradicts with TS 38.508 table 4.6.1-11, which has the value as 'not checked': #### – RRCReestablishmentComplete Table 4.6.1-11: RRCReestablishmentComplete Derivation Path: TS 38.331 [6], clause 6.2.2. Information Element; Value/remark; Comment; Condition. RRCReestablishmentComplete ::= SEQUENCE {. rrc-TransactionIdentifier; RRC-TransactionIdentifier. criticalExtensions CHOICE {. rrcReestablishmentComplete SEQUENCE {. lateNonCriticalExtension; Not checked. nonCriticalExtension; Not checked. }. }. }. |
| Summary of change | Changed the default value for p_RRCReestablishmentComplete_v1610_IEs to ‘*’, so the nonCriticalExtension, when included by a UE, is allowed but not checked. |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR5GC_RRCReestablishment_Def(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, NextHopChainingCount p_NextHopChainingCount := 0, template (omit) NR_SCellToAddModList_Type p_SCellConfig := omit, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template(omit) charstring p_VerdictMsg := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := omit, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { //@sic R5-206369 sic@ //Steps 1-2 f_NR5GC_RRCReestablishment_Steps1To2(p_NR_SourceCellId, p_NR_TargetCellId, p_ReestablishmentCause, p_RNTI_Value, p_VerdictMsg); //@sic R5-211391 sic@ //Steps 3-7 f_NR5GC_RRCReestablishment_Steps3To7(p_NR_SourceCellId, p_NR_TargetCellId, p_NextHopChainingCount, -, p_RRCReestablishmentComplete_v1610_IEs); //Steps 8-13 //p_SCellConfig is used only to send RRCReconfiguration to UE - The SCell in SS shall already be configured f_NR5GC_RRCReestablishment_Steps8To13(p_NR_SourceCellId, p_NR_TargetCellId, p_SCellConfig, p_RlfTimersAndConstants, -, p_ReceiveForRRCreconfigurationComplete); }

After Change

function f_NR5GC_RRCReestablishment_Def(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, NextHopChainingCount p_NextHopChainingCount := 0, template (omit) NR_SCellToAddModList_Type p_SCellConfig := omit, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template(omit) charstring p_VerdictMsg := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := *, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { //@sic R5-206369 sic@ //Steps 1-2 f_NR5GC_RRCReestablishment_Steps1To2(p_NR_SourceCellId, p_NR_TargetCellId, p_ReestablishmentCause, p_RNTI_Value, p_VerdictMsg); //@sic R5-211391 sic@ //Steps 3-7 f_NR5GC_RRCReestablishment_Steps3To7(p_NR_SourceCellId, p_NR_TargetCellId, p_NextHopChainingCount, -, p_RRCReestablishmentComplete_v1610_IEs); //Steps 8-13 //p_SCellConfig is used only to send RRCReconfiguration to UE - The SCell in SS shall already be configured f_NR5GC_RRCReestablishment_Steps8To13(p_NR_SourceCellId, p_NR_TargetCellId, p_SCellConfig, p_RlfTimersAndConstants, -, p_ReceiveForRRCreconfigurationComplete); }