3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220251

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0763 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS 5GS test case 7.18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case 7.18, main difference with rest of NR IMS Call test cases which replies including EVS in SDP offer, needs to include AMR-WB codec. With implementation of R5-217835 function f_IMS_BuildSDP_InitialAnswerAudio_TX was splitted in two: one to be used with NR and the other case to be used in Legacy. f_IMS_SDP_AudioMediaAttributes_InitialAnswer5G_TX (function to set intial answer for NR case) only contemplates the inclusion of EVS as initial SDP answer, this need to be change and AMRWB case must be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added AMR-WB case to be included in initial SDP answer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.18.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Corrections required	1

1.1	Correction to function f_IMS_SDP_AudioMediaAttributes_InitialAnswer5G_TX	1

# Corrections required

## Correction to function f_IMS_SDP_AudioMediaAttributes_InitialAnswer5G_TX

| Function name | f_IMS_SDP_AudioMediaAttributes_InitialAnswer5G_TX |
| --- | --- |
| Reason for change | f_IMS_SDP_AudioMediaAttributes_InitialAnswer5G_TX (function to set intial answer for NR case) only contemplates the inclusion of EVS as initial SDP answer, this need to be change and AMRWB case must be added. |
| Summary of change | Added AMR-WB case to be included in initial SDP answer. |
| TTCN module | Common\IMS\IMS_SDP_Messages.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_IMS_SDP_AudioMediaAttributes_InitialAnswer5G_TX(SDP_media_desc p_MediaDescription_Offer, IMS_SDP_AudioCodec_Type p_AudioCodec, charstring p_FmtAudio) return SDP_attribute_list { var template (omit) SDP_attribute_list v_SDP_MediaAttributes; var template (omit) SemicolonParam_List v_FmtpParameterList_Answer := omit; select (p_AudioCodec) { case (EVS_16000){ v_FmtpParameterList_Answer := f_SDP_SelectedEVSCodecParameters_AnswerTX(p_MediaDescription_Offer); // @sic R5s210606 sic@ v_SDP_MediaAttributes := f_SDP_MediaAttributes_EVS_AudioDef(p_FmtAudio, valueof(v_FmtpParameterList_Answer)); // valueof to avoid compiler warning } } return valueof(v_SDP_MediaAttributes); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_SDP_AudioMediaAttributes_InitialAnswer5G_TX(SDP_media_desc p_MediaDescription_Offer, IMS_SDP_AudioCodec_Type p_AudioCodec, charstring p_FmtAudio) return SDP_attribute_list { var template (omit) SDP_attribute_list v_SDP_MediaAttributes; var template (omit) SemicolonParam_List v_FmtpParameterList_Answer := omit; select (p_AudioCodec) { case (EVS_16000){ v_FmtpParameterList_Answer := f_SDP_SelectedEVSCodecParameters_AnswerTX(p_MediaDescription_Offer); // @sic R5s210606 sic@ v_SDP_MediaAttributes := f_SDP_MediaAttributes_EVS_AudioDef(p_FmtAudio, valueof(v_FmtpParameterList_Answer)); // valueof to avoid compiler warning } case (AMR_16000){ v_SDP_MediaAttributes := f_SDP_MediaAttributes_AMR_WB_AudioDef(p_FmtAudio); } } return valueof(v_SDP_MediaAttributes); } <<SKIPPED CODE>>