3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220264

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2268 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 7.1.1.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In test case 7.1.1.5.1, the setting of CRC mode to normal is using a different activation time. This results in updating MAC/ Phy during the transmission of previous data and waiting for ACK. In order to avoid this, it is proposed to use the same activation time as sending the next subsequent data (similar to 7.1.1.5.2/7.1.1.5.4 when error mode is set) The TTCN changes were accepted as part of R5s211597 but was not implemented in 21wk49 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved the function of setting CRC to normal before sending the data at Step 15 and removed v_Timing2 calculation as its calculated later on for sending the data |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.1.ENDC, 7.1.1.5.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required for test cases 7.1.1.5.1	2

1.1	Correction to f_TC_7_1_1_5_1_NR_TestBody	2

# Corrections required for test cases 7.1.1.5.1

## Correction to f_TC_7_1_1_5_1_NR_TestBody

| Function name | f_TC_7_1_1_5_1_NR_TestBody |
| --- | --- |
| Reason for change | In this test case the setting of CRC mode to normal is using a different activation time. This results in updating MAC/ Phy during the transmission of previous data and waiting for ACK. In order to avoid this it is proposed to use the same activation time as sending the next subsequent data (similar to 7.1.1.5.2/7.1.1.5.4 when error mode is set) |
| Summary of change | Moved the function of setting CRC to normal before sending the data at Step 15 and removed v_Timing2 calculation as its calculated later on for sending the data |
| TTCN module |  |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_5_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) runs on NR_BASE_PTC { … … … //@siclog "Step 13" siclog@ // set CRC error as error mode //Configure SS to corrupt CRC in transmission of DL MAC PDU f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Erroneous)); v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); v_Timing1 := f_NR_SubFrameTiming_AddSlots(v_Timing1,tsc_OnDurationTimer * v_SlotsInSubframe - 1); v_Timing1 := f_NR_LastDL_Slot(nr_Cell1,v_Timing1); //v_Timing1 := f_NR_SubFrameTiming_AddSlots(v_Timing1, ((tsc_OnDurationTimer-10)*v_SlotsInframe) +f_NR_DRX_LastDL_Slot(nr_Cell1)); // The other things in Step 13 automatically taken care by SS // step 13 and 15 should be on one HARQ process v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(4, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(4, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID_SCG_Bearer, v_EncodedRlcPdu1) };//@sic R5s211665 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); // set CRC error as normal mode v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_DRX_HARQ_RTT_TimerDL); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_Timing2))); //@siclog "Step 15" siclog@ // With HARQ RTT = 4slots, tsc_DRXRetransTimer_DL =12 slots,(tsc_DRX_HARQ_RTT_TimerDL + tsc_DRXRetransTimer_DL -1=15 slots)and DL-UL Periodicity =5 slots, it should be in the first PDCCH occasion in Drx-RetransmissionTimerDL // In the last PDCCH occasion when the drx-RetransmissionTimerDL for MAC PDU in Step 13 is still running v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_NR_SlotOffsetK1 + tsc_DRX_HARQ_RTT_TimerDL + tsc_DRXRetransTimer_DL ); //@sic R5s200711 R5s200833 sic@ v_Timing2 := f_NR_LastDL_Slot(nr_Cell1,v_Timing2); //v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_DRX_HARQ_RTT_TimerDL + tsc_DRXRetransTimer_DL -1 ); // The other things in Step 15 automatically taken care by SS DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing2), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); t_Watchdog.start; //@siclog "Step 14" siclog@ // Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 14? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

After Change:

function f_TC_7_1_1_5_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) runs on NR_BASE_PTC { … … … //@siclog "Step 13" siclog@ // set CRC error as error mode //Configure SS to corrupt CRC in transmission of DL MAC PDU f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Erroneous)); v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); v_Timing1 := f_NR_SubFrameTiming_AddSlots(v_Timing1,tsc_OnDurationTimer * v_SlotsInSubframe - 1); v_Timing1 := f_NR_LastDL_Slot(nr_Cell1,v_Timing1); //v_Timing1 := f_NR_SubFrameTiming_AddSlots(v_Timing1, ((tsc_OnDurationTimer-10)*v_SlotsInframe) +f_NR_DRX_LastDL_Slot(nr_Cell1)); // The other things in Step 13 automatically taken care by SS // step 13 and 15 should be on one HARQ process v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(4, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(4, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID_SCG_Bearer, v_EncodedRlcPdu1) };//@sic R5s211665 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); // set CRC error as normal mode //removed calculation of v_Timing2 and moved Test mode config along with data //v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_DRX_HARQ_RTT_TimerDL); //@siclog "Step 15" siclog@ // With HARQ RTT = 4slots, tsc_DRXRetransTimer_DL =12 slots,(tsc_DRX_HARQ_RTT_TimerDL + tsc_DRXRetransTimer_DL -1=15 slots)and DL-UL Periodicity =5 slots, it should be in the first PDCCH occasion in Drx-RetransmissionTimerDL // In the last PDCCH occasion when the drx-RetransmissionTimerDL for MAC PDU in Step 13 is still running v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_NR_SlotOffsetK1 + tsc_DRX_HARQ_RTT_TimerDL + tsc_DRXRetransTimer_DL ); //@sic R5s200711 R5s200833 sic@ v_Timing2 := f_NR_LastDL_Slot(nr_Cell1,v_Timing2); //v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_DRX_HARQ_RTT_TimerDL + tsc_DRXRetransTimer_DL -1 ); f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_Timing2))); // The other things in Step 15 automatically taken care by SS DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing2), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); t_Watchdog.start; //@siclog "Step 14" siclog@ // Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 14? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");