3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220365

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2407 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcases 8.1.5.8.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - MacBearerRouting should be added for the RRC Reconfiguration procedure at Step 1 as done for the RRC Resume procedure at Step 6a6. - The default OnSR UL grant configuration should be configured as in the function f_NR5GC_RRC_ProcedureDelay, UL grants are stopped. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - MacBearerRouting is added - OnSR UL Grant configuration configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.2.1.NR5GC, 8.1.5.8.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required for NR5GC testcases 8.1.5.8.2.x	2

1.1	Correction to f_TC_8_1_5_8_2_x_NR5GC_TestBody	2

# Corrections required for NR5GC testcases 8.1.5.8.2.x

## Correction to f_TC_8_1_5_8_2_x_NR5GC_TestBody

| Function name | f_TC_8_1_5_8_2_x_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - MacBearerRouting should be added for the RRC Reconfiguration procedure at Step 1 as done for the RRC Resume procedure at Step 6a6. - The default OnSR UL grant configuration should be configured as in the function f_NR5GC_RRC_ProcedureDelay, UL grants are stopped. |
| Summary of change | - MacBearerRouting is added - OnSR UL Grant configuration configured. |
| TTCN module | RRC_Others_CA_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { <<SKIPPED CODE>> // @siclog "Steps 1-3" siclog@ // The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3) // The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH. // Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) //@sic R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_RrcNasPduList_REQ(p_PCell, tsc_NR_RbId_SRB1, cs_TimingInfo_NR(v_TimingInfo), cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext))), car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete), 16, //converted in slots inside the function "Step 3"); // @siclog "Steps 4-5" siclog@ //The SS transmits an RRCReconfiguration message to release SCell(Cell 3) //UE transmits an RRCReconfigurationComplete message f_NR5GC_Rel_1SCell(p_PCell, p_SCell); <<SKIPPED CODE>> // @siclog "Steps 6a6 - 6a7" siclog@ f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCResume(tsc_NR_RRC_TI_Def, bit2oct(encvalue(v_SCGConfig)), v_RadioBearerConfig), crs_MacBearerRouting(p_PCell)), car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete, -, crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 6a7"); //DRBs are not reconfigured in SS: that is ok as they are not used f_NR_SS_RachProcedureConfig(p_PCell, v_NR_RachProcedureConfig); // @siclog "Steps 6a8 - 6a9" siclog@ //@sic R5-214618 sic@ f_NR5GC_Rel_1SCell(p_PCell, p_SCell); //Release CA so that MAC bearer routing is disabled } }

After Change:

function f_TC_8_1_5_8_2_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { <<SKIPPED CODE>> // @siclog "Steps 1-3" siclog@ // The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell(Cell 3) // The SS starts transmitting a periodic DCI from (16*2microsec+1+TDL) slots after step 1 to schedule PUSCH. // Check: Does the UE transmit an RRCReconfigurationComplete message within (16*2microsec+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) //@sic R5s210842 change 1 sic@ v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell,500); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); // 5 ms later f_NR_ConfigPCell_AddRelSCells(p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_SCGConfigCA, cs_TimingInfo_NR(v_TimingInfo2)); f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_RrcNasPduList_REQ(p_PCell, tsc_NR_RbId_SRB1, cs_TimingInfo_NR(v_TimingInfo), cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext)),-,-,crs_MacBearerRouting(p_PCell)), //WA# Add MacBearerRouting car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete, - , crs_MacBearerRouting(p_PCell)), //WA# Add MacBearerRouting 16, //converted in slots inside the function "Step 3"); f_NR_ULGrantConfiguration_Start(p_PCell); //WA# Def UL Grant Config <<SKIPPED CODE>> // @siclog "Steps 6a6 - 6a7" siclog@ f_NR5GC_RRC_ProcedureDelay(p_PCell, cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_TimingInfo), cs_38508_RRCResume(tsc_NR_RRC_TI_Def, bit2oct(encvalue(v_SCGConfig)), v_RadioBearerConfig), crs_MacBearerRouting(p_PCell)), car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete, -, crs_MacBearerRouting(p_PCell)), 16, //converted in slots inside the function "Step 6a7"); //DRBs are not reconfigured in SS: that is ok as they are not used f_NR_SS_RachProcedureConfig(p_PCell, v_NR_RachProcedureConfig); f_NR_ULGrantConfiguration_Start(p_PCell); //WA# Def UL Grant Config // @siclog "Steps 6a8 - 6a9" siclog@ //@sic R5-214618 sic@ f_NR5GC_Rel_1SCell(p_PCell, p_SCell); //Release CA so that MAC bearer routing is disabled } }